DATE:	April 2, 2026

TO:	Ramona Martinez, District VI Manager, Water Rights Division

Lorraine Garcia, Deputy District Manager, Water Rights Division

Cindy Stokes, Interstate Stream Commission

THROUGH:	Katie Zemlick, Chief, Hydrology Bureau, Santa Fe, NM

Laura Petronis, Supervisor, Hydrology Bureau, Santa Fe, NM

FROM:	Brad Wolaver, Senior Hydrologist, Hydrology Bureau, Santa Fe, NM

SUBJECT:	DRAFT Calendar Year 2025 hydrologic impacts on the Rio Grande, Rio Pojoaque-

Nambe, Rio Tesuque, and La Cienega springs due to Buckman well field

pumping, RG-20516 et al.

KEYWORD:	Miscellaneous Memorandum, District VI, Santa Fe County, OSE Rio Grande Basin, City of Santa Fe

ID:	MSC_2026_xxx

# Summary and Conclusion

The memorandum presents estimates of the effects of City of Santa Fe (City) Buckman Well Field (Buckman) pumping on the Rio Grande, Rio Pojoaque-Nambe, Rio Tesuque, and La Cienega Springs.  Stream depletion to the Rio Grande, Rio Pojoaque-Rio Nambe, and Rio Tesuque is evaluated from 1972 to 2025.  Impacts to La Cienega spring discharge are evaluated from 1988 to 2025.  All dates are referenced to calendar year pumping (January through December).  These depletion estimates support regulatory oversight by the New Mexico Office of the State Engineer (OSE).  The OSE Water Rights Division, District 6 evaluates compliance with Buckman well field permit conditions, and the OSE and Interstate Stream Commission (ISC) coordinate to determine whether San Juan-Chama water releases are required to offset Rio Grande surface water depletions. The evaluation this year was conducted using an automated data processing pipeline executed in Python that underwent rigorous validation and verification procedures described in detail in Appendix A.

Key findings of the 2025 evaluation are (see Tables 1–5 for detailed results):

Buckman Pumping in 2025

- Total 2025 pumping: 1351.91 acre-feet (ac-ft; which was very similar to 2024 pumping of 1372.95 ac-ft), twelfth lowest annual value since 1988

- Wells 10 through 13 accounted for 25.7% of total pumping (347.86 ac-ft)

- The three wells in the model row and column located closest to the Rio Grande contributed 46.3% of total pumping (625.96 ac-ft; Buckman 1, 7, and 8; model row 13, column 11)

Stream Depletion in 2025

- Rio Pojoaque-Nambe system: 61.252 ac-ft

- Rio Tesuque: 33.595 ac-ft

- Rio Grande above Otowi gage: 102.37 ac-ft

- Rio Grande below Otowi gage: 874.90 ac-ft

La Cienega Spring Impacts in 2025

- Springs flow reduction: 3.92 ac-ft

# Automated Workflow: Buckman Well Field Depletion Pipeline Calculations

New for 2025, the evaluation was conducted using an automated data processing pipeline executed in Python and FORTRAN, replacing the prior manual Excel and FORTRAN-based workflow. The pipeline ingests daily pumping records for the 13 Buckman wells, runs the MODFLOW96 groundwater flow model (Core, 1996; Barroll and Keyes, 2005), and generates Tables 1–5) for reporting.

The pipeline was subjected to rigorous verification and validation (V&V) testing as documented in Appendix A. All source code, specifications, and test results are version-controlled on GitHub at https://github.com/brad-wolaver-nmose/santafe-buckman/tree/master/buckman.

The pipeline consists of five sequential steps shown on the figure below.

The five steps of the pipeline are:

Step 1. Ingest Pumping Data

Daily pumping records (365 days for 13 wells) are read from the City of Santa Fe CSV file (Buckman_Well_Prod_2025.csv). Volumes reported in million gallons per day (MGD) are converted to acre-feet (ac-ft) using the conversion factor 1 MG = 3.06889 AF. The step produces Table 1 (historical annual pumping by well) and Table 2 (monthly pumping breakdown). (Python)

Step 2. Generate MODFLOW Input Files

The well file (.wel) is extended to include the current reporting year, maintaining a continuous pumping record from 1988 through 2025. A MODFLOW name file (.nam) and the other MODFLOW files for the revised Santa Fe Superposition Model (Core, 1996, Barroll and Keyes, 2005) are assembled into the run directory. (Python)

Step 3. Run MODFLOW-96 Groundwater Flow Model

The MODFLOW96 executable (modflow96.exe) runs the model and produces binary flux (.flx) files containing cell-by-cell flow budgets for river and general head boundary conditions. (MODFLOW-96)

Step 4. Run Stream Depletion Post-Processor

The FORTRAN post-processor sfmodflx_2245.exe reads the binary flux files and extracts stream depletions by reach, writing the results to a calendar-year summary file (2025). (sfmodflx_2245)

Step 5. Generate Stream Depletion Tables

The calendar-year summary is parsed to produce Tables 3, 4, and 5. Pre-1988 residual streamflow depletion based on projection of the analytical model results from Core (2003) are applied to each reach. Streamflow depletion in cubic feet per second (cfs) is converted to acre-feet using the relation cfs × days × 86,400 / 43,560 = ac-ft. The five stream reaches reported are Rio Pojoaque-Nambe, Rio Tesuque, Rio Grande above Otowi Bridge, Rio Grande below Otowi Bridge, and La Cienega Springs. Final tables are written to Excel (.xlsx) format. (Python)

Input data for the 2025 analysis include daily pumping CSV from the City of Santa Fe (Buckman_Well_Prod_2025.csv), which includes 365 daily records for 13 Buckman wells, and MODFLOW model files.

The following items are also maintained on the GitHub repository:

Specification Suite: Documentation of the pipeline design and verification procedures. The specification suite (Specifications for Buckman Depletion Pipeline.docx) was created after-the-fact with the purpose of documenting the workflow for reproducibility and ongoing maintenance, and as a potential template to designing similar AI-assisted development workflows.  The specification suite, Python code, and supporting files were created using Anthropic Claude Code v2.0.61 with Opus 4.5 (Anthropic, 2026a, 2026b) and subsequently manually edited.

requirements.txt: Python package dependencies and version pins used to run the pipeline.

# Simulated stream depletions on the Rio Grande and Santa Fe-area tributaries

Streamflow depletion on the Rio Grande, Rio Pojoaque-Nambe, and Rio Tesuque due to Buckman pumping from 1972 to 2025 was estimated by following the general procedure described in Compton (1988).  It specifies that residual hydrologic effects on the Rio Grande and its tributaries resulting from 1972 through 1987 Buckman pumping be calculated using the NMOSE analytical model (Spinks, Michael P., 1988), and that post-1987 pumping effects on nearby streams be calculated using the McAda and Wasiolek (1988) ground-water flow model.

Residual hydrologic effects on the Rio Pojoaque-Nambe and Rio Tesuque due to 1972–1987 Buckman pumping were described in Spinks (1988) for 1988 through 2010 using the NMOSE analytical model.  Core (2003) projected 1972–1987 Buckman pumping effects on the Rio Tesuque and Rio Pojoaque-Nambe after 2010.  Residual hydrologic effects on the Rio Grande due to 1972–1987 Buckman pumping were described in Fleming (1989) using the NMOSE analytical model.

Prior to 2004, the McAda-Wasiolek (1988) model was used to calculate impacts for 1988 through 2003 (based on pumping from 1988 through 2003).  For the 2004 calculation (based on pumping from 1988 through 2004), and for subsequent annual calculations, the revised superposition version of the McAda-Wasiolek (1988) model (Core, 1996; Barroll and Keyes, 2005) was used and was run with MODFLOW96.  Use of the superposition model has consolidated the number of separate model runs needed, and additionally has allowed for estimation of effects at La Cienega springs (see below).  The total diversion for each of the Buckman wells was divided equally between model layers 1 and 2 and monthly pumping was simulated in the model, which is consistent with how pumping has been simulated in previous years.  For this analysis, 2025 monthly pumping from the Buckman wells was added to the well file (which contained the previously entered 1988 through 2024 pumping), the model was run, and the results were processed. Table 3 summarizes the estimated streamflow depletion on the Rio Pojoaque-Nambe and Rio Tesuque from 1988 to 2030 due to Buckman pumping from 1972 to 2025. Table 4 summarizes the monthly estimated streamflow depletion on the Rio Grande above and below Otowi gage for 2025.

# Simulated Flow Reduction from La Cienega Springs

The effect of Buckman pumping on La Cienega spring flow was calculated using the revised superposition version of the McAda-Wasiolek (1988) model (Core, Andrew B., 1996; Barroll and Keyes, 2025). The total diversion for each of the Buckman wells was divided equally between model layers 1 and 2 and monthly pumping was simulated in the model.

Table 5 summarizes the estimated depletion to La Cienega springs from 2004 to 2030 due to Buckman pumping from 1988 to 2025.

# References

Adrion, W.R., Branstad, M.A. and Cherniavsky, J.C. (1982) “Validation, Verification, and Testing of Computer Software,” ACM Computing Surveys, 14(2), pp. 159–192. Available at: https://doi.org/10.1145/356876.356879.

Anthropic (2026a) Announcements, Introducing Claude Opus 4.5 Nov 24, 2025. Available at: https://www.anthropic.com/news/claude-opus-4-5, accessed February 23, 2026.

Anthropic (2026b) Claude Code Docs, Getting started, Claude Code overview. Available at https://code.claude.com/docs/en/overview, accessed February 23, 2026.

Barroll, P. and Keyes, E. (2025) “Santa Fe Model: Resolution of Discrepancy between Superposition and Calibrated Versions of the Model.”

Compton, M.B. (1988) Untitled letter to Sangre de Cristo Water Company / V. Phillip Soice, Re: File No. RG-20516 - Buckman Well Field, dated January 14, 1988.

Core, A.B. (2003) Residual Effects of 1972-1987 pumping at Buckman well field on the Rio Tesuque and Pojoaque River, New Mexico Office of the State Engineer Memorandum to Tom Morrison, Hydrology Bureau Chief, dated April 30, 2003.

Core, Andrew B. (1996) “Espanola Basin, Santa Fe Region Modified McAda-Wasiolek Model User Manual”, November 1996 New Mexico State Engineer Office Technical Division Hydrology Report 96-2.

Fleming, William M. (1989) Untitled letter to Mr. Philip Soice, Manager, Sangre de Cristo Water Company , dated June 13, 1988.

Mary P. Anderson and William W. Woessner (2002) Applied Groundwater Modeling, Simulation of Flow and Advective Transport.

McAda, Douglas P. and Wasiolek, Maryann (1988) Simulation of the Regional Geohydrology of the Tesuque Aquifer System near Santa Fe, New Mexico, U.S. Geological Survey Water-Resources Investigations Report 87-4056.

Spinks, Michael P. (1988) Letter from M. Spinks, NMOSE to Phillip Soice, Division Manager, Sangre de Cristo Water Co. dated June 1, 1988.

WineHQ (2026) Wine. Available at: https://www.winehq.org/, accessed February 24, 2026.

Wolaver, B. (2025) “Calendar Year 2024 hydrologic impacts on the Rio Grande, Rio Pojoaque- Nambe, Rio Tesuque, and La Cienega springs due to Buckman well field pumping, RG-20516 et al.”

# Tables

Table 1. Reported Annual Buckman Well Field pumping (acre-feet)

Table 2. 2025 Annual Buckman Well Field pumping (acre-feet)

The numbers in this table were converted from acre-feet to cfs and added to the well file for 2025.

Table 3. Estimated impacts from reported Buckman Well Field pumping through 2025 on the Rio Pojoaque-Nambe and Rio Tesuque (acre-feet)

- Residual impacts from 1972–1987 Buckman pumping were calculated using the NMOSE analytical model (Spinks, 1988), with input data from Spinks (1988) and Core (2003).  Estimated residual impacts decline to near nil by 2017 for the Rio Pojoaque–Nambe, and by 2050 for the Rio Tesuque.

- Impacts from 1988–2025 Buckman pumping (with no pumping after 2025) were calculated as follows:

  - For 1988–2003, using the McAda and Wasiolek (1988) numerical groundwater flow model (Compton, 1988);

  - For 2004–2025, using the Revised Superposition Model (Core, 1996; Barroll and Keyes, 2005); and

  - Model cell placements for wells 5 and 8 were based on GPS measurements taken on April 28, 2004.

- ** Indicates values prior to correction of simulated pumping rates in February 2005 and February 2006; corrected value are shown in parentheses for those years.

Table 4. Estimated 2025 monthly effects on the Rio Grande due to Buckman Well Field pumping from 1972 to 2025 (acre-feet)

The Revised Superposition Model (Core, 1996; Barroll and Keyes, 2005) was used to calculate depletions from pumping during 1988–2025, while depletions due to Buckman Well Field pumping from 1972 to 1987 calculated using the Analytical Model are 0 ac-ft in 2025 based on data in Fleming (1989).

Table 5. Estimated impacts on La Cienega Springs due to Buckman Well Field pumping through 2025 (acre-feet)

The Revised Superposition Model (Core, 1996; Barroll and Keyes, 2005) was used to calculate model-predicted streamflow depletion on La Cienega Springs due to simulated 1988–2025 Buckman wellfield pumping (no pumping after 2025).  Calculations used model cell placements for Buckman Well Field wells 5 and 8 based on 4/28/2004 GPS measurements.

# Appendix A: Manual and Automated Verification and Validation of the Buckman Wellfield Depletion Pipeline

## 1. Introduction

This appendix documents the manual and automated verification and validation (V&V) procedures used to confirm that results of stream depletion caused by Buckman well field pumping calculated using the Python pipeline are correct.

- In groundwater modeling:

  - “Verification” confirms that the numerical code correctly solves the governing equations; for example, by comparing model output against analytical or other solutions (Mary P. Anderson and William W. Woessner, 2002), and

  - “Validation” assesses whether a calibrated model accurately represents the physical system, typically by comparing simulated results against a data set not used during calibration (Mary P. Anderson and William W. Woessner, 2002).

- However, in this appendix:

  - “Verification” and “validation” are used in the software engineering sense (Adrion, Branstad and Cherniavsky, 1982; described below), not in the groundwater modeling sense.

  - Similarly, “pipeline” here refers to an automated data-processing workflow comprised of a sequence of computational steps where the output of each feeds into the next, and should not be confused with water infrastructure.

The purpose of the Buckman well field depletion pipeline is to use a Python workflow to convert daily groundwater pumping records into annual stream depletion estimates.

### Verification: Are we building the product right? Does the software have any bugs?

Verification confirms that the computational pipeline correctly implements the intended calculations and solves the equations correctly. It confirms that unit conversions are correct, that MODFLOW converged, that head values are reasonable, and that the mass balance closes. Verification does not ask whether the conceptual model is correct, only whether the arithmetic was performed without error. Verification also ensures that the model is working correctly by comparing the results from previous years with the current model results (

(e.g., 2022, 2023, 2025 results from prior OSE Hydrology Bureau memoranda).

In this pipeline, verification  includes:

- Checks for potentially spurious data (negative daily pumping values, missing data, and non-numeric entries),

- Sum daily pumping data for 13 wells and make sure it matches reported total daily pumping,

- Sum monthly well field pumping and compare to reported total annual pumping,

- Check unit conversion is correct,

- Check MODFLOW converged correctly,

- Check that summations in output tables are correct, and

- Other checks described in detail below.

### Validation: Are we building the right product? Do model results match physical reality?

Validation confirms that the pipeline outputs are physically reasonable and consistent with prior published results (e.g., 2022, 2023, 2024 results). It also checks whether year-over-year changes are physically reasonable and detects statistical outliers that warrant investigation.  All tolerance values, thresholds, and acceptance criteria referenced in this appendix are defined in configuration files within the pipeline codebase (e.g., Tolerance Thresholds in constants.py) and are reproduced in the summary table at the end of this appendix.

In this pipeline, validation  includes checks that:

- Year-over-year changes in pumping and depletion fall within historical ranges,

- Cumulative stream depletion never decreases (physically impossible), and

- Annual stream depletion summed across all reaches does not exceed annual Buckman pumping. This check is evaluated for the current processing year and produces a soft flag if violated (the tolerance allows annual depletion to exceed annual pumping by up to 0.1% of pumping), allowing processing to continue while alerting the analyst to review. Because stream depletion is a lagged response to the full pumping history, the annual depletion-to-pumping ratio varies from year to year; for the Buckman wellfield, this ratio typically ranges from 0.07 to 0.11. Separately, cumulative depletions for each reach (Rio Pojoaque, Rio Tesuque, and La Cienega Springs) are checked to ensure they increase monotonically year-over-year, since cumulative aquifer impact cannot decrease (tolerance: 0.01 ac-ft).

## 2. Pipeline Overview

The Buckman well field depletion pipeline converts daily groundwater pumping records into annual stream depletion estimates and consists of five steps:

Step 1 reads daily pumping data from a CSV file provided by the City of Santa Fe, converts million-gallons-per-day to acre-feet, and produces Table 1 (historical pumping by well) and Table 2 (current year monthly pumping).

Step 2 generates MODFLOW input files. It reads the prior year’s well-pumping file (.wel), appends the current year’s monthly pumping rates, and creates a MODFLOW name file (.nam) that points to all required model inputs.

Step 3 executes MODFLOW96 (modflow96.exe), which produces binary flux files (.flx) containing cell-by-cell water budgets for the Barroll and Keyes (2025) model. The pipeline was developed using Anthropic Claude Code (Anthropic, 2026b, 2026a) in a Linux environment (Windows Subsystem for Linux 2), where Wine (WineHQ, 2026) provides a compatibility layer for running the Windows MODFLOW96 and post-processor executables. The pipeline can run on any platform that supports Python 3.10+ and the required scientific Python packages; on non-Windows platforms (i.e., Max), Wine is additionally required to execute the MODFLOW96 and post-processor (sfmodflx_2245.exe) Windows binaries.

Step 4a runs sfmodflx_2245.exe, a FORTRAN post-processor that reads the binary flux files and extracts stream depletion values for each reach. This post-processor identifies stream contributions using fixed row-and-column windows written into the FORTRAN source code. (This post-processor had been previously prepared by former Hydrology Bureau staff).

Step 4b parses the post-processor output, accesses previously generated tables of residual depletions due to 1972-1987 Buckman well field pumping, and generates Excel tables: Table 3 (Rio Pojoaque-Nambe and Rio Tesuque annual depletions), Table 4 (Rio Grande above and below Otowi Bridge), and Table 5 (La Cienega Springs annual impact).

Year-over-year chaining. Tables 3 and 5 use a chaining mechanism.  When processing year N, years prior to N are copied directly from the prior year’s output file rather than recomputed. Only year N and beyond are calculated from the current MODFLOW run. This ensures that historical values, once established by their respective MODFLOW runs, remain fixed and reproducible regardless of subsequent code updates. A regression test confirms that future changes to the Python  processing code or its package dependencies produce identical results for a known set of inputs. If no prior-year output exists (first-time run), the pipeline falls back to independently verified baseline values (presented in Wolaver, 2025).

# Part I: Manual Validation and Verification

Manual verification procedures occur after the pipeline runs and after the automated procedures described below are executed.  Because this automated workflow is new and used for the first time in the analysis of 2025 pumping data, the importance of a hydrologist confirming results of the pipeline are correct by a hydrologist, the manual verification procedure is described prior to the lengthier description of the automated verification procedures.

Manual verification procedures (visual inspection of output tables, comparison to historical reports, expert review of physical reasonableness) are documented separately.

## Manual Verification

Manual verification confirms that the pipeline computes correctly at each step. In particular, manual verification checks that input data are read accurately, unit conversions are applied properly, MODFLOW input files are constructed correctly, and output tables faithfully represent the model results.

### 1. Input Data Validation (Step 1)

The raw daily pumping CSV provided by the City of Santa Fe (Buckman_Well_Prod_2024.csv and OSE_Buckman_Well_Production_2025.csv; Personal Communication from Christopher Helvey to Brad Wolaver, January 28, 2026). The data were independently checked using a manual spreadsheet with manual check formulas added)

Annual sum verification. The annual sum of daily pumping values in million gallons (MG) for each well was computed manually in a check spreadsheet. The manually summed annual MG was converted to ac-ft (Row 372–374, Buckman_Well_Prod_2025_BDW_CHECK.xlsx) using the standard conversion factor:

1 MG x (1,000,000 gal / 325,851 gal/ac-ft) = 3.0689 ac-ft/MG .

The manually summed acre-foot values were compared to the corresponding annual sum produced by Step 1 of the pipeline (Column N, 2025_Table_2_output.xlsx). The two values agreed within 0.01 acre-feet for all wells. This agreement confirms annual sums by well that Step 1 correctly reads and aggregates daily pumping records from the City’s CSV.

Similarly, the annual sums for each well produced by Step 1 of the pipeline (Column N, 2025_Table_2_output.xlsx) were compared to the year 2025 values in Table 1 (Row 39, 2025_Table_1_updated.xlsx).  Additionally, values for 1988–2024 for wells in Table 2 of the year 2024 memorandum (Wolaver, 2025) were compared to Table 2 (Rows 2–38, 2025_Table_1_updated.xlsx) and found to be identical within 0.01 acre-feet.

### 2. MODFLOW Well File Verification (Step 2)

The correct conversion of monthly pumping data to MODFLOW well file input rates (cfs) was verified by selecting two representative months spot-checking. The two months included:

- A low-value month (May 2025, Well 1, total pumping = 0.036 MG; Column B, Rows 122–152, Buckman_Well_Prod_2025_BDW_CHECK.xlsx) and

- A high-value month (September 2025, Well 6, total pumping = 13.597 MG; Column G, Rows 245–274; Buckman_Well_Prod_2025_BDW_CHECK.xlsx).

These endpoints reflect the approximate range of pumping values encountered in the 2025 dataset.  Pumping rates in the MODFLOW well file (.wel) for the Barroll and Keyes (2005) model are cubic feet per second (cfs), not acre-feet. Thus, the conversion from monthly MG to cfs is:

Q (cfs) = V (MG) x 10^6 / (7.48 gal/ft^3) / (D x 86,400 s/day)

where D is the number of days in the month. Because each Buckman well is split equally between MODFLOW Layer 1 and Layer 2, the per-layer rate is Q/2.

| Month | Well | MG | Days | Manual cfs | Manual Per-layer cfs | .wel file value | Agreement |
| --- | --- | --- | --- | --- | --- | --- | --- |
| May 2025 | Well 1 | 0.036 | 31 | 0.00180 | 0.000898 | -0.00090 | Within 0.00001 cfs |
| Sep 2025 | Well 6 | 13.597 | 30 | 0.7013 | 0.35065 | -0.35063 | Within 0.0001 cfs |

Note: The manual verification uses the direct conversion Q (cfs) = V (MG) × 10⁶ / (7.48 gal/ft³) / (D × 86,400 s/day), while the pipeline converts MG to acre-feet first (using the USGS factor 1 MG = 3.06889 AF) and then to cfs via Q (cfs) = V (AF) × 43,560 ft³/AF / (D × 86,400 s/day). The two paths differ by approximately 0.007% due to rounding in the MG-to-AF conversion factor. The .wel file values agree with the pipeline's conversion path; differences from the direct manual formula are within 0.0003 cfs for all entries.

The negative sign in the .wel file denotes extraction per MODFLOW convention. The spot-checks confirm that the pipeline correctly converts monthly pumping from MG to cfs and evenly divides the monthly pumping rate between Layer 1 and Layer 2.

Year-over-year well file consistency. The 2024 and 2025 .wel files were compared for months that should be identical in both files. The pipeline appends a new year (2025) to the previous year’s file (2024) without modifying historical entries. For the 2024 and 2025 well files, January, September, and December 2024 pumping rates were identical in both files, as was a historical check of January, September, and December 2013. This confirms that the update mechanism for the well file for year N+1 correctly preserves all entries from prior years.

### 3. Post-Processor and Table 3 Verification (Steps 3–4)

Table 3 reports annual stream depletions for Rio Pojoaque–Nambe and Rio Tesuque reaches. The values due to post-1987 Buckman well field pumping originate from MODFLOW96 binary flux files (.flx).  The flux files are for a general head boundary (GHB) representing La Cienega Springs and for a river boundary (RIV), which represents the Rio Grande and its tributaries. The two flux files are processed by the FORTRAN post-processor (sfmodflx_2245.exe) to extract stream depletion by reach. Depletion values are then parsed by the Python pipeline (step4_generate_depletion_tables.py) to produce the final Table 3.

Cross-year consistency of post-processor output. The raw post-processor outputs from the 2024 (CY2024.txt) and 2025 (CY2024.txt) MODFLOW runs were compared for overlapping historical years. Rio Pojoaque, Rio Tesuque, Rio Grande, and La Cienega Springs depletion values for 2007, 2023, and 2024 were identical between the two runs. This agreement confirms that the chain from CSV input through MODFLOW through the post-processor is functioning correctly and that identical inputs produce identical outputs.

Output formatting correction. Manual review of Table 3 identified that step4_generate_depletion_tables.py was writing raw superposition depletion results for all years 1988–2030, rather than writing only year N+1 results (2025–2030) and chaining historical values from the prior year’s output. This distinction is important because the superposition MODFLOW model (Barroll and Keyes, 2005) was published in 2005, and superposition results for years prior to 2005 were not used. Accordingly, the script was corrected to only depletion results from the superposition model for year N+1.  All prior-year values were chained from the previous output file. After the correction, the depletion results for year 2024 and 2025 model runs presented in Table 3 for years 2006 through 2024 were compared and confirmed identical, demonstrating that chaining of prior-year values is working correctly. This finding illustrates the value of manual verification, in that the formatting issue would not have been caught by automated tests that check only numerical accuracy.

Regression Test Results: Verifying Pipeline Replication (1988–2024) vs. Wolaver (2025). “Regression” testing is used here in the software engineering sense, not statistical regression (i.e., least-squares curve fitting). Code changes made to process a new year’s data, such as bug fixes, reorganizing code for clarity (refactoring), or updates to Python package dependencies, could inadvertently alter how prior years are calculated. The regression test guards against this by re-running the pipeline on frozen 2024 inputs and confirming that all outputs remain identical to the previously verified results (Wolaver, 2025). Table 3 values for years 1988–2024 produced by the pipeline match the corresponding values in the 2024 depletion memo (Wolaver, 2025), confirming that the pipeline reproduces previously verified results.

### 4. Table 4 Verification: Rio Grande Depletions Above and Below Otowi (Step 4)

Table 4 reports annual stream depletion on the Rio Grande for reaches above and below the Otowi gage. The MODFLOW model cells that define each reach (specified by row, column, and layer) are listed in the pipeline’s configuration file (constants.py). as the ABOVE_OTOWI_CELLS and BELOW_OTOWI_CELLS dictionaries. The script that creates Table 4 (step4_generate_depletion_tables.py) reads these model cell definitions and uses them to extract the corresponding depletion values from the post-processor output.- Two manual verifications of Table 4 results were done:

- Table 4 values produced by the pipeline for the 2024 analysis year match the 2024 depletion memo (Wolaver, 2025), and

- Monthly values of depletion above and below Otowi were calculated manually for 2025 (TABLE_4_Rio_Grande_Otowi_2025_MANUAL_VERIFY.xlsx). The manually calculated monthly and total 2025 depletion values above and below Otowi matched those calculated by the pipeline (TABLE_4_Rio_Grande_Otowi_2025.xlsx).

The two successful manual verifications of Table 4 confirm that both the year N+1 chaining mechanism and the depletion calculation are working correctly.

### 5. Table 5 Verification: La Cienega Springs (Step 4)

Annual impacts to La Cienega Springs are reported as annual depletion totals in Table 5. A chaining mechanism is used for Table 5, as was done with Table 3. In the chaining mechanism year N results serve as the starting point, and only year N+1 through 2030 annual depletions are computed from the current MODFLOW run. Two manual verifications of Table 5 results were done:

- Table 5 values produced by the pipeline match the corresponding values in the 2024 depletion memo (Wolaver, 2025), and

- Monthly values of La Cienega Springs depletion were also calculated manually for 2025 and summed (TABLE_5_La_Cienega_Springs_2025_MANUAL_VERIFY.xlsx). The manually calculated total 2025 La Cienega Springs depletion matched that calculated by the pipeline (TABLE_5_La_Cienega_Springs_2025.xlsx)

The two successful manual verifications of Table 5 confirm that both the chaining mechanism and the depletion calculation are working correctly.

### 6. 2024 Replication

All five output tables (Tables 1–5) produced by the pipeline for the 2024 analysis year were compared to the verified 2024 depletion memo (Wolaver, 2025). Tables 1 and 2 (pumping summaries) agreed within 0.01 ac-ft, attributable to minor rounding differences between Excel and Python floating-point representation. Tables 3, 4, and 5 (depletion tables) essentially matched the 2024 memo values exactly. This critical software quality check confirms that the pipeline faithfully reproduces the complete set of previously verified results.

### 7. Anomalous Value Investigation and City Follow-Up

The automated pipeline flagged two items requiring human review for the 2025 analysis year.  The pipeline was executed using the plan documented in .claude/plans/P_FULL_PIPELINE_2024_2025_PLAN.md and results of executing this plan were recorded in .claude/plans/P_FULL_PIPELINE_2024_2025_OUTPUT.md). The two items flagged area:

  - May 2025 low-pumping anomaly. The historical bounds check flagged May 2025 total pumping as anomalously low compared to the 2022–2024 historical range. This flag was investigated by contacting the City of Santa Fe to verify the source data (Personal Communication from Christopher Helvey to Brad Wolaver, February 25, 2026). The City confirmed that the May 2025 pumping values in the CSV are correct and reflect actual operational conditions during that month.

  - Year-over-year percentage change flag. The temporal consistency check flagged a year-over-year percentage change attributable to the low May 2025 pumping. Based on the City’s confirmation that the underlying data are correct, this flag was reviewed and determined to be acceptable. The percentage change reflects a genuine operational variation in 2025 compared to 2022–2024 rather than a data error.

Both flags were resolved through communicating with the City of Santa Fe, demonstrating the intended interaction between the automated flagging system and manual review.

# Part II: Automated Verification

These automated checks execute without manual intervention every time the pipeline runs and produce a machine-readable record of what was checked and whether it passed. Verification confirms that computations in the pipeline are correct, formulas are implemented accurately, units are converted properly, and the numerical model converged to a stable solution.

## 1. Input Data Quality Validation

Before any computation begins, the pipeline validates the raw daily pumping data received from the City of Santa Fe. These checks run during Step 1 (data ingestion) and prevent bad data from propagating into MODFLOW input files and resulting depletion tables.

Daily record screening. Every daily pumping value for all 13 Buckman wells is checked for three categories of invalid data:

- Negative values (physically impossible),

- Blank or NaN entries (missing data), and

- Non-numeric entries (data entry errors).

Zero values are treated as valid. Any flagged value is recorded in a quality flag register (defined in src/pipeline_manifest.py and output to output/manifests/) that propagates through the monthly aggregation, ensuring that months containing suspect data are marked in the output tables.

Three-tier daily sum verification. For each day, the pipeline independently sums the 13 individual well values and compares the result to the Buckman Well Field Pumping (BWP) total column provided in the source CSV. Discrepancies are classified into three severity tiers:

- Tier 1 (noise): BWP total is nonzero but below 0.0015 MGD. These values fall below the database precision threshold (100-gallon rounding in the City’s SCADA system) and are flagged as informational only.

- Tier 2 (formula error): All 13 well values are zero but the BWP total exceeds 0.0015 MGD. This indicates a formula error in the source CSV in that the total is computing a nonzero value from zero inputs.

- Tier 3 (rounding): The absolute difference between the computed sum and the BWP total is compared against three thresholds:

    - Differences <0.001 MGD pass silently,

    - Differences 0.001–0.005 MGD are flagged as informational (expected Excel rounding), and

    - Differences >0.005 MGD are flagged as errors requiring CSV review.

Annual sum verification. After monthly aggregation, the pipeline sums all 12 monthly totals for each well and compares the result to the annual sum row in the source CSV. The tolerance is 0.01 MG (10,000 gallons). A mismatch indicates either a missing month in the aggregation or a discrepancy in the source data.

## 2. Unit Conversion Accuracy

The pipeline converts between three unit systems:

  - Million gallons (MG) and million gallons per day (MGD), the units used in the City of Santa Fe's pumping records,

  - Cubic feet per second (cfs), the volumetric rate used in MODFLOW well input files, and

  - Acre-feet (ac-ft), the reporting unit for the State Engineer.

A conversion error at any stage would propagate through all downstream tables.

Conversion formulas:

| From | To | Formula | Used in |
| --- | --- | --- | --- |
| MG | ac-ft | MG x 3.0689 | Step 1 (Tables 1,2) |
| MGD | cfs | MGD x 1.547 | Step 2 (.wel file) |
| cfs | ac-ft | cfs x days x 86,400 / 43,560 | Step 4 (Tables 3,4,5) |

This formula is implemented in stream_depletions.py and is calendar-aware: February uses 29 days in leap years (2024) and 28 days in non-leap years (2025). The pipeline uses Python’s calendar.isleap() function to determine the correct day count for each year.

Hand-calculable verification example. For 1.0 cfs over a 30-day month:

1.0 cfs x 30 days x 86,400 s/day = 2,592,000 ft³
2,592,000 ft³ / 43,560 ft³/ac-ft = 59.504 ac-ft

The automated test suite verifies unit conversions across all month lengths (28, 29, 30, and 31 days), leap and non-leap years, zero and near-zero values, and round-trip conversions (i.e., convert a value from Unit A → Unit B, then convert the result back to Unit A). A total of 240 automated tests cover the full pipeline: input validation, unit conversions, MODFLOW file parsing, table generation, geometry checks, and edge-case handling. These tests confirm that every calculation function executes without error and returns values of the correct type and magnitude. The automated test suite includes:

| Category | Examples |
| --- | --- |
| Unit conversions | cfs→ac-ft for all month lengths, MGD→cfs, MG→ac-ft |
| Input verification | Negative values rejected, blank cells detected, annual sums match |
| MODFLOW file parsing | .wel file reads correctly, .nam file references valid files |
| Table generation | Row totals match, column totals match, no NaN values |
| Geometry checks | The FORTRAN post-processor extracts depletion values from a fixed subset of the model grid (rows 28–35, columns 10–20); the pipeline verifies that all six La Cienega Springs GHB cells fall within this subset- |
| Edge cases | Empty CSV, missing columns, malformed dates |

Leap year sensitivity. The pipeline’s leap year handling was verified by running both the 2024 (leap) and 2025 (non-leap) analyses and confirming that February day counts are correct in each case.

## 3. MODFLOW Numerical Integrity

MODFLOW96 solves the groundwater flow equation iteratively using the Strongly Implicit Procedure (SIP). At the end of each stress period, the model computes a volumetric water budget and reports the percent discrepancy between water entering and leaving the model domain.

The pipeline automatically extracts every percent discrepancy value from the MODFLOW listing file (.lst) and checks the maximum discrepancy across all stress periods. For this pipeline, the target threshold is 0.1%. This tolerance is achievable because the MODFLOW model uses a superposition approach(Mary P. Anderson and William W. Woessner, 2002). As a superposition model, it only simulates the relative change in head caused by pumping, without recharge, evapotranspiration or other specified-flux boundary conditions (Mary P. Anderson and William W. Woessner, 2002). In contrast, an elevation-based (absolute head) model includes multiple sources and sinks that may converge acceptably at higher percent discrepancies.

This check is evaluated as a soft target rather than a hard stop. If the maximum discrepancy exceeds 0.1% but remains below 1.0%, the pipeline flags the result for review but continues processing.  A small number of stress periods exceeding the 1.0% may also be acceptable if these periods do not materially influence the depletion results for a given year.  However, widespread or large discrepancies indicate that the solver may not have adequately converged, and that model reports should be reviewed.  For the 2024 run, all stress periods converged with discrepancies below 0.1%.

## 4. Internal Consistency Checks

Four automated consistency checks verify that data flows correctly through the pipeline without loss, corruption, or arithmetic error.

Pumping conservation. The total annual pumping read from the input CSV (via Table 2) must equal the total annual pumping applied in the MODFLOW well file, after unit conversion. The pipeline parses both files independently, converts the well file rates from ft³/s back to acre-feet, and compares the totals. The acceptable difference is 0.1% relative, which accommodates floating-point rounding in the unit conversion. This check catches errors such as if a well were omitted from the MODFLOW file, a unit conversion applied twice, or a month being skipped.

Depletion constraint. Annual stream depletion summed across all reaches does not exceed annual Buckman pumping. This check is evaluated for the current processing year and produces a soft flag if violated (tolerance: 0.1% for numerical precision), allowing processing to continue while alerting the analyst to review. Because stream depletion is a lagged response to the full pumping history, the annual depletion-to-pumping ratio varies from year to year; for the Buckman wellfield, this ratio typically ranges from 0.07 to 0.11, well below the physical limit. Separately, cumulative depletions for each reach (Rio Pojoaque, Rio Tesuque, and La Cienega Springs) are checked to ensure they increase monotonically year-over-year, since cumulative aquifer impact cannot decrease (tolerance: 0.01 ac-ft).

Table sum integrity. Within each output table, row totals must equal the sum of monthly values, and column totals must equal the sum of individual entries. The pipeline checks every summation in Table 2 (CSV format) and, when the openpyxl library is available, in Tables 3–5 (Excel format). The acceptable difference is 0.01 ac-ft, which accommodates display rounding in Excel.

Missing data detection. If any cell in the input data or output tables contains NaN (not a number) or is unexpectedly empty, the pipeline fails immediately rather than propagating a silent error. This catches problems such as a corrupted input file, a parsing failure in the post-processor output, or a division by zero in the conversion logic.

## 5. Spreadsheet Cross-Check Formulas

The output Excel files for Tables 4 and 5 contain embedded formulas that allow an independent reviewer to verify the pipeline’s arithmetic without running any Python code. These formulas are computed by Excel (or LibreOffice Calc) when the file is opened, providing a second independent calculation path.

Table 4 (Rio Grande at Otowi) contains eight cross-check formula rows:

The first four verify that the CFS totals for above-Otowi and below-Otowi reaches are the correct sums of individual MODFLOW cell values. Two use Excel’s SUMIF function to sum all cells labeled “above” or “below” in column R. Two use explicit SUM formulas over the known contiguous cell ranges. Each formula computes the difference between the formula result and the Python-computed total. If the pipeline computed correctly, all eight values display as 0.000000.

The remaining four verify the acre-feet conversion. Each formula multiplies the monthly CFS value by the number of days in that month, by 86,400 seconds per day, and divides by 43,560 cubic feet per acre-foot. The result is compared to the Python-computed ac-ft value. Again, a correct computation produces 0.000000.

Table 5 (La Cienega Springs) contains three cross-check formula columns:

This table reports the total model-predicted reduction in spring flow attributable to the full history of Buckman pumping through each year (Column B). This value is not a running sum of annual depletions; it is the impact for a given year based on the cumulative pumping of prior years, which increases over time as the effect of historical pumping propagates through the aquifer. Three cross-check formula columns verify internal consistency of the spreadsheet (these columns appear in the Excel file but are not reproduced in the memorandum): Column C computes year-over-year change in Column B, Column D reconstructs Column B by summing all year-over-year changes from 2004 forward, and Column E reports the difference between Columns B and D. A correct table produces 0.000000 in every row of Column E.

These embedded formulas serve as an audit mechanism: any reviewer with Excel can verify the pipeline’s arithmetic by simply opening the file and inspecting the cross-check columns.

## 6. Historical Chaining Verification

When the pipeline processes year N, it produces depletion values for all years from the model start (1988 for Tables 3 and 4, 2004 for Table 5) through the projection horizon (currently 2030). Values for years before N are not recomputed from the current MODFLOW run; instead, they are copied from the prior year’s output file. This chaining mechanism ensures that historical values, once established, remain fixed regardless of code changes or reprocessing.

The chaining verification has two components:

Within-table chaining. The pipeline reads the prior year’s Excel output, extracts historical values, and writes them directly into the current year’s table. For the 2025 analysis, years 1988–2024 in Table 3 were copied from the 2024 output, and years 2004–2024 in Table 5 were copied from the 2024 output. Only years 2025–2030 were recomputed from the 2025 MODFLOW run.

Cross-model verification. A separate verification script (verify_depletion.py) parses the raw post-processor output from both the 2024 and 2025 MODFLOW runs and computes superposition acre-feet for every year from 1988 through 2030. For historical years (1988-2024), the two model runs should produce identical values because the pumping data and model parameters are the same. The script flags any historical year where the absolute difference exceeds 0.001 ac-ft. Differences in years 2025–2030 are expected and reported for informational purposes, since these years incorporate different pumping data.

The output of this verification is an Excel file with side-by-side values and a difference column. Cells exceeding the 0.001 ac-ft threshold are highlighted in yellow. For the 2024–2025 comparison, all historical years agreed within the threshold, confirming that the two MODFLOW runs are consistent for the overlapping period.

## 7. Geometry Verification

The FORTRAN post-processor (sfmodflx_2245.exe) uses hardcoded cell rectangles to determine which MODFLOW cells contribute to each stream reach. For La Cienega Springs, the extraction rectangle covers model rows 28–35 and columns 10–20. Only GHB cells within this rectangle are included in the “LC SPRINGS” depletion total.

La Cienega Springs is represented in the MODFLOW model by six General Head Boundary (GHB) cells. The pipeline reads the GHB input file, extracts the (layer, row, column) coordinates of all GHB cells, and verifies that every cell falls within the FORTRAN extraction rectangle.

This check guards against a specific failure mode: if the MODFLOW grid geometry were ever modified such that a La Cienega GHB cell moved outside the extraction rectangle, that cell’s flux contribution would be silently excluded from the depletion total, resulting in an underestimate of the spring impact. Because this underestimate would not produce any error message from either MODFLOW or the post-processor, it could go undetected without an explicit geometry check.

The current six GHB cells are located at rows 30–32, columns 12–15, which is within the extraction rectangle (rows 28–35, columns 10–20) with at least a one-cell buffer on all sides. The automated check verifies both the cell coordinates and the buffer margin.

# Part II: Automated Validation

Validation (as defined above) confirms that pipeline outputs are physically reasonable and consistent with independently verified reference data.

## 1. 2024 Results

The 2024 analysis was the first year fully processed with the automated pipeline. All outputs were manually verified against legacy Excel spreadsheets produced by the prior manual workflow (Wolaver, 2025). This baseline ensures that future updates to the Python processing code or its dependencies (such as bug fixes, reorganization or simplifying the code without changing what it does, or routine Python library updates) do not inadvertently alter results for a known set of inputs.

The regression test works by re-running the entire pipeline from frozen inputs (the same input CSV and MODFLOW files used for the original 2024 analysis) and comparing every cell in every output table against the verified reference outputs (located in buckman\validation\2024\expected_outputs).

The comparison uses a hybrid tolerance approach: a cell passes if the absolute difference is within 0.01 ac-ft, or if the relative difference is within 0.1%. This handles both small values (where a tiny absolute difference is acceptable even if the relative difference appears large) and large values (where a small percent difference is acceptable even if the absolute difference exceeds 0.01 ac-ft). These thresholds are tight enough to detect any meaningful computational error while accommodating minor floating-point difference between Python and Excel.

Before running the regression, the pipeline verifies the integrity of frozen input files by computing SHA-256 cryptographic hashes and comparing them against stored reference hashes. A hash is a fixed-length numerical signature computed from a file’s contents. The hash changes if the file is modified in any way, providing a reliable means of confirming that input data has not changed between runs.  Thus, if any input file has been modified since the baseline was established, the regression test fails immediately with a message identifying which file changed. This prevents the baseline comparison from being invalidated by accidental input modifications.

The regression test also enforces strict handling of missing data: any NaN value in the output or any cell that is empty when the reference has a value (or vice versa) causes an immediate failure.

## 2. Cross-Model Depletion Comparison

When processing a new year, the pipeline generates a verification spreadsheet that compares the current MODFLOW run against the prior year’s run. This comparison is performed at the raw post-processor output level, before any table formatting, chaining, or rounding, in order to detect discrepancies at their source.

For the 2025 analysis, the verification script parsed both the 2024 and 2025 post-processor output files and computed annual superposition acre-feet for Rio Pojoaque and Rio Tesuque across all years from 1988 through 2030. The comparison applies a 0.001 ac-ft threshold: any historical year (1988–2024) where the two model runs disagree by more than this amount is flagged.

Historical years should agree exactly (within floating-point precision) because the MODFLOW model uses the same grid, parameters, and historical pumping data for both runs. A disagreement would indicate a problem with the MODFLOW run itself, such as a different version of the executable, a corrupted input file, or a convergence issue.

Years 2025–2030 are expected to differ between the two runs because the 2025 run incorporates the new 2025 pumping data, while the 2024 run projected those years based on 2024 pumping.

The output is an Excel workbook with year-by-year comparisons, difference columns, and yellow highlighting for any flagged values.

## 3. Historical Bounds and Ballpark Checks

Before running the full test suite, the pipeline executes a rapid ballpark check that tests for gross physics violations. This check uses historical bounds derived from the 2022–2024 analysis period.

Hard failures halt the pipeline immediately:

- Negative total pumping (physically impossible)

- Total pumping exceeding three times the historical maximum for 2022, 2023, and 2024 (4,118.70 ac-ft; the historical range is 866–1,373 ac-ft)

- Negative stream depletion in any reach (physically impossible)

- Non-monotonic cumulative depletion (i.e., cumulative impact cannot decrease year-over-year; tolerance of 0.01 ac-ft for floating-point errors)

Soft flags allow processing to continue but require human review:

- Total pumping exceeding three times the historical maximum for 2022, 2023, and 2024 (4,118.70 ac-ft; the historical range is 866–1,373 ac-ft)

- Total pumping exceeding 2× the 1988–2025 historical maximum of 5,937 ac-ft (i.e., exceeding 11,874 ac-ft). The historical range is 314–5,937 ac-ft, reflecting both pre-diversion from the Buckman Direct Diversion (BDD; 1988–2010, typically 3,000–6,000 ac-ft/yr) and post-diversion (2011–2025, typically 300–1,700 ac-ft/yr) operating regimes.

- Any metric more than two standard deviations from the historical mean

The ballpark check is designed to catch fatal-flaw errors early, such as loading the wrong year’s input file, a unit conversion that is off by an order of magnitude, or a post-processor that failed silently, before executing the full regression test.

## 4. Temporal Consistency

The temporal consistency checks compare the current year’s results against year-over-year patterns observed in the 2022–2024 historical period. All temporal checks produce soft flags that require human review; none are hard stops. This reflects the fact that year-over-year changes may be legitimate, such as a drought year followed by a wet year.  Soft flags should be manually reviewed by a hydrogeologist to identify if they are valid or a result in normal changes in well field operation.

Year-over-year pumping change. The threshold is 200%, derived from the maximum observed year-over-year change in the 1988–2025 operational record (175%, from 2012 to 2013) plus a buffer. The full record shows that large year-over-year swings are common for this wellfield (four years exceed 65% and seven exceed 50%) particularly during transitions between surface-water availability regimes. A change exceeding 200% has never been observed and would warrant investigation as a possible data-entry error.

Depletion-to-pumping ratio change. The depletion-to-pumping ratio is computed as total annual tributary stream depletion (Rio Pojoaque-Nambe plus Rio Tesuque, from Table 3) divided by total annual Buckman pumping.  La Cienega Springs depletions (Table 5) are excluded because their magnitude (~0.15 ac-ft/yr annual change) is negligible relative to tributary depletions. This ratio varies inversely with pumping, so that large changes in pumping produce large changes in the ratio. A year-over-year change exceeding 45%, which is derived from the maximum observed ratio change (36.5%, from 2023 to 2024) plus a 10% buffer, produces a soft flag suggesting the total model response may be inconsistent with historical behavior.

Seasonal pattern correlation. The pipeline computes the Pearson correlation between the normalized monthly pumping profile of the current year and the historical average profile. The threshold is r = 0.75. A correlation below this value indicates a potential stress period misalignment, such as if January data were accidentally labeled as February. A one-month shift in the monthly profile typically produces correlations in the range of 0.5–0.7, while normal year-to-year operational variation produces correlations above 0.85.

Multi-year envelope. The pipeline checks whether the current year’s values fall within a plausible range derived from three prior years (2022–2024). Because only three years of data are available, a traditional statistical confidence interval would be impractical, producing bounds that provide no meaningful constraint. Instead, the pipeline uses a simpler approach: it computes the historical mean and expands it by a buffer equal to the greater of ±20% or ±1.5 times the coefficient of variation (the ratio of the standard deviation to the mean). When year-to-year variability is low, the 20% threshold prevents the envelope from getting unrealistically tight; when variability is high, the buffer scales with the data. This approach is -transparent about data limitations while still providing a defensible bound.

# Part III: Automated Quality Assurance Infrastructure

## 1. Test Orchestration and Failure Handling

The pipeline’s test suite is orchestrated by run_all_tests.py, which executes checks in a defined priority order. If a higher-priority check fails, lower-priority checks are skipped to avoid wasting time on results that cannot be trusted.

The execution order is:

- Ballpark check: Physics violation screening. A hard failure (exit code 3) halts all further testing.

- Smoke tests: A smoke test is a fast, broad-coverage check designed not to verify numerical accuracy, but simply to confirm that no part of the codebase is broken. If a smoke test fails, it indicates a fundamental code error that must be resolved before any analytical results can be trusted.  190 automated tests verify that all Python functions execute without errors, return the correct types, and produce values of the expected magnitude. A failure indicates a code bug.

- Edge case and geometry tests: 30 tests verify that the pipeline handles malformed inputs, boundary conditions, and missing data gracefully, such as negative pumping values, empty CSV files, missing well columns, leap year edge cases. 16 tests verify MODFLOW grid geometry, including the GHB cell validation geometry described above.

- Conservation tests: 4 tests verify the physics-based constraints described above in Internal Consistency Checks, such as budget closure, pumping conservation, depletion constraint, and table sum integrity. These require MODFLOW output files to be present; if files are missing, the tests are skipped rather than failed.

- Temporal consistency: The year-over-year pattern checks described in the Temporal Consistency section and produce soft flags only.

- Provenance manifest: Generates the audit trail described in the Provenance and Audit Trail section.

The test suite uses three exit codes:

- 0: All hard-stop checks passed; soft flags may exist,

- 1: One or more hard-stop checks failed, and

- 3: Physics violation detected in the ballpark check.

A total of 240 automated tests are distributed across 7 test files containing 5,177 lines of test code.

Error messages follow a forensic format designed for debugging. Each message reports what failed, where it failed (file path and function), the actual value observed, the expected value or threshold, and a physical interpretation of the discrepancy. This format allows the hydrologist to diagnose the cause by reading the message.

## 2. Provenance and Audit Trail

Every pipeline run produces a provenance manifest (found here: \buckman\output\manifests) and a workflow log (\buckman\output\logs) that together document the complete chain of custody for the analysis.

Pipeline manifest. A JSON file recording that includes:

- SHA-256 hash of every input file, so any modification can be detected,

- Git commit hash of the pipeline code, so the exact version can be recovered

- Results of every automated test, with pass/fail status and measured values,

- Flag register listing any soft flags that require human review, and

- Run metadata including timestamps, machine identification, and Python version.

Workflow log. A nine-section document (produced in both Markdown and Microsoft Word formats) summarizing the pipeline execution. Sections include: metadata, executive summary, input file inventory, step-by-step execution results, output file inventory, verification summary, physical interpretation, assumptions and limitations, and an approval block with signature lines for review.

The provenance manifest answers the question: Can you prove exactly how these depletion numbers were calculated? By recording input hashes and the git commit, a hydrologist can reproduce the exact computation at any future date. By recording test results, the manifest demonstrates that quality checks were applied and passed.

## 3. Summary Table: All Automated Tolerances and Thresholds

The following table consolidates every numerical tolerance and threshold used by the automated verification framework. “Hard” checks halt the pipeline on failure. “Soft” checks produce flags requiring human review.

| Check | Tolerance | Units | Hard/Soft | Source File |
| --- | --- | --- | --- | --- |
| Verification: Input Data |  |  |  |  |
| Daily data: negative values | 0 | MGD | Hard | step1_ingest_buckman_data.py |
| Daily sum: noise threshold | 0.0015 | MGD | Informational | step1_ingest_buckman_data.py |
| Daily sum: error threshold | 0.005 | MGD | Hard | step1_ingest_buckman_data.py |
| Check | Tolerance | Units | Hard/Soft | Source File |
| Annual sum per well | 0.01 | MG | Hard | step1_ingest_buckman_data.py |
| Verification: Computation |  |  |  |  |
| MODFLOW budget closure | 0.1 | % discrepancy | Hard | test_conservation.py |
| Pumping conservation (input = applied) | 0.1 | % relative | Hard | test_conservation.py |
| Depletion cannot exceed pumping | 0.001 | ratio overshoot | Hard | test_conservation.py |
| Table sum integrity | 0.01 | ac-ft | Hard | test_conservation.py |
| Cross-model historical agreement | 0.001 | ac-ft | Hard | verify_depletion.py |
| Cumulative depletion monotonicity | 0.01 | ac-ft | Hard | ballpark_check.py |
| GHB cells within extraction rectangle | exact | cell coordinates | Hard | step4_generate_depletion_tables.py |
| Spreadsheet cross-check formulas | 0.000000 | various | Informational | Tables 4 and 5 (Excel) |
| Validation: Regression |  |  |  |  |
| Table 1 (annual pumping) | 0.01 ac-ft or 0.1% | hybrid | Hard | tolerances.yaml |
| Table 2 (monthly pumping) | 0.01 ac-ft or 0.1% | hybrid | Hard | tolerances.yaml |
| Table 3 (tributary depletions) | 0.001 ac-ft or 0.1% | hybrid | Hard | tolerances.yaml |
| Table 4 (Rio Grande depletions) | 0.1 ac-ft or 1% | hybrid | Hard | tolerances.yaml |
| Check | Tolerance | Units | Hard/Soft | Source File |
| Table 5 (La Cienega Springs) | 0.01 ac-ft or 0.5% | hybrid | Hard | tolerances.yaml |
| NaN or unexpected empty cell | zero tolerance | — | Hard | tolerances.yaml |
| Validation: Physical Bounds |  |  |  |  |
| Total pumping hard maximum | 3x historical max (4,118.70 ac-ft) | ac-ft | Hard | bounds.yaml |
| Negative pumping | 0 | ac-ft | Hard | bounds.yaml |
| Negative depletion | 0 | ac-ft | Hard | bounds.yaml |
| Total pumping soft maximum | 2x historical max (2,745.80 ac-ft) | ac-ft | Soft | bounds.yaml |
| Statistical outlier (2-sigma) | 2.0 | standard deviations | Soft | bounds.yaml |
| Validation — Temporal |  |  |  |  |
| Year-over-year pumping change | 65 | % | Soft | temporal_consistency.py |
| Year-over-year ratio change | 45 | % | Soft | temporal_consistency.py |
| Seasonal correlation | 0.75 | Pearson r | Soft | temporal_consistency.py |
| Multi-year envelope buffer | max(20%, 1.5 x CV) | fraction | Soft | temporal_consistency.py |