======================================================================== BUCKMAN WELLFIELD DEPLETION PIPELINE - WORKFLOW LOG ========================================================================

## 1. Header / Metadata

| Field | Value |
| --- | --- |
| Run Timestamp | 2026-02-25 12:08:56 |
| Processing Year | 2025 |
| Operator | bradwolaver |
| Machine | OSE-HYDRO-D0P3 |
| OS | Linux 5.15.167.4-microsoft-standard-WSL2 |
| Python | 3.10.12 |
| Git Commit | 965ca40fd793 (dirty) |
| Branch | master |

## 2. Executive Summary

- Total Pumping: 0.00 AF

- Rio Grande Depletion (Below Otowi): 874.90 AF

- Rio Grande Depletion (Above Otowi): 102.37 AF

- Verification Status: FLAGS

- Tests Passed: 218

- Tests Failed: 0

## 3. Input File Inventory

| File | Size | SHA-256 (truncated) | Role |
| --- | --- | --- | --- |
| thruCY2165.wel | 3233.1 KB | 7bb62f160837d586... | MODFLOW template file |
| CY2023.nam | 0.2 KB | 74e478de45988b6e... | MODFLOW template file |
| sflcs.bcf | 20.9 KB | abd982121a0b994d... | MODFLOW template file |
| thruCY2165.bas | 97.4 KB | 999d1d3eaaf0f204... | MODFLOW template file |
| bounds.yaml | 6.6 KB | 27645f14c927ee30... | Physical bounds validation |

## 4. Step-by-Step Execution Log

### Step 1: Ingest Buckman Pumping Data

- Input: Buckman_Well_Prod_2025.csv

- Output: Tables 1 and 2 (historical and monthly pumping)

- Records processed: 12 months

### Step 2: Update MODFLOW Files

- Input: Prior year WEL file + Table 2 pumping data

- Output: thruCY2165_2025.wel, CY2025.nam

### Step 3: Run MODFLOW96

- Executable: modflow96.exe (via Wine)

- Output: CY2025.lst, flux files (.flx)

### Step 4: Generate Depletion Tables

- Post-processor: sfmodflx_2245.exe

- Output: Tables 3, 4, 5 (stream depletions)

## 5. Output File Inventory

| File | Size | SHA-256 (truncated) | Role |
| --- | --- | --- | --- |
| TABLE_3_Rio_Pojoaque_Tesuque_2025.xlsx | 7.9 KB | ff370a8c3919fb03... | Table 3 - Rio Pojoaque/Tesuque depletions |
| TABLE_4_Rio_Grande_Otowi_2025.xlsx | 12.0 KB | eb0c52e5f43b462f... | Table 4 - Rio Grande above/below Otowi |
| TABLE_5_La_Cienega_Springs_2025.xlsx | 6.1 KB | b2a61480c297edbd... | Table 5 - La Cienega Springs |
| CY2025.lst | 9856.9 KB | 605af45ed01e552a... | MODFLOW listing file |
| thruCY2165_2025.wel | 3179.9 KB | e8ca8c594c7b91ab... | MODFLOW well file |

## 6. Verification Summary

| Layer | Name | Passed | Failed | Skipped |
| --- | --- | --- | --- | --- |
| Layer 0 (smoke) | - | 187 | 0 | 3 |
| Layer 0.5 (edge) | - | 30 | 0 | 0 |
| Layer 1 (conservation) | - | 1 | 0 | 3 |

TOTAL: 218 passed, 0 failed

## 7. Physical Interpretation (Regulatory Context)

### Pumping Analysis

Total Pumping: 0.00 AF in calendar year 2025.

Historical Context: - 5-year average: 1100 AF - Year-over-year change: -100.0% - Below-average pumping year

Well Distribution: - Most productive wells account for majority of extraction - Monthly distribution reflects seasonal demand patterns

### Rio Grande Compact Implications

Depletions below Otowi Bridge (874.90 AF) are chargeable against New Mexico’s Rio Grande Compact delivery obligation to Texas.

Key Points: 1. Rio Grande Compact (1938) allocates water among CO, NM, TX 2. Otowi gauge is the accounting point for NM deliveries 3. Groundwater depletions that reduce Rio Grande flow below Otowi constitute a debit against NM’s allocation

2025 Impact: - Below Otowi depletion: 874.90 AF - Above Otowi depletion: 102.37 AF (does not affect compact) - This represents 0.2160% of typical annual NM allocation

### Tributary Stream Impacts (Table 3)

Rio Pojoaque: - 2025 Depletion: 61.25 AF - Residual Component: 0.00 AF (pre-1988 pumping tail) - Superposition Component: 61.25 AF (1988-2025 pumping) - Note: Pojoaque residual reached zero in 2015 (Core 2003 projection)

Rio Tesuque: - 2025 Depletion: 33.60 AF - Residual Component: 12.39 AF (pre-1988 pumping tail) - Superposition Component: 21.21 AF (1988-2025 pumping) - Note: Tesuque residual continues through ~2050 (Core 2003 projection)

Scientific Basis: These calculations combine: 1. MODFLOW96 superposition model (1988-present pumping effects) 2. Core 2003 analytical projections (1972-1987 pumping residual effects)

### La Cienega Springs (Table 5)

2025 Status: - Cumulative impact since 2004: 3.92 AF - Annual increment: 0.18 AF

Context: - La Cienega Springs lies ~15 km south of Buckman Wellfield - Spring depletion impacts began to be calculated in 2005 (Barroll & Keyes) - Impacts are small due to distance and hydrogeologic setting - Values represent cumulative effects, not annual flow reduction

## 8. Assumptions and Limitations

### MODFLOW96 Superposition Model

- Linear superposition assumes aquifer response is proportional to pumping

- Model domain covers Santa Fe basin; boundary conditions fixed

- Calibrated to 2003 aquifer test data

### Pre-1988 Pumping (Core 2003 Analytical Residuals)

- Buckman wellfield pumping began in 1972

- MODFLOW superposition only simulates 1988-present

- Core 2003 extended Spinks 1988 projections through 2050

- Pojoaque residual exhausted in 2015; Tesuque continues

### La Cienega Springs

- GHB representation added in 2005 (Barroll & Keyes)

- Prior to 2005: specified flux boundary (no explicit depletion calc)

- Cumulative values reported, not annual reductions

### Model Limitations

- Steady-state aquifer assumption (no transient storage)

- No climate/recharge variability modeled

- Stream-aquifer interaction simplified

- Cell-based discretization limits spatial resolution

## 9. Approval Block

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Prepared by |  |  |  |
| Reviewed by |  |  |  |
| Approved by |  |  |  |

======================================================================== END OF WORKFLOW LOG ========================================================================

Generated: 2026-02-25 12:08:56 Git: 965ca40fd793 (master)