## AUTOMATED WORKFLOW: BUCKMAN WELL FIELD DEPLETION PIPELINE CALCULATIONS

New for 2025, the evaluation was conducted using an automated data processing pipeline executed in Python and FORTRAN, replacing the prior manual Excel-based workflow. The pipeline ingests daily pumping records for the 13 Buckman wells, runs the MODFLOW-96 groundwater flow model, and generates the depletion tables (Tables 1 through 5) used for compliance reporting. The pipeline was subjected to rigorous verification and validation (V&V) testing as documented in Appendix A. All source code, specifications, and test results are version-controlled on GitHub at https://github.com/brad-wolaver-nmose/santafe-buckman/tree/master/buckman.

The pipeline consists of five sequential steps:

- Step 1 — Ingest Pumping Data. Daily pumping records (365 days for 13 wells) are read from the City of Santa Fe CSV file (Buckman_Well_Prod_2025.csv). Volumes reported in million gallons per day (MGD) are converted to acre-feet (AF) using the USGS conversion factor 1 MG = 3.06889 AF. The step produces Table 1 (historical annual pumping by well) and Table 2 (monthly pumping breakdown). (Python)

- Step 2 — Generate MODFLOW Input Files. The cumulative well (.wel) file is extended to include the current reporting year, maintaining the continuous pumping record from 1988 through 2025. A MODFLOW name (.nam) file and 10 baseline model files are assembled into the run directory. (Python)

- Step 3 — Run MODFLOW-96 Groundwater Flow Model. The MODFLOW-96 executable (modflow96.exe) solves the groundwater flow equations and produces binary flux (.flx) files containing cell-by-cell flow budgets. (MODFLOW-96)

- Step 4 — Run Stream Depletion Post-Processor. The FORTRAN post-processor sfmodflx_2245.exe reads the binary flux files and extracts stream depletions by reach, writing the results to a calendar-year summary file (CY2025). (sfmodflx_2245)

- Step 5 — Generate Depletion Tables. The calendar-year summary is parsed to produce Tables 3, 4, and 5. Pre-1988 analytical residuals from Core (2003) are applied to each reach. Stream depletion flows in cubic feet per second (cfs) are converted to acre-feet using the relation cfs × days × 86,400 / 43,560 = AF. The five stream reaches reported are Rio Pojoaque, Rio Tesuque, Rio Grande above Otowi Bridge, Rio Grande below Otowi Bridge, and La Cienega Springs. Final tables are written to Excel (.xlsx) format. (Python)

Figure X shows the pipeline workflow from raw pumping data through final depletion tables.

Input data for the 2025 analysis:

- Daily pumping CSV from the City of Santa Fe (Buckman_Well_Prod_2025.csv) — 365 daily records for 13 Buckman wells

- Baseline MODFLOW model files calibrated through 2023

The following items are also maintained on the GitHub repository:

- Specification Suite — After-the-fact documentation of the pipeline design and verification procedures, created February 20 and 23, 2026 using Claude Code v2.0.61 / Opus 4.5, and subsequently manually edited for accuracy and completeness.

- requirements.txt — Python package dependencies and version pins used to run the pipeline.