# Methodology: Tables 1 and 2 (Pumping Data)

## Overview

Tables 1 and 2 report annual pumping volumes from the Buckman wellfield. These tables are generated from monthly XML reports provided by the City of Santa Fe.

Generated by: ingest_buckman_data.py

## Data Source

### Input Files

- Format: CSV files exported from City of Santa Fe XML reports

- Location: input/ directory

- Naming: YYYY_MM_MON.csv (e.g., 2024_01_JAN.csv)

- Content: Daily flow rates in MGD (million gallons per day) for each of 13 Buckman wells

### Wells

The Buckman wellfield consists of 13 production wells:

| Well # | Well ID | Well # | Well ID |
| --- | --- | --- | --- |
| 1 | B-1 | 8 | BPA-2 |
| 2 | B-2 | 9 | BPA-3 |
| 3 | B-3 | 10 | BPA-4 |
| 4 | B-4 | 11 | BPA-5 |
| 5 | B-6 | 12 | BPA-6 |
| 6 | PM-1 | 13 | BPA-7 |
| 7 | BPA-1 |  |  |

## Unit Conversion

### Source Units

- Daily values: MGD (million gallons per day)

- Monthly totals: MG (million gallons)

### Target Units

- AFY (acre-feet per year)

- AF (acre-feet) for monthly values

### Conversion Factor

1 acre-foot = 325,851 gallons (USGS definition)
1 million gallons = 3.06889 acre-feet

Formula:

AF = MG × 3.06889

### Calculation Process

- Sum daily MGD values for each month (gives MG)

- Multiply MG by 3.06889 to get AF

- Sum 12 monthly AF values to get annual AFY

## Table 1: Historical Pumping by Well

### Purpose

Reports annual pumping volumes for each well from 1988 through the current year.

### Structure

- Rows: Calendar years (1988-present)

- Columns: Well numbers (1-13), Total, Rank

- Units: Acre-feet per year (AFY)

### Rank Column

Years are ranked by total pumping volume: - Rank 1 = lowest pumping year (drought conservation) - Higher rank = higher pumping

### Data Sources by Year

| Period | Source |
| --- | --- |
| 1988-2023 | Historical data (pre-loaded in validation file) |
| Current year | Calculated from monthly XML reports |

### Output Files

- YYYY_Table_1_updated.xlsx - Excel format with formatting

- YYYY_Table_1_updated.csv - CSV format for data processing

## Table 2: Current Year Pumping Detail

### Purpose

Reports monthly pumping volumes for each well in the current reporting year.

### Structure

- Rows: 13 wells + Total row

- Columns: 12 months (JAN-DEC) + Annual Total

- Units: Acre-feet (AF) for monthly, Acre-feet per year (AFY) for annual

### Summary Statistics

The output includes: - Grand total pumping for the year - Wells 10-13 subtotal (BPA-4 through BPA-7) - Wells 1, 7, 8 subtotal (B-1, BPA-1, BPA-2) - Percentage breakdowns

### Output Files

- YYYY_Table_2_output.xlsx - Excel format with formatting

- YYYY_Table_2_output.csv - CSV format for data processing

## Additional Outputs

### Monthly CSV Files

Individual monthly summaries are saved as: - YYYY_MM_MON.csv (e.g., 2024_01_JAN.csv)

### Input Summary

- input_summary.csv - Lists all input files processed

## Data Quality Checks

### Validation Tolerance

- Daily MGD tolerance: 0.001 MGD (1,000 gallons/day)

- Monthly MG tolerance: 0.01 MG (10,000 gallons)

### Checks Performed

- All 13 wells present in each month

- No negative values

- Monthly totals within expected range

- Annual total compared to historical averages

## Example: 2024 Values

### Table 2 Summary

| Metric | Value |
| --- | --- |
| Annual Total | 1,372.92 AFY |
| Wells 10-13 | 386.86 AFY (28.2%) |
| Wells 1, 7, 8 | 713.75 AFY (52.0%) |

### Monthly Breakdown (sample)

| Month | Total (AF) |
| --- | --- |
| JAN | 111.68 |
| FEB | 84.01 |
| MAR | 88.52 |
| … | … |
| DEC | 123.36 |
| Annual | 1,372.92 |

## Code References

| Component | File | Lines |
| --- | --- | --- |
| MG_TO_AF conversion | ingest_buckman_data.py | 55-57 |
| Well mapping | ingest_buckman_data.py | 46-50 |
| generate_table2_output() | ingest_buckman_data.py | 697-830 |
| write_table2_xlsx() | ingest_buckman_data.py | 832-1000 |
| generate_table1_output() | ingest_buckman_data.py | 1005-1180 |
| write_table1_xlsx() | ingest_buckman_data.py | 1187-1350 |

## Source Documents

| Document | Purpose |
| --- | --- |
| City of Santa Fe monthly XML reports | Raw pumping data |
| validation/Table_1_data_afy_2024.xlsx | Historical data & validation |
| validation/Table_2_2024.xlsx | Current year validation |

Document generated: February 2026 Data validated against City of Santa Fe records