# Methodology: Tables 3, 4, and 5 (Stream Depletion Calculations)

## Overview

Tables 3, 4, and 5 report stream depletion impacts from Buckman wellfield pumping. These tables are generated from the MODFLOW96 superposition model post-processor output, with additional analytical residuals applied for tributary streams.

Generated by: generate_depletion_tables.py and stream_depletions.py

## Data Sources

### Primary Source: MODFLOW Post-processor Output

- File: output/modflow/YYYY/depletions/CYYYYY

- Generated by: sfmodflx_2245.exe post-processor

- Input files:

  - CYYYYY_ghb.flx (General Head Boundary flux)

  - CYYYYY_riv.flx (River flux)

- Units: CFS (cubic feet per second), monthly averages

### Secondary Source: Analytical Residuals (Table 3 only)

- Spinks 1988: Original “triangular model” for 1972-1987 pumping effects

- Core 2003: Extended Spinks projections through 2050

## Table 3: Rio Pojoaque-Nambe & Rio Tesuque

### Purpose

Reports annual stream depletion impacts to the Rio Pojoaque-Nambe and Rio Tesuque tributaries.

### Two Components Required

The MODFLOW superposition model only simulates pumping from 1988 onward. However, Buckman wellfield pumping began in 1972. The effects of 1972-1987 pumping continue to propagate through the aquifer system and must be accounted for separately.

Component 1: Analytical Residual (Pre-1988 Pumping Effects) - Source: Spinks 1988, extended by Core 2003 - Represents the “tail” of stream depletion from 1972-1987 pumping - Decreases over time as effects dissipate

Component 2: MODFLOW Superposition (1988-Present Pumping Effects) - Source: Post-processor “R POJOAQUE” and “R TESUQUE” stream totals - Monthly CFS values converted to annual acre-feet

### Formula

Total Impact = Analytical Residual + MODFLOW Superposition

### Pojoaque Residual Values

From stream_depletions.py:85-93 (CORE_2003_POJOAQUE dictionary): - 1988: 40.432 AF - Decreasing over time - 2015: 0.316 AF (last non-zero value) - 2016 onward: 0 AF (residual exhausted)

### Tesuque Residual Values

From stream_depletions.py:97-107 (CORE_2003_TESUQUE dictionary): - 1988: 21.015 AF - Follows linear regression: y = -0.4908 × year + 1006.2 - 2024: 12.877 AF - 2025: 12.387 AF - Continues through approximately 2050

### Example: 2024 Calculation

| Stream | Residual (AF) | Superposition (AF) | Total (AF) |
| --- | --- | --- | --- |
| Pojoaque | 0.000 | 60.797 | 60.797 |
| Tesuque | 12.877 | 20.706 | 33.583 |

### Historical Context

- Spinks 1988: SEO memo dated June 1, 1988, created the original “triangular model” projecting depletion effects through 2010

- Core 2003: Extended Spinks projections using linear regression to estimate when residual effects would reach zero

## Table 4: Rio Grande Above and Below Otowi

### Purpose

Reports annual stream depletion impacts to the Rio Grande, separated into regions above and below the Otowi gauge.

### Single Component Only

- MODFLOW superposition only - no analytical residuals required

- The Rio Grande was not included in the Spinks/Core analytical models

### Otowi Cell Definitions

From stream_depletions.py:

Above Otowi (10 cells):

1 1 16, 1 2 16, 1 3 16, 1 4 16, 1 5 15,
1 6 14, 1 7 14, 1 8 13, 1 9 13, 1 10 12

Below Otowi (16 cells):

1 11 11, 1 12 11, 1 13 11, 1 14 10, 1 15 9,
1 15 10, 1 16 9, 1 17 8, 1 18 6, 1 18 7,
1 19 6, 1 20 5, 1 21 4, 1 21 5, 1 22 4, 1 23 3

### Calculation

- Sum monthly CFS values for all cells in each group

- Convert to annual acre-feet

### Example: 2024 Values

| Region | Depletion (AF) |
| --- | --- |
| Above Otowi | 101.43 |
| Below Otowi | 842.95 |
| Total | 944.38 |

## Table 5: La Cienega Springs

### Purpose

Reports cumulative stream depletion impacts to La Cienega Springs.

### Single Component Only

- MODFLOW superposition only - no analytical residuals required

### Why No Analytical Residuals?

La Cienega Springs was not explicitly modeled until 2005. From Barroll & Keyes 2005 (Introduction, p.1):

“OSE developed a superposition version of this model in which they added the ability to calculate impacts to La Cienega Springs by replacing what had originally been a specified flux boundary with a generalized head boundary (GHB).”

Prior to 2005: - The calibrated MODFLOW model used a specified flux boundary - This approximated spring behavior but did not explicitly calculate depletion impacts - No Spinks/Core analytical model exists for La Cienega Springs

Explicit depletion calculations begin in 2004.

### Key Insight: Values Are Already Cumulative

The MODFLOW superposition model output at “LC SPRINGS” represents total cumulative depletion from all pumping since 1988, not annual depletion.

This is because superposition sums the effects of all historical pumping stress periods.

### Calculation

Cumulative_YYYY = Convert LC SPRINGS CFS to AF
Annual_YYYY = Cumulative_YYYY - Cumulative_(YYYY-1)

### Example: 2024 Values

| Metric | Value (AF) |
| --- | --- |
| Previous Cumulative (2023) | 3.54 |
| Current Cumulative (2024) | 3.74 |
| Annual Increment | 0.20 |

### Historical Cumulative Values

From stream_depletions.py:48-76 (LA_CIENEGA_CUMULATIVE dictionary): | Year | Cumulative AF | Year | Cumulative AF | |——|—————|——|—————| | 2004 | 0.45 | 2018 | 2.75 | | 2005 | 0.66 | 2019 | 2.90 | | 2006 | 0.83 | 2020 | 3.06 | | 2007 | 0.99 | 2021 | 3.21 | | 2008 | 1.16 | 2022 | 3.37 | | 2009 | 1.32 | 2023 | 3.54 | | 2010 | 1.49 | 2024 | 3.74 | | 2011 | 1.65 | 2025 | 3.92 | | 2012 | 1.82 | 2026 | 4.10 | | 2013 | 1.97 | 2027 | 4.27 | | 2014 | 2.13 | 2028 | 4.46 | | 2015 | 2.29 | 2029 | 4.62 | | 2016 | 2.45 | 2030 | 4.80 | | 2017 | 2.60 | | |

## Unit Conversion

### CFS to Acre-Feet

AF = CFS × days_in_month × 1.9835

Where: - CFS = average flow rate for the month (cubic feet per second) - days_in_month = number of days in that month - 1.9835 = conversion factor (cfs-day to acre-feet)

Annual total = sum of all 12 monthly values

## Source Documents

| Document | Date | Purpose |
| --- | --- | --- |
| Spinks 1988 | June 1, 1988 | Original analytical model for 1972-1987 pumping effects |
| Core 2003 | April 30, 2003 | Extended Spinks projections through 2050 |
| Barroll & Keyes 2005 | Nov 29, 2005 | Added La Cienega Springs GHB to superposition model |
| MSC_2025_002 | June 17, 2025 | Official 2024 validation values |

## Code References

| Component | File | Lines |
| --- | --- | --- |
| CORE_2003_POJOAQUE | stream_depletions.py | 85-93 |
| CORE_2003_TESUQUE | stream_depletions.py | 97-107 |
| LA_CIENEGA_CUMULATIVE | stream_depletions.py | 48-76 |
| ABOVE_OTOWI_CELLS | stream_depletions.py | 111-115 |
| BELOW_OTOWI_CELLS | stream_depletions.py | 118-124 |
| generate_table3_data() | stream_depletions.py | 501-590 |
| generate_table4_data() | stream_depletions.py | 720-820 |
| generate_table5_data() | stream_depletions.py | 880-973 |

Document generated: February 2026 Workflow verified against MSC_2025_002 official values