# Appendix A: Specification Index

# Buckman Depletion Pipeline SPECIFICATION SUITE Index

The specification suite was created after-the-fact for the Buckman pipeline depletion workflow.  The purpose of the specifications is to document the workflow for reproducibility and ongoing maintenance, and as a potential template to designing similar AI-assisted development workflows.  The suite was created on February 20 and 23, 2026 using Anthropic Claude Code v2.0.61 with Opus 4.5 (Anthropic, 2026a, 2026b) and subsequently manually edited.

In these specifications, “pipeline” refers to an automated data-processing workflow comprised of a sequence of computational steps where the output of each feeds into the next, and should not be confused with water infrastructure. The purpose of the Buckman well field depletion pipeline is to use a Python workflow to convert daily groundwater pumping records into annual stream depletion estimates.

## How the Specification Suite is Organized

### Scientific Foundation Specifications (Tier 1: Domain Specifications)

The “Why” Behind the Workflow: Six specifications document the hydrologic science, modeling assumptions, and domain knowledge that underpin the workflow.

### Implementation Specifications (Tier 2)

Twelve specifications describe how each workflow module was built. Each specification is written as a self-contained prompt that can be pasted into a fresh Claude Code session to reconstruct the described module, with the Tier 1 scientific foundation documents available as context.

## Tier 1: Domain Specifications

| Specification | Domain | Key Content |
| --- | --- | --- |
| DS-01 | Buckman Wellfield & Well Production Data | 13 wells, CSV format, million gallons (MG)→acre-ft (AF) conversion, daily→monthly aggregation, Tables 1–2 |
| DS-02 | MODFLOW96 Santa Fe Local-Scale Model | Grid (23×25×2), boundary conditions (RIV, GHB), well package format, AF→cubic feet per second (cfs) conversion, stress periods, convergence |
| DS-03 | Stream Depletion Analysis | Superposition principle, Core (2003) analytical residuals, FORTRAN post-processor input/output (I/O), cfs→AF conversion, cell-to-stream mapping, Tables 3–5 |
| DS-04 | Year-Chaining & Temporal Architecture | WEL file accumulation (1988→present), Table 1 row extension, Table 3 historical residual impact preservation, directory structure, leap years |
| DS-05 | Quality Assurance Framework | Policy describing hard-fail versus flag thresholds for potential errors, physical bounds, and tolerance criteria. Includes the automated regression harness, which is a system to verify that code modifications still reproduce the validated 2024 results for Tables 1–5 of Wolaver (2025), confirming consistent outputs as the code is updated or modified. |
| DS-06 | Regulatory & Compliance Context | Rio Grande Compact, OSE reporting, provenance chain-of-custody, analyst disposition |

### Build Order

DS-01 (Well Production): Foundational data

DS-02 (MODFLOW96): Numerical model context

DS-03 (Stream Depletion): Core science (depends on DS-01, DS-02)

DS-04 (Year Chaining): Architectural pattern (depends on DS-01, DS-02)

DS-05 (QA Framework): Verification philosophy

DS-06 (Regulatory): Compliance requirements

## Tier 2: Implementation Specifications

| Specification | Module | Depends On | Key Content |
| --- | --- | --- | --- |
| IS-01 | Project Scaffold & Constants | — | Directory structure, requirements.txt, constants module, pyproject.toml |
| IS-02 | CSV Ingestion & Table 2 | IS-01 | step1_ingest_buckman_data.py: CSV parser, daily validation, monthly aggregation, MG→AF, Table 2 XLSX |
| IS-03 | Table 1 Chaining | IS-02 | Table 1 template loading, row extension, historical preservation, XLSX export |
| IS-04 | WEL File Management | IS-01 | step2_update_modflow.py: year n WEL parser, AF→cfs conversion, year n+1 WEL generation |
| IS-05 | MODFLOW Execution Wrapper | IS-04 | step3_run_modflow.sh: NAM generation, static MODFLOW input file staging to year n+1, Wine execution (automated execution of Fortran binary), convergence verification |
| IS-06 | Post-Processor & Output Parsing | IS-05 | Automated execution of FORTRAN post-processor, parse_postprocessor_output(), cell coordinate extraction |
| IS-07 | Stream Depletion Library | IS-01 | stream_depletions.py: cfs_to_acre_feet(), analytical residuals, classification of cells as above or below Otowi gage |
| IS-08 | Table 3 Generation | IS-06, IS-07 | Rio Pojoaque & Tesuque depletion: residual + superposition, year chaining (i.e, inserting results for year n+1 to year n table), XLSX export |
| IS-09 | Tables 4 & 5 Generation | IS-06, IS-07 | Rio Grande above/below Otowi (Table 4), La Cienega (Table 5) |
| IS-10 | Test Suite | IS-02–IS-09 | pytest smoke tests (workflow runs without failure), edge cases (extreme high/low input values), conservation checks (bass balance verification), conftest.py |
| IS-11 | Validation Framework | IS-10 | Physical plausibility bounds, temporal consistency (verification that calculated values follow physically reasonable trends through time), regression harness, tolerance configuration (acceptable margin-of-error thresholds for calculated output verification) |
| IS-12 | Provenance | IS-10 | pipeline_manifest.py, workflow_logger.py, SHA-256 hashing, JSON manifest |

### Dependency Graph

Diagram showing the required sequence of how each implementation specification builds on prior ones for the full workflow.

## Specification to Source Code Traceability Matrix

The Buckman pipeline was developed iteratively through AI-assisted coding prior to formal specification. This table retrospectively maps each source file to its governing  implementation specification (IS) and supporting domain specification(s; DS). The domain specifications provided the hydrologic and modeling context for each module.

| Source File | Implementation Specification | Domain Specification |
| --- | --- | --- |
| step1_ingest_buckman_data.py | IS-02, IS-03 | DS-01 |
| step2_update_modflow.py | IS-04 | DS-02, DS-04 |
| step3_run_modflow.sh | IS-05 | DS-02 |
| step4_generate_depletion_tables.py | IS-06 | DS-03 |
| stream_depletions.py | IS-07, IS-08, IS-09 | DS-03, DS-04 |
| run_all_tests.py | IS-10, IS-11 | DS-05 |
| src/pipeline_manifest.py | IS-12 | DS-06 |
| src/workflow_logger.py | IS-12 | DS-06 |
| validation/ballpark_check.py | IS-11 | DS-05 |
| validation/temporal_consistency.py | IS-11 | DS-05 |
| validation/2024/run_regression_2024.py | IS-11 | DS-05 |
| tests/test_ingest_buckman_data.py | IS-10 | DS-01 |
| tests/test_update_modflow.py | IS-10 | DS-02 |
| tests/test_stream_depletions.py | IS-10 | DS-03 |
| tests/test_generate_depletion_tables.py | IS-10 | DS-03 |
| tests/test_conservation.py | IS-10 | DS-05 |
| tests/test_edge_cases.py | IS-10 | DS-05 |
| tests/test_modflow_geometry.py | IS-10 | DS-02 |

## Reference Documents

Several supporting documents provide technical detail too specific for inclusion in individual specifications. This table identifies where each reference document is located and which specifications rely on it.

| Document | Path | Used By |
| --- | --- | --- |
| MODFLOW Cell Mapping | docs/MODFLOW_CELL_MAPPING.md | DS-02, DS-03, IS-06, IS-07 |
| File Dependencies | docs/FILE_DEPENDENCIES.md | DS-04, IS-04, IS-05 |
| Handoff Prompt | docs/claude_code_handoff_prompt.md | DS-05, DS-06 |
| Historical Bounds | validation/historical/bounds.yaml | DS-05, IS-11 |
| Tolerances | validation/2024/tolerances.yaml | DS-05, IS-11 |
| Excel Format Specs | docs/EXCEL_FORMAT_SPECIFICATIONS.md | IS-02, IS-03, IS-08, IS-09 |
| New Year Checklist | docs/NEW_YEAR_CHECKLIST.md | DS-04 |
| Testing Framework | docs/TESTING_FRAMEWORK.md.md | DS-05, IS-10 |

## Specification-to-Code Verification Checklist

Note that specifications were documented after the pipeline was complete. As a result, verification confirms each specification accurately describes the existing source code and validated outputs, rather than having guided their development. Verification includes these steps:

- Completeness: Every function in the pipeline is described in at least one specification.

- Accuracy: Numerical values, unit conversions, and well locations documented in the specifications match what is coded in the pipeline.

- Dependency: No circular dependencies between specs; build order is valid.

- Reproducibility: A fresh Claude Code session using only the specifications should successfully rebuild the workflow and pass all validation tests.

- Review by Subject Matter Expert: Domain specifications DS-01 through DS-06 reviewed for hydrologic and modeling accuracy.

## Templates

Two templates are provided in support of development of specifications for future workflows which follow the structure of the specification suite presented in this memorandum:

- Domain Specification Template: Documents the scientific basis, assumptions, and hydrologic context (Appendix X), and

- Implementation Specification Template: Describes what each module in the workflow does, what input data it requires, what output it produces, and what other modules and specifications must be completed first (i.e., development requirements; Appendix Y).

# Appendix B: Domain Specifications (DS-01 to DS-06)

# DS-01: Buckman Wellfield & Well Production Data

Tier 1 Domain Specification – Captures scientific basis, assumptions, and domain knowledge for a hydrologist audience. Reusable across projects. A reader with groundwater modeling background can understand the system without seeing code.

Status: Draft Author: Claude Code (Anthropic) + Brad Wolaver (NMOSE) Created: 2026-02-20 Last Updated: 2026-02-20

## 1. Purpose & Scope

This specification documents the Buckman Wellfield production data ingestion process for the City of Santa Fe’s water rights compliance reporting. The Buckman Wellfield consists of 13 production wells operated under New Mexico Office of the State Engineer (OSE) permits. Daily pumping records in million gallons per day (MGD) are received as CSV files, aggregated to monthly and annual totals, converted to acre-feet (AF), validated against internal checksums, and formatted into Tables 1 and 2 for annual depletion reports submitted to the OSE. Accurate pumping data is the upstream input to all subsequent MODFLOW modeling and stream depletion calculations.

## 2. Scientific Basis

### 2.1 Key Equations

Volume conversion (MG to AF): AF = MG * 3.06889

Derivation:

1 acre-foot = 325,851 gallons

1 MG = 1,000,000 gallons

AF/MG = 1,000,000 gal / 325,851 gal/AF = 3.06889 AF/MG

Daily-to-monthly aggregation:

Monthly_MG = SUM(Daily_MGD_i)   for i = 1..N_days

Dimensional analysis: MGD * day = (MG/day) * day = MG

The numerical sum of daily MGD values IS the monthly MG total because summing MGD values over N days implicitly multiplies by 1 day per record.

### 2.2 Worked Example

Step 1: Sum daily MGD to monthly MG

Suppose Buckman Well #1 pumps the following in January 2024 (31 days): - Days 1-15: 1.200 MGD - Days 16-31: 0.800 MGD (16 days)

Monthly_MG = (15 * 1.200) + (16 * 0.800) = 18.000 + 12.800 = 30.800 MG

Step 2: Convert monthly MG to AF

Monthly_AF = 30.800 MG * 3.06889 AF/MG = 94.522 AF

Step 3: Annual total

Sum all 12 monthly AF values for the well’s annual production.

## 3. Assumptions

| # | Assumption | Justification | Risk if Wrong | Mitigation |
| --- | --- | --- | --- | --- |
| 1 | Daily CSV values are in MGD (million gallons per day) | Column header format “BWell N¦Flow Mgd” and consistent with historical files | Wrong unit interpretation would produce values off by orders of magnitude | Cross-check monthly totals against CSV Sum row |
| 2 | Zero daily values are valid (well not pumping) | Wells are routinely shut down for maintenance or seasonal demand reduction | None. Zero is a physically valid state | Flagging system distinguishes zero from missing |
| 3 | Negative daily values are physically impossible | Flow meters measure only forward flow; negative values indicate data entry error | Negative values would corrupt monthly totals | Flagged as “NEGATIVE” in validation |
| 4 | Blank/NaN values indicate missing data, not zero pumping | Missing SCADA records differ from confirmed zero-production days | Treating blanks as zeros could undercount pumping if well was actually operating | Flagged as “BLANK”; months with any blanks marked Has_Flagged_Data=True |
| 5 | The CSV Sum row provides independent annual totals for each well | City database computes these sums independently from the daily values | If Sum row is also derived from daily values, it is not truly independent | Tolerance-based comparison (0.01 MG) catches discrepancies |
| 6 | Well 3 and Well 3A are reported as a single combined well (“3/3A”) | Historical convention from OSE permit structure; permit RG-20516-S covers both | Separate accounting could yield different per-well totals | Documented in WELL_OSE_MAP and Table 1/2 headers |
| 7 | 1 MG = 3.06889 AF is the correct conversion factor | Derived from: 1 AF = 325,851 gallons | Using incorrect factor would propagate error to all downstream tables | Factor is hardcoded and validated against USGS reference |

## 4. Data Sources & Provenance

| Source | Description | Access Method | Update Frequency | QC Procedure |
| --- | --- | --- | --- | --- |
| City of Santa Fe | Daily well production CSV files | Email delivery to NMOSE | Annually (January–February for prior year) | Three-tier daily sum verification |
| CSV file | Buckman_Well_Prod_{year}.csv with 365–366 daily rows (365 for non-leap years, 366 for leap years) + 4 summary rows (Sum, Avg, Max, Min) | Local filesystem: ./input/csv/ | One file per calendar year | Column presence check, date range validation, annual sum comparison |
| USGS unit definitions | Gallon-to-AF conversion factor | Published standard (USGS NAWQA glossary) | Static | Hardcoded constant verified against source |

### CSV File Structure

Header row: "1/1/2024-12/31/2024", "BWell 1|Flow Mgd", ..., "BWell 13|Flow Mgd", "BWP|Flow Mgd|MGD|Formula"
Row 1-N:    Date, Well_1_MGD, Well_2_MGD, ..., Well_13_MGD, BWP_Total_MGD
            (N = 365 for non-leap years, 366 for leap years)
Row N+1:    "Sum", annual_MG_1, annual_MG_2, ..., annual_MG_13, annual_MG_total
Row N+2:    "Avg", avg_MGD_1, ..., avg_MGD_total
Row N+3:    "Max", max_MGD_1, ..., max_MGD_total
Row N+4:    "Min", min_MGD_1, ..., min_MGD_total

Column naming convention:

Individual wells: "BWell N|Flow Mgd" where N = 1–13

Total column: "BWP|Flow Mgd|MGD|Formula" (BWP = Buckman Well Production, formula-computed sum)

## 5. Key Constants & Conversions

| Constant | Value | Units | Derivation | Source |
| --- | --- | --- | --- | --- |
| MG_TO_AF_FACTOR | 3.06889 | AF/MG | 1,000,000 gal / 325,851 gal/AF | USGS |
| NOISE_THRESHOLD_MGD | 0.0015 | MGD | 1,500 gal/day; below flow meter precision for production wells | Observed database rounding artifacts (900-1,500 gal range) |
| DAILY_SUM_TOLERANCE_INFO_MGD | 0.001 | MGD | 1,000 gal/day; typical Excel formula rounding | Empirical from historical CSV files |
| DAILY_SUM_TOLERANCE_ERROR_MGD | 0.005 | MGD | 5,000 gal/day; ~0.2% of average daily production | Engineering judgment |
| ANNUAL_SUM_TOLERANCE_MG | 0.01 | MG | 10,000 gal; accumulated rounding from 366 daily values | Empirical from historical CSV files |

### Dimensional Analysis

MGD to monthly MG:

MGD --> MG (monthly)

SUM_{day=1}^{N} (MGD_day) = MG_month

Units: [MG/day] * [day] = [MG]
       The sum of N daily MGD values = N-day volume in MG

MG to AF:

MG --> AF

AF = MG * (1,000,000 gal/MG) / (325,851 gal/AF)
   = MG * 3.06889 AF/MG

Complete chain: Daily MGD to Annual AF

Daily MGD --> Monthly MG --> Monthly AF --> Annual AF

Step 1: Monthly_MG = SUM(Daily_MGD)         [sum over days in month]
Step 2: Monthly_AF = Monthly_MG * 3.06889   [unit conversion]
Step 3: Annual_AF  = SUM(Monthly_AF)        [sum over 12 months]

## 6. Domain-Specific Constraints

### Physical Bounds

| Parameter | Min | Max | Units | Basis |
| --- | --- | --- | --- | --- |
| Daily well production | 0.0 | ~10.0 | MGD | Zero when shut down; max assuming 1–2 dominate pumping |
| Daily total production (BWP) | 0.0 | ~30.0 | MGD | Sum of 13 wells; historical max ~25 MGD |
| Monthly well production | 0.0 | ~310.0 | MG | 10 MGD * 31 days = 310 MG theoretical max |
| Annual well production | 0.0 | ~3,650.0 | MG | 10 MGD * 365 days theoretical max per well |
| Annual total production | 0.0 | ~5,000.0 | AF | Historical range (1988–2024): 1,500–4,000 AF/year |

### Conservation Laws

Mass balance: Sum of 13 individual well daily totals must equal BWP daily total (within tolerance), which is verified at three tiers: noise (0.0015 MGD), info (0.001 MGD), and error (0.005 MGD).

Temporal consistency: Monthly MG totals summed over 12 months must equal the CSV annual Sum row value for each well (within 0.01 MG).

### Well Permitting

Buckman Wellfield operates under OSE permit RG-20516-S (base permit) and supplemental permits -S-2 through -S-13.

Annual reporting to the OSE includes requirement for historical annual pumping (Table 1) and monthly pumping for current year (Table 2), both in acre-feet.

## 7. Well Inventory

### 7.1 Buckman Wells and OSE Permit Numbers

| Well # | OSE Permit Number | MODFLOW Name | Report Label | Notes |
| --- | --- | --- | --- | --- |
| 1 | RG-20516-S-5 | BUCKMAN 1 | Buckman #1 | Primary production well |
| 2 | RG-20516-S-6 | BUCKMAN 2 | Buckman #2 |  |
| 3 | RG-20516-S | BUCKMAN 3A | Buckman #3/3A | Combined wells 3 and 3A; base permit |
| 4 | RG-20516-S-2 | BUCKMAN 4 | Buckman #4 |  |
| 5 | RG-20516-S-3 | BUCKMAN 5 | Buckman #5 |  |
| 6 | RG-20516-S-4 | BUCKMAN 6 | Buckman #6 |  |
| 7 | RG-20516-S-7 | BUCKMAN 7 | Buckman #7 | Primary production well |
| 8 | RG-20516-S-8 | BUCKMAN 8 | Buckman #8 | Primary production well |
| 9 | RG-20516-S-9 | BUCKMAN 9 | Buckman #9 |  |
| 10 | RG-20516-S-10 | BUCKMAN 10 | Buckman #10 | High-capacity production well |
| 11 | RG-20516-S-11 | BUCKMAN 11 | Buckman #11 | High-capacity production well |
| 12 | RG-20516-S-12 | BUCKMAN 12 | Buckman #12 | High-capacity production well |
| 13 | RG-20516-S-13 | BUCKMAN 13 | Buckman #13 | High-capacity production well |

### 7.2 CSV Column Mapping

Each well maps to a CSV column with the header format "BWell N|Flow Mgd":

"BWell 1|Flow Mgd"   --> Well 1  (RG-20516-S-5)
"BWell 2|Flow Mgd"   --> Well 2  (RG-20516-S-6)
"BWell 3|Flow Mgd"   --> Well 3  (RG-20516-S, reported as 3/3A)
"BWell 4|Flow Mgd"   --> Well 4  (RG-20516-S-2)
...
"BWell 13|Flow Mgd"  --> Well 13 (RG-20516-S-13)

"BWP|Flow Mgd|MGD|Formula"  --> Total (formula-computed sum of all wells)

## 8. Data Validation Framework

### 8.1 Three-Tier Daily Sum Verification

Each day, the sum of 13 individual well values is compared to the BWP total column from the CSV. The BWP column is the City’s own formula, not an external reference.

Tier 1 – Noise Floor (database precision artifacts):

Condition: 0 < BWP < 0.0015 MGD

Interpretation: Below instrument precision; 1,500 gal/day is within the 100-gallon rounding range of the SCADA system

Action: Log as INFO; no impact on monthly totals

Physical meaning: Database stores values with ~100-gallon precision, producing artifacts in the 900-1,500 gal/day range

Tier 2 – Formula Error (logical inconsistency):

Condition: All 13 wells = 0 but BWP >= 0.0015 MGD

Interpretation: Impossible physical state; BWP should be zero when no wells are pumping

Action: Flag as ERROR; requires CSV review

Physical meaning: The City’s formula column has a bug or data entry error

Tier 3 – Precision Tolerance (Excel rounding):

Condition: |SUM(wells)

BWP| evaluated against thresholds

OK: difference <= 0.001 MGD (1,000 gal/day)

INFO: 0.001 < difference <= 0.005 MGD - ERROR: difference > 0.005 MGD (5,000 gal/day, ~0.2% of average daily production)

Physical meaning: Expected rounding differences from Excel formula precision

### 8.2 Annual Sum Verification

For each well, the sum of 12 monthly MG values is compared to the CSV Sum row:

|SUM(monthly_MG) - CSV_Sum_MG| <= 0.01 MG  (10,000 gallons)

This catches systematic rounding accumulation across the year.

### 8.3 Data Quality Flags

Each daily value receives a quality flag:

| Flag | Meaning | Action |
| --- | --- | --- |
| (empty) | Valid data point | Use in aggregation |
| BLANK | Missing/NaN value | Include in sum as 0; mark month as Has_Flagged_Data |
| NEGATIVE | Negative flow value | Include in sum; mark month as Has_Flagged_Data |

If any day in a month has a flag, the entire month is marked Has_Flagged_Data=True and the monthly CSV output shows Data_Quality=FLAGGED for that well.

## 9. Output Products

### 9.1 Table 1: Historical Annual Pumping

- Content: Annual pumping by well in acre-feet per year (AF/yr)

- Format: Wells 1–13 as columns, years as rows (one row added per year)

- Well 3 label: “3/3A” in the column header (combined wells)

- Files: {year}_Table_1_updated.csv and {year}_Table_1_updated.xlsx

- Purpose: Historical trend analysis for OSE annual report

### 9.2 Table 2: Monthly Pumping for Current Year

- Content: Monthly pumping by well in acre-feet per month (AF/month)

- Format: Wells 1–13 as rows, months JAN-DEC as columns, with Total column

- Files: {year}_Table_2_output.csv and {year}_Table_2_output.xlsx

- Purpose: Input to MODFLOW well package update (Step 2); OSE annual report

- Downstream dependency: step2_update_modflow.py reads this file directly

### 9.3 Monthly CSVs

- Content: Per-well MG and AF values with data quality flags for one month

- Format: 13 well rows + Calculated_Sum row

- Files: {year}_{MM}_{MON}.csv (e.g., 2024_07_JUL.csv)

- Purpose: Intermediate data verification

### 9.4 Input Summary

- Content: Data quality summary across all wells and months

- File: input_summary.csv

- Purpose: Audit trail for flagged data

## 10. Dimensional Safety

The pipeline uses the Python pint library (Grecco, Hernán, 2026) for unit tracking:

ureg = UnitRegistry()
ureg.define('million_gallon = 1e6 * gallon = MG')
ureg.define('million_gallon_per_day = million_gallon / day = MGD')

This ensures that: 1. Daily MGD values multiplied by day units yield MG volumes 2. MG-to-AF conversion uses the pint library’s built-in gallon-to-acre-foot relationship 3. Unit mismatches raise DimensionalityError at runtime rather than producing silent numerical errors

## 11. Data Sources

City of Santa Fe, Buckman Wellfield SCADA System, daily production CSV files delivered via email annually

## 12. Cross-References

| Related Specification | Relationship |
| --- | --- |
| DS-02 | Table 2 output (AF/month) is the input to MODFLOW well package update |
| DS-03 | Pumping data drives all stream depletion calculations downstream |
| IS-01 | Implementation spec for step1_ingest_buckman_data.py |

Document Maintenance

Next Review: When CSV format changes or new wells are added to the Buckman Wellfield - Change Triggers: New well drilled (update WELL_OSE_MAP); CSV column format change; USGS conversion factor revision; change in OSE reporting requirements

# DS-02: MODFLOW96 Santa Fe Local-Scale Model

## 1. Purpose & Scope

This specification documents the MODFLOW96 groundwater flow model (Barroll, Peggy and Keyes, Eric, 2005) used to simulate Buckman Wellfield pumping effects on streams and springs in the Santa Fe area. The model is a local-scale, two-layer finite-difference representation of the Santa Fe Group aquifer system. Each year, monthly pumping data from Table 2 (see DS-01) is converted to MODFLOW well package rates, the model is executed, and binary flux outputs are passed to a FORTRAN post-processor for stream depletion calculations (see DS-03). This is the computational engine at the center of the Buckman depletion workflow pipelinee.

Critical distinction: This pipeline uses the original MODFLOW96 executable (1996 release), not MODFLOW-2005, MODFLOW-6, or FloPy. The model was built in the early 2000s and has been maintained as a frozen configuration with annual well-package updates only. All file formats, solver settings, and package structures follow MODFLOW96 conventions.

## 2. Scientific Basis

### 2.1 Key Equations – Pumping Rate Conversion

Well pumping is converted from acre-feet per month (from Table 2) to cubic feet per second (MODFLOW internal units):

rate_ft3s = -(AF / N_layers) * 43,560 ft^3/AF / (days_in_month * 86,400 s/day)

Where:
  AF             = monthly pumping in acre-feet (from Table 2)
  N_layers       = 2 (pumping split equally between Layer 1 and Layer 2)
  43,560         = ft^3 per acre-foot
  days_in_month  = calendar days in the month (28-31)
  86,400         = seconds per day
  Negative sign  = MODFLOW convention for extraction (pumping OUT of aquifer)

### 2.2 Worked Example

Convert Well 1 January 2024 pumping (16.887963 AF) to MODFLOW rate:

Step 1: Split between layers
  AF_per_layer = 16.887963 / 2 = 8.443982 AF

Step 2: Convert to ft^3
  volume_ft3 = 8.443982 * 43,560 = 367,771.4 ft^3

Step 3: Convert to rate (ft^3/s)
  seconds_in_jan = 31 * 86,400 = 2,678,400 s
  rate = 367,771.4 / 2,678,400 = 0.13730 ft^3/s

Step 4: Apply sign convention
  MODFLOW_rate = -0.13730 ft^3/s

Each layer receives -0.13730 ft^3/s for January 2024.

### 2.3 Boundary Condition Physics

River (RIV) Package – Tributaries:

Q_riv = C_riv * (h - h_riv)    when h > R_bot
Q_riv = C_riv * (R_bot - h_riv) when h <= R_bot

Where:
  Q_riv  = flux between aquifer and river [L^3/T]
  C_riv  = riverbed conductance [L^2/T]
  h      = aquifer head in the cell [L]
  h_riv  = river stage [L]
  R_bot  = riverbed bottom elevation [L]

General Head Boundary (GHB) Package – La Cienega Springs:

Q_ghb = C_ghb * (h - h_ghb)

Where:
  Q_ghb  = flux between aquifer and boundary [L^3/T]
  C_ghb  = boundary conductance [L^2/T]
  h      = aquifer head in the cell [L]
  h_ghb  = specified head at boundary [L]

Positive flux indicates reduced spring discharge (depletion from pumping).

## 3. Assumptions

| # | Assumption | Justification | Risk if Wrong | Mitigation |
| --- | --- | --- | --- | --- |
| 1 | Pumping is constant throughout each month (steady-state stress periods) | Monthly-average pumping rates are sufficient for annual depletion reporting; daily transient effects average out | Underestimates peak depletion effects during high-pumping days | Acceptable for annual compliance; daily transient model would add complexity without regulatory benefit |
| 2 | Pumping is split equally between Layer 1 and Layer 2 | Well screens span both layers in the model; equal split is a simplification | Actual drawdown distribution may differ from modeled | Long-standing model convention; changing requires re-calibration |
| 3 | Model grid and boundary conditions are static (unchanged since 2003) | Only the well package is updated annually; all other packages use frozen 2003 calibration | Geological or hydrological changes not captured | Low risk for compliance-scale depletion estimates |
| 4 | MODFLOW96 solver convergence is adequate | SIP solver converges for the stress magnitudes typical of Buckman pumping | Non-convergence would produce invalid heads and fluxes | Listing file (.lst) checked for convergence warnings |
| 5 | CRLF line endings are required | MODFLOW96 is a DOS/Windows executable; expects Windows text format | Wrong line endings could cause parsing errors | All generated files use \r\n explicitly |
| 6 | Binary flux files (.flx) are correctly formatted for sfmodflx_2245.exe | The FORTRAN post-processor reads MODFLOW binary output | Format mismatch would produce wrong depletion values or crashes | Validated against historical results |
| 7 | Wine on Linux/WSL executes MODFLOW96 identically to native Windows | Binary is run via wine modflow96.exe on Linux/WSL environments | Floating-point differences could affect convergence | Results validated against native Windows runs |

## 4. Data Sources & Provenance

| Source | Description | Access Method | Update Frequency | QC Procedure |
| --- | --- | --- | --- | --- |
| Table 2 CSV | Monthly pumping by well in AF ({year}_Table_2_output.csv) | Output from Step 1 (DS-01) | Annually | Validated in Step 1 before use |
| 2023 MODFLOW files | Static MODFLOW input files (BCF, BAS, GHB, RIV, OC, SIP) | input/modflow/2023/ directory | Never (frozen calibration) | Checksummed at pipeline deployment |
| Prior year WEL file | Well package from previous year’s run | output/modflow/{year-1}/thruCY2165_{year-1}.wel | Annually (chained from prior run) | Structure validated during parsing |
| MODFLOW96.exe | Original 1996 MODFLOW executable | input/modflow/2023/modflow96.exe | Never (frozen executable) | Binary checksum |

## 5. Key Constants & Conversions

| Constant | Value | Units | Derivation | Source |
| --- | --- | --- | --- | --- |
| ACRE_FT_TO_FT3 | 43,560 | ft^3/AF | 1 acre = 43,560 ft^2; 1 AF = 1 acre * 1 ft | Standard |
| SECONDS_PER_DAY | 86,400 | s/day | 24 * 60 * 60 | Standard |
| NUM_LAYERS | 2 | dimensionless | Model has 2 aquifer layers; pumping split equally | Core (2003) |
| WELLS_PER_MONTH | 26 | entries | 13 wells * 2 layers | Model structure |
| LINES_PER_MONTH | 27 | lines | 1 header + 26 well entries | WEL file format |
| BASELINE_YEAR | 2024 | year | First year using original 2023 input files | Pipeline convention |

### Dimensional Analysis – AF to ft^3/s

AF --> ft^3/s (per layer)

rate = (AF / N_layers) * (43,560 ft^3/AF) / (days * 86,400 s/day)

Cancel units:
  [AF] * [ft^3/AF] / [day * s/day]
= [ft^3] / [s]
= ft^3/s

Example: 16.887963 AF, January (31 days), 2 layers
  = (16.887963/2) * 43560 / (31 * 86400)
  = 8.443982 * 43560 / 2678400
  = 367771.4 / 2678400
  = 0.13730 ft^3/s

Apply negative sign: -0.13730 ft^3/s (MODFLOW extraction convention)

## 6. Model Grid Specification

### 6.1 Grid Dimensions

Rows:    35
Columns: 25
Layers:  2

Total cells: 35 * 25 * 2 = 1,750

Note: The Buckman Wellfield wells occupy rows 13–20, columns 11–16 within the active wellfield area. However, the full model domain extends to at least row 35 to accommodate GHB (General Head Boundary) cells representing La Cienega Springs at rows 30–32, and the FORTRAN post-processor GHB extraction rectangle spans rows 28–35, columns 10–20. The RIV (River) package cells span rows 9–17, columns 14–25. Rio Grande cells extend from row 1 (Above Otowi) through row 23 (Below Otowi). Not all cells in the 35x25 grid are necessarily active; inactive cells are defined in the BAS (Basic) package boundary array.

### 6.2 Well Locations (WELL_GRID_MAP)

Each well occupies two cells (Layer 1 and Layer 2) at the same row/column:

| Well Name | Well # | Row | Column | Notes |
| --- | --- | --- | --- | --- |
| BUCKMAN 1 | 1 | 13 | 11 | Co-located with Wells 7, 8 |
| BUCKMAN 2 | 2 | 14 | 11 | Co-located with Wells 3A, 4 |
| BUCKMAN 3A | 3 | 14 | 11 | Co-located with Wells 2, 4 |
| BUCKMAN 4 | 4 | 14 | 11 | Co-located with Wells 2, 3A |
| BUCKMAN 5 | 5 | 15 | 12 |  |
| BUCKMAN 6 | 6 | 14 | 12 | Co-located with Well 9 |
| BUCKMAN 7 | 7 | 13 | 11 | Co-located with Wells 1, 8 |
| BUCKMAN 8 | 8 | 13 | 11 | Co-located with Wells 1, 7 |
| BUCKMAN 9 | 9 | 14 | 12 | Co-located with Well 6 |
| BUCKMAN 10 | 10 | 17 | 13 |  |
| BUCKMAN 11 | 11 | 19 | 14 |  |
| BUCKMAN 12 | 12 | 19 | 15 |  |
| BUCKMAN 13 | 13 | 20 | 16 |  |

Note on co-located wells: Multiple wells in the same grid cell is a consequence of model resolution. The 35x25 grid represents the regional aquifer at a scale where some closely-spaced Buckman wells fall within the same cell. Pumping rates are additive within each cell.

### 6.3 Boundary Condition Cells

River (RIV) Package – Tributaries:

Spatial extent: Rows 9–17, Columns 14–25

Streams represented: Rio Pojoaque, Rio Tesuque, Rio Grande

Stream assignment determined by FORTRAN post-processor (see DS-03)

General Head Boundary (GHB) Package – La Cienega Springs:

6 cells representing spring discharge zones:

| Cell # | Layer | Row | Column | Head (ft) | Conductance (ft^2/day) |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 30 | 14 | 5449.0 | 100.0 |
| 2 | 1 | 31 | 12 | 5447.0 | 100.0 |
| 3 | 1 | 31 | 14 | 5447.0 | 100.0 |
| 4 | 1 | 31 | 15 | 5447.0 | 100.0 |
| 5 | 1 | 32 | 13 | 5445.0 | 100.0 |
| 6 | 1 | 32 | 12 | 5445.0 | 100.0 |

Spatial extent: Rows 30-32, Columns 12-15, all in Layer 1.

## 7. Stress Period Structure

### 7.1 Monthly Stress Periods

The model uses 12 stress periods per year, one for each calendar month. Each stress period has a constant pumping rate for all wells.

Within the .wel file, each month is structured as:

26                                    <-- Header: 26 well entries follow
         1        13        11  -0.13730  BUCKMAN 1 JAN 2024    <-- Layer 1
         2        13        11  -0.13730  BUCKMAN 1 JAN 2024    <-- Layer 2
         1        14        11  -0.31553  BUCKMAN 2 JAN 2024    <-- Layer 1
         2        14        11  -0.31553  BUCKMAN 2 JAN 2024    <-- Layer 2
         ...
         1        20        16  -0.00000  BUCKMAN 13 JAN 2024   <-- Layer 1
         2        20        16  -0.00000  BUCKMAN 13 JAN 2024   <-- Layer 2

Entry format: {layer:10d}{row:10d}{col:10d}  {rate:8.5f}  {well_name} {month} {year}

### 7.2 Annual Structure

- 12 months per year

- 27 lines per month (1 header + 26 entries)

- 324 lines per year (12 * 27)

- Wells ordered 1-13; each well has Layer 1 entry followed by Layer 2

### 7.3 Year Chaining

The .wel file is cumulative, containing data from 1988 through the current year plus projections through 2165. Each year, the target year’s 324 lines are replaced with actual pumping data:

thruCY2165.wel structure:
  [Lines for 1988-{year-1}]   <-- Historical data (preserved from prior runs)
  [324 lines for {year}]      <-- Updated with actual pumping from Table 2
  [Lines for {year+1}-2165]   <-- Projection data (preserved from prior runs)

For the baseline year (2024), input comes from input/modflow/2023/thruCY2165.wel. For subsequent years, input comes from output/modflow/{year-1}/thruCY2165_{year-1}.wel.

### 7.4 Zero Pumping Convention

When a well has zero pumping for a month, the rate is formatted as -0.00000 (negative zero with 5 decimal places) to match the validation file format and maintain the MODFLOW sign convention.

## 8. Model Execution

### 8.1 Baseline Files

Ten files are copied from input/modflow/2023/ to output/modflow/{year}/ before each run:

| File | Package | Purpose |
| --- | --- | --- |
| modflow96.exe | Executable | MODFLOW96 solver |
| sflcs.bcf | Block-Centered Flow | Hydraulic properties (K, Ss, layer types) |
| sflcs.sip | SIP Solver | Solver parameters (convergence criteria, iterations) |
| thruCY2165.bas | Basic | Grid dimensions, boundary array, initial heads |
| thruCY2165.ghb | General Head Boundary | La Cienega Springs cells |
| thruCY2165.oc | Output Control | What to save and when |
| thruCY2165.riv | River | Tributary stream cells |
| sfmodflx_2245.exe | Post-processor | FORTRAN depletion calculator |
| verify_modflow_run.py | Verification | Output validation script |
| verify_depletion.py | Verification | Depletion validation script |

### 8.2 NAM File (Name File)

The .nam file tells MODFLOW96 which packages and files to use:

# File Type      Unit File Name
LIST            23    CY{year}.lst
BAS             21    thruCY2165.bas
BCF             11    sflcs.bcf
OC              10    thruCY2165.oc
RIV             14    thruCY2165.riv
GHB             15    thruCY2165.ghb
SIP             17    sflcs.sip
WEL             12    thruCY2165_{year}.wel
DATA(BINARY)    24    CY{year}_riv.flx
DATA(BINARY)    34    CY{year}_ghb.flx

Year-specific files: .lst (listing), .wel (wells), _riv.flx and _ghb.flx (binary flux outputs). Static files: .bas, .bcf, .oc, .riv, .ghb, .sip (unchanged between years).

### 8.3 Execution Environment

- Platform: Windows executable run via Wine on Linux/WSL

- Command: wine modflow96.exe < CY{year}.nam

- Runtime: Under 1 minute for full simulation (1988–2165)

- Output: .lst listing file + binary flux files (_riv.flx, _ghb.flx)

### 8.4 Model Outputs

| Output File | Format | Size | Content |
| --- | --- | --- | --- |
| CY{year}.lst | Text | ~50 MB | Listing file with convergence info, water budget |
| CY{year}_riv.flx | Binary | ~31 MB | River package cell-by-cell flux for all stress periods |
| CY{year}_ghb.flx | Binary | ~31 MB | GHB package cell-by-cell flux for all stress periods |

The binary .flx files are read by the FORTRAN post-processor (sfmodflx_2245.exe) to extract stream depletions (see DS-03).

## 9. Domain-Specific Constraints

### Physical Bounds

| Parameter | Min | Max | Units | Basis |
| --- | --- | --- | --- | --- |
| Well pumping rate (per layer) | -10.0 | 0.0 | ft^3/s | Negative = extraction; magnitude limited by pump capacity |
| Days in month | 28 | 31 | days | Calendar |
| Stress periods per year | 12 | 12 | count | Monthly time steps |
| Well entries per stress period | 26 | 26 | count | 13 wells * 2 layers |
| GHB head values | 5445.0 | 5449.0 | ft | La Cienega Springs elevation range |
| GHB conductance | 100.0 | 100.0 | ft^2/day | Calibrated value (all cells identical) |

### Conservation Laws

- Water budget: MODFLOW internally enforces mass balance. The listing file reports total inflow, total outflow, and percent discrepancy for each stress period. Discrepancy > 1% indicates convergence problems.

- Flux sign convention: Positive flux = flow INTO aquifer; Negative flux = flow OUT of aquifer (pumping).

### File Format Constraints

- Line endings: CRLF (\r\n) required for Windows/MODFLOW96 compatibility

- Column alignment: Well entry fields must be exactly formatted ({layer:10d}{row:10d}{col:10d}) for MODFLOW96’s fixed-format reader

- Rate precision: 5 decimal places ({rate:8.5f}) matching historical file format

## 10. References

### Data Sources

- City of Santa Fe, Buckman Wellfield pumping records (via DS-01 ingestion)

## 11. Cross-References

| Related Spec | Relationship |
| --- | --- |
| DS-01 | Table 2 output provides monthly AF values that become MODFLOW well rates |
| DS-03 | MODFLOW binary flux outputs are input to stream depletion analysis |
| IS-02 | Implementation spec for step2_update_modflow.py |

Document Maintenance: - Next Review: If model grid changes, solver is upgraded, or additional wells are drilled - Change Triggers: New well added to WELL_GRID_MAP; model re-calibration; MODFLOW version upgrade; change in layer structure

# DS-03: Stream Depletion Analysis

## 1. Purpose & Scope

This specification documents the stream depletion analysis that converts MODFLOW boundary-condition fluxes into depletion tables for the OSE annual compliance report. The analysis quantifies how Buckman Wellfield pumping depletes flows in the Rio Grande, Rio Pojoaque-Nambe, Rio Tesuque, and La Cienega Springs. It combines numerical MODFLOW results (1988–present pumping via superposition) with analytical residuals from Core (2003) for pre-1988 pumping effects. The outputs are Tables 3, 4, and 5 of the annual depletion report.

## 2. Scientific Basis

### 2.1 Superposition Principle

The MODFLOW model calculates stream depletions from 1988 through the current year using the superposition principle: the model simulates only the change in heads and fluxes caused by Buckman pumping, relative to a no-pumping baseline. Stream depletion is the difference in boundary-condition flux between the pumping and no-pumping scenarios.

Because the aquifer flow equation is linear (for confined conditions or when head changes are small relative to saturated thickness), effects of different pumping periods can be superposed:

Total Depletion = Depletion(pre-1988 pumping) + Depletion(1988-present pumping)
                = Core (2003) Analytical Residuals + MODFLOW Superposition Results

### 2.2 Key Equations

cfs to acre-feet conversion:

AF = cfs * days * 86,400 s/day / 43,560 ft^3/AF
   = cfs * days * 1.9835

Where:
  cfs   = cubic feet per second (monthly average depletion rate)
  days  = number of days in the month
  1.9835 = 86,400 / 43,560 (exact conversion factor)

Table 3 total impact:

Total_Impact_AF = Residual_AF + Superposition_AF

Where:
  Residual_AF      = Core (2003) analytical model value for pre-1988 pumping
  Superposition_AF = SUM over 12 months of (cfs_month * days_month * 1.9835)

Table 5 annual increment:

Annual_Increment_AF = Cumulative_AF(year) - Cumulative_AF(year - 1)

Where cumulative values come from MODFLOW GHB flux output.

Rio Tesuque residual (1988-2050):

CORE_2003_TESUQUE dict contains explicit values for all 63 years (1988-2050).
No extrapolation formula is needed.
Residual reaches 0.117 AF in 2050 and is 0.0 for years beyond 2050.

### 2.3 Worked Example – Table 3 (Rio Pojoaque, 2024)

Step 1: Look up Core (2003) residual for Rio Pojoaque in 2024
  Pojoaque residual exhausted after 2015 (dictionary ends at 2015: 0.316)
  For 2024: Residual_AF = 0.0 AF

Step 2: Extract MODFLOW superposition (R POJOAQUE monthly cfs)
  jan=0.083581, feb=0.083553, ..., dec=0.083xxx (12 values from post-processor)

Step 3: Convert each month from cfs to AF
  January: 0.083581 * 31 * 1.9835 = 5.137 AF
  February: 0.083553 * 29 * 1.9835 = 4.804 AF  (2024 is a leap year)
  ...sum all 12 months...

Step 4: Sum for annual superposition total
  Superposition_AF = SUM(monthly_AF) = ~60.797 AF (approximate)

Step 5: Calculate total impact
  Total_Impact_AF = 0.0 + 60.797 = 60.797 AF

### 2.4 Worked Example – cfs to AF Conversion

Given: 1.0 cfs flow for 30 days
  AF = 1.0 * 30 * 86400 / 43560
     = 1.0 * 30 * 1.9835
     = 59.505 AF

Verification: 1 cfs = 1 ft^3/s
  Volume = 1 ft^3/s * 30 days * 86400 s/day = 2,592,000 ft^3
  AF = 2,592,000 / 43,560 = 59.505 AF

## 3. Assumptions

| # | Assumption | Justification | Risk if Wrong | Mitigation |
| --- | --- | --- | --- | --- |
| 1 | Superposition is valid (aquifer response is linear) | Buckman pumping causes small head changes relative to total saturated thickness; aquifer behavior approximately linear | Non-linear effects would cause depletion estimates to be inaccurate | Model has been calibrated and validated |
| 2 | Core (2003) analytical residuals are correct | Published technical report with peer review; values manually transcribed from PROJECTION.XLS | Transcription errors would affect Table 3 totals | Values are hardcoded constants verified against source document |
| 3 | Pojoaque residual is zero after 2015 | Core (2003) table ends at 2015: 0.316; physical basis is that pre-1988 pumping effects have fully propagated | If residual still exists, Table 3 slightly underestimates Pojoaque depletion | Conservative for compliance (reports lower depletion than actual) |
| 4 | Tesuque residual values are explicit through 2050 | CORE_2003_TESUQUE dict contains 63 entries (1988-2050) derived from Core (2003) PROJECTION.XLS; residual reaches 0.117 AF in 2050 and is 0.0 after | No extrapolation needed; values beyond 2050 return 0.0 | Dict values verified against source document; linear decline pattern is consistent with late-time analytical solution behavior |
| 5 | FORTRAN post-processor cell ranges match actual MODFLOW cells | Hardcoded rectangles in sfmodflx_2245.for must encompass all RIV and GHB cells | Cells outside rectangles are silently excluded from depletion totals | See docs/MODFLOW_CELL_MAPPING.md for validation procedure |
| 6 | All GHB cells represent La Cienega Springs | No other GHB features in the model | Adding other GHB features would corrupt La Cienega totals | Model geometry has been stable since 2003 |
| 7 | La Cienega MODFLOW output is cumulative from 2004 | Annual increment = cum(N) - cum(N-1) | Wrong reference point would produce incorrect annual values | Historical cumulative values validated against published Table 5 (Wolaver, 2025) |
| 8 | Stream labels in FORTRAN output match Python parser expectations exactly | “R POJOAQUE”, “R TESUQUE”, “RIO GRANDE”, “RIV TOTAL”, “LC SPRINGS” | String mismatch causes ValueError at runtime (fail-fast) | Regex matching with whitespace normalization |

## 4. Data Sources & Provenance

| Source | Description | Access Method | Update Frequency | QC Procedure |
| --- | --- | --- | --- | --- |
| MODFLOW binary flux files | CY{year}_riv.flx and CY{year}_ghb.flx | Output from Step 3 (MODFLOW run) | Annually | Convergence check in .lst file |
| FORTRAN post-processor output | CY{year} formatted text file | Output from sfmodflx_2245.exe | Annually | Parsed and validated in Python |
| Core (2003) PROJECTION.XLS | Analytical residual values for pre-1988 pumping | Hardcoded dictionaries in stream_depletions.py | Static (2003 publication) | Manual verification against source document |
| Historical Table 3 baseline | validation/2024/expected_outputs/Table_3_expected.xlsx | Local filesystem | Annual update with validated results | Cross-checked against prior year’s published report |
| La Cienega cumulative values | LA_CIENEGA_CUMULATIVE dictionary | Hardcoded in stream_depletions.py | Annually (manually updated from Table 5 output) | Manual entry from validated Excel output |

## 5. Key Constants & Conversions

| Constant | Value | Units | Derivation | Source |
| --- | --- | --- | --- | --- |
| cfs-to-AF factor | 1.9835 | AF/(cfs*day) | 86,400 s/day / 43,560 ft^3/AF | Standard unit conversion |
| SECONDS_PER_DAY | 86,400 | s/day | 24 * 60 * 60 | Standard |
| FT3_PER_AF | 43,560 | ft^3/AF | 1 acre * 1 foot | Standard |

### Dimensional Analysis – cfs to AF

cfs --> AF (for N days)

AF = cfs * N_days * 86,400 s/day / 43,560 ft^3/AF

Cancel units:
  [ft^3/s] * [days] * [s/day] / [ft^3/AF]
= [ft^3] / [ft^3/AF]
= [AF]

Simplified: AF = cfs * N_days * 1.9835

## 6. Three-Layer Data Processing System

Stream depletions flow through three independent processing layers. Each layer defines cell-to-stream assignments independently, creating a fragile chain where consistency must be manually maintained.

### 6.1 Layer 1: MODFLOW Input Files

MODFLOW GHB and RIV package files define the physical locations of boundary condition cells:

- RIV package (thruCY2165.riv): Tributary stream cells at rows 9-17, columns 14-25

- GHB package (thruCY2165.ghb): La Cienega Springs cells at rows 30-32, columns 12-15

These files contain only (layer, row, col, head, conductance) – no stream labels.

### 6.2 Layer 2: FORTRAN Post-Processor (sfmodflx_2245.exe)

The FORTRAN post-processor reads MODFLOW binary flux files and assigns cells to streams using hardcoded spatial rules.

Cell extraction rectangles (SUBROUTINE FILL):

| Package | Start Row | End Row | Start Col | End Col | Streams |
| --- | --- | --- | --- | --- | --- |
| RIV (JJ2=1) | 9 | 17 | 14 | 25 | R POJOAQUE, R TESUQUE, RIO GRANDE |
| GHB (JJ2=2) | 28 | 35 | 10 | 20 | LC SPRINGS |

Stream assignment logic (SUBROUTINE TOTAL):

For RIV cells within rows 9-17, columns 14-25: - R POJOAQUE: (row == 9 AND col > 13) OR (row < 13 AND col > 21) - R TESUQUE: col > 18 (and not already assigned to Pojoaque) - RIO GRANDE: All other RIV cells - RIV TOTAL: Sum of R POJOAQUE + R TESUQUE + RIO GRANDE

For GHB cells:

LC SPRINGS: Sum of ALL GHB cells (no spatial logic; all GHB cells are La Cienega)

Output format:

YEAR: 2024        jan         feb        ...        dec
  LAY ROW COL
    1   9  14    0.025737    0.025725   ...    0.025xxx
    1   9  15    0.013421    0.013415   ...    0.013xxx
    ...
0  R POJOAQUE    0.083581    0.083553   ...    0.083xxx
0  R TESUQUE     0.141234    0.141198   ...    0.141xxx
0  RIO GRANDE    0.562847    0.562791   ...    0.562xxx
0  RIV  TOTAL    0.787662    0.787542   ...    0.787xxx
0  LC SPRINGS    0.083581    0.083581   ...    0.083xxx

### 6.3 Layer 3: Python Parser (stream_depletions.py)

The Python parser reads the FORTRAN output text file using regex matching:

Stream name registry:

"R POJOAQUE"   -- Rio Pojoaque-Nambe
"R TESUQUE"    -- Rio Tesuque
"RIO GRANDE"   -- Rio Grande main stem
"RIV TOTAL"    -- Total river depletions (sum of above 3)
"LC SPRINGS"   -- La Cienega Springs (GHB package)

Parsing rules: - Cell data lines: ^\s+(\d+)\s+(\d+)\s+(\d+)\s+(12 float values) - Stream summary lines: ^0\s+(stream_name)\s+(12 float values) - Year headers: ^YEAR:\s+(\d{4})\s+jan - Whitespace in stream names normalized (e.g., "RIV  TOTAL" becomes "RIV TOTAL")

### 6.4 Critical Risk: No Automatic Connection Between Layers

There is NO automated validation that FORTRAN cell ranges match actual MODFLOW cell locations. If the model geometry changes (e.g., a new GHB cell at row 27), the FORTRAN rectangle would miss it silently. See docs/MODFLOW_CELL_MAPPING.md for a complete analysis of this risk and mitigation procedures.

## 7. Core (2003) Analytical Residuals

### 7.1 Rio Pojoaque-Nambe Residuals

Decreasing residual effect from pre-1988 (specifically 1972-1987) Buckman pumping on Rio Pojoaque-Nambe. Values from Core (2003) PROJECTION.XLS:

| Year | Residual (AF) |  | Year | Residual (AF) |
| --- | --- | --- | --- | --- |
| 1988 | 40.432 |  | 2002 | 17.753 |
| 1989 | 39.244 |  | 2003 | 16.543 |
| 1990 | 37.971 |  | 2004 | 15.429 |
| 1991 | 36.557 |  | 2005 | 14.404 |
| 1992 | 34.928 |  | 2006 | 13.462 |
| 1993 | 33.112 |  | 2007 | 12.595 |
| 1994 | 31.185 |  | 2008 | 11.797 |
| 1995 | 29.226 |  | 2009 | 11.061 |
| 1996 | 27.296 |  | 2010 | 10.383 |
| 1997 | 25.439 |  | 2011 | 6.151 |
| 1998 | 23.678 |  | 2012 | 4.693 |
| 1999 | 22.028 |  | 2013 | 3.234 |
| 2000 | 20.491 |  | 2014 | 1.775 |
| 2001 | 19.068 |  | 2015 | 0.316 |

After 2015: Residual = 0.0 AF (effect fully exhausted).

### 7.2 Rio Tesuque Residuals

Longer-lasting residual effect on Rio Tesuque. Initial values increase (peaking around 1995-1996 at ~25.7 AF) before declining:

| Year | Residual (AF) |  | Year | Residual (AF) |
| --- | --- | --- | --- | --- |
| 1988 | 21.015 |  | 2010 | 19.639 |
| 1989 | 22.333 |  | 2011 | 19.258 |
| 1990 | 23.391 |  | 2012 | 18.767 |
| 1991 | 24.227 |  | 2013 | 18.276 |
| 1992 | 24.868 |  | 2014 | 17.785 |
| 1993 | 25.327 |  | 2015 | 17.295 |
| 1994 | 25.615 |  | 2016 | 16.804 |
| 1995 | 25.747 |  | 2017 | 16.313 |
| 1996 | 25.737 |  | 2018 | 15.822 |
| 1997 | 25.608 |  | 2019 | 15.331 |
| 1998 | 25.378 |  | 2020 | 14.841 |
| 1999 | 25.067 |  | 2021 | 14.350 |
| 2000 | 24.691 |  | 2022 | 13.859 |
| 2001 | 24.265 |  | 2023 | 13.368 |
| 2002 | 23.800 |  | 2024 | 12.877 |
| 2003 | 23.308 |  | 2025 | 12.387 |
| 2004 | 22.797 |  | 2026 | 11.896 |
| 2005 | 22.273 |  | 2027 | 11.405 |
| 2006 | 21.743 |  | 2028 | 10.914 |
| 2007 | 21.212 |  | 2029 | 10.424 |
| 2008 | 20.683 |  | 2030 | 9.933 |
| 2009 | 20.157 |  |  |  |

2031-2050: Explicit values continue in CORE_2003_TESUQUE dict (63 entries total, 1988-2050):

| Year | Residual (AF) |  | Year | Residual (AF) |
| --- | --- | --- | --- | --- |
| 2031 | 9.442 |  | 2041 | 4.534 |
| 2032 | 8.951 |  | 2042 | 4.043 |
| 2033 | 8.460 |  | 2043 | 3.552 |
| 2034 | 7.970 |  | 2044 | 3.062 |
| 2035 | 7.479 |  | 2045 | 2.571 |
| 2036 | 6.988 |  | 2046 | 2.080 |
| 2037 | 6.497 |  | 2047 | 1.589 |
| 2038 | 6.006 |  | 2048 | 1.098 |
| 2039 | 5.516 |  | 2049 | 0.608 |
| 2040 | 5.025 |  | 2050 | 0.117 |

After 2050: Residual = 0.0 AF (effect fully exhausted). The Tesuque residual reaches 0.117 AF in 2050, and get_analytical_residual() returns 0.0 for all years beyond 2050 via .get(year, 0.0).

## 8. Otowi Gage Cell Definitions

Table 4 divides the Rio Grande into reaches above and below the Otowi Bridge stream gage:

### 8.1 Above Otowi Cells (10 cells)

| # | Layer | Row | Column |
| --- | --- | --- | --- |
| 1 | 1 | 1 | 16 |
| 2 | 1 | 2 | 16 |
| 3 | 1 | 3 | 16 |
| 4 | 1 | 4 | 16 |
| 5 | 1 | 5 | 15 |
| 6 | 1 | 6 | 14 |
| 7 | 1 | 7 | 14 |
| 8 | 1 | 8 | 13 |
| 9 | 1 | 9 | 13 |
| 10 | 1 | 10 | 12 |

### 8.2 Below Otowi Cells (16 cells)

| # | Layer | Row | Column |
| --- | --- | --- | --- |
| 1 | 1 | 11 | 11 |
| 2 | 1 | 12 | 11 |
| 3 | 1 | 13 | 11 |
| 4 | 1 | 14 | 10 |
| 5 | 1 | 15 | 9 |
| 6 | 1 | 15 | 10 |
| 7 | 1 | 16 | 9 |
| 8 | 1 | 17 | 8 |
| 9 | 1 | 18 | 6 |
| 10 | 1 | 18 | 7 |
| 11 | 1 | 19 | 6 |
| 12 | 1 | 20 | 5 |
| 13 | 1 | 21 | 4 |
| 14 | 1 | 21 | 5 |
| 15 | 1 | 22 | 4 |
| 16 | 1 | 23 | 3 |

### 8.3 Buckman Wells Cell

Cell (1, 13, 11) – the MODFLOW cell containing the Buckman wellfield cluster (Wells 1, 7, 8). This cell’s depletion is reported separately in Table 4 to show the direct impact at the wellfield location on the Rio Grande.

## 9. La Cienega Springs (Table 5)

### 9.1 Cumulative Depletion Baseline

La Cienega Springs depletion is tracked as a cumulative value starting from 2004. The MODFLOW GHB flux output represents cumulative impact from all 1988-present pumping. Annual increment is computed by differencing:

Annual_Increment(year) = Cumulative(year) - Cumulative(year - 1)

### 9.2 Historical Cumulative Values (LA_CIENEGA_CUMULATIVE)

These values are manually entered from official Table 5 outputs:

| Year | Cumulative AF | Annual Increment AF |
| --- | --- | --- |
| 2004 | 0.45 | 0.45 (first year) |
| 2005 | 0.66 | 0.21 |
| 2006 | 0.83 | 0.17 |
| 2007 | 0.99 | 0.16 |
| 2008 | 1.16 | 0.17 |
| 2009 | 1.32 | 0.16 |
| 2010 | 1.49 | 0.17 |
| 2011 | 1.65 | 0.16 |
| 2012 | 1.82 | 0.17 |
| 2013 | 1.97 | 0.15 |
| 2014 | 2.13 | 0.16 |
| 2015 | 2.29 | 0.16 |
| 2016 | 2.45 | 0.16 |
| 2017 | 2.60 | 0.15 |
| 2018 | 2.75 | 0.15 |
| 2019 | 2.90 | 0.15 |
| 2020 | 3.06 | 0.16 |
| 2021 | 3.21 | 0.15 |
| 2022 | 3.37 | 0.16 |
| 2023 | 3.54 | 0.17 |
| 2024 | 3.74 | 0.20 |
| 2025 | 3.92 | 0.18 |
| 2026 | 4.10 | 0.18 |
| 2027 | 4.27 | 0.17 |
| 2028 | 4.46 | 0.19 |
| 2029 | 4.62 | 0.16 |
| 2030 | 4.80 | 0.18 |

Key observations:

Annual increments are small (0.15-0.20 AF/yr), reflecting the distance from the wellfield to La Cienega Springs

Cumulative depletion is growing slowly and approximately linearly

Total cumulative depletion through 2030 is less than 5 AF

### 9.3 GHB Cell Configuration

Six GHB cells represent La Cienega Springs (see DS-02, Section 6.3): - Rows 30-32, Columns 12-15, all Layer 1 - Conductance: 100.0 ft^2/day (all cells) - Head elevation: 5,445-5,449 ft (decreasing southward)

### 9.4 Year Chaining for Table 5

Table 5 uses the prior year’s cumulative value to compute the current year’s annual increment. This creates a dependency chain:

Table 5 for year N requires:
  1. MODFLOW flux output for year N (cumulative cfs from GHB cells)
  2. LA_CIENEGA_CUMULATIVE[N-1] (prior year's cumulative AF)

The cumulative AF for year N is computed from the MODFLOW output, then the annual increment

= cumulative(N) - cumulative(N-1).

After validation, cumulative(N) is manually added to LA_CIENEGA_CUMULATIVE for use in the following year.

## 10. Output Products

### 10.1 Table 3: Tributary Stream Depletions

- Content: Rio Pojoaque-Nambe and Rio Tesuque depletion impacts (AF/year)

- Structure per stream: Residual (Analytical) + Superposition (MODFLOW) = Total Impact

- Time series: 1988 through current year (rows extended annually)

- Historical preservation: Years 1988 through (year-1) use values from baseline file to ensure exact reproduction

- File: TABLE_3_Rio_Pojoaque_Tesuque_{year}.xlsx

### 10.2 Table 4: Rio Grande Depletions Above and Below Otowi

- Content: Cell-level and aggregated Rio Grande depletions

- Above Otowi: 10 cells aggregated (monthly cfs + annual AF)

- Below Otowi: 16 cells aggregated (monthly cfs + annual AF)

- Total Rio Grande: Sum of above + below (monthly cfs + annual AF)

- Buckman cell: Cell (1,13,11) depletion reported separately

- Stream summaries: RIO GRANDE, R POJOAQUE, LC SPRINGS, R TESUQUE, RIV TOTAL

- Days per month: Uses non-leap year (365-day) convention for Table 4

- File: TABLE_4_Rio_Grande_{year}.xlsx

### 10.3 Table 5: La Cienega Springs

- Content: Cumulative and annual increment depletion (AF)

- Monthly detail: 12 monthly cfs values converted to AF

- Cumulative: Running total from 2004 through current year

- Annual increment: Current year minus prior year cumulative

- Uses leap year: Calendar-accurate days for conversion (unlike Table 4)

- File: TABLE_5_La_Cienega_Springs_{year}.xlsx

## 11. Domain-Specific Constraints

### Physical Bounds

| Parameter | Min | Max | Units | Basis |
| --- | --- | --- | --- | --- |
| Stream depletion rate | 0.0 | ~1.0 | cfs | Pumping-induced reduction in stream baseflow |
| Pojoaque annual depletion | 0.0 | ~100.0 | AF/yr | Historical range including residuals |
| Tesuque annual depletion | 0.0 | ~80.0 | AF/yr | Historical range including residuals |
| Rio Grande annual depletion | 0.0 | ~500.0 | AF/yr | Largest depletion (closest to wellfield) |
| La Cienega annual increment | 0.0 | ~0.5 | AF/yr | Very small due to distance from wellfield |
| La Cienega cumulative | 0.0 | ~10.0 | AF | Slow accumulation since 2004 |

### Conservation Laws

- Depletion non-negativity: Stream depletion from pumping cannot be negative (pumping cannot increase streamflow in a superposition model)

- Flux consistency: RIV TOTAL = R POJOAQUE + R TESUQUE + RIO GRANDE (sum of stream components)

- Above + Below = RIO GRANDE: Sum of Above Otowi cells + Below Otowi cells should equal the RIO GRANDE stream summary

- Cumulative monotonicity: La Cienega cumulative depletion should increase each year (pumping causes ongoing depletion that accumulates)

## 12. Leap Year Handling

The pipeline handles leap years differently for different tables:

| Table | February Days | Total Days | Rationale |
| --- | --- | --- | --- |
| Table 3 | Calendar-accurate (28 or 29) | 365 or 366 | Superposition conversion uses actual year |
| Table 4 | 28 (always) | 365 | Validation file convention uses non-leap year |
| Table 5 | Calendar-accurate (28 or 29) | 365 or 366 | La Cienega uses actual year |

This inconsistency is inherited from the original Excel-based workflow and preserved for backward compatibility with validation files.

## 13. References

### Data Sources

- MODFLOW binary flux files from Step 3 model execution

- Core (2003) PROJECTION.XLS (analytical residual tables)

- Historical Table 5 outputs (La Cienega cumulative values)

## 14. Cross-References

| Related Spec | Relationship |
| --- | --- |
| DS-01 | Well production data is the ultimate input to the MODFLOW model |
| DS-02 | MODFLOW model execution produces the binary flux files consumed here |
| IS-03 | Implementation spec for stream_depletions.py |
| docs/MODFLOW_CELL_MAPPING.md | Detailed documentation of the three-layer cell identification system |

Document Maintenance:

Next Review: When Core (2003) residuals are updated, new streams are added, or model geometry changes

Change Triggers: New analytical residual model replacing Core (2003); Otowi gage cell reclassification; La Cienega GHB cell changes; FORTRAN post-processor modification; change in OSE reporting requirements

# DS-04: Year-Chaining & Temporal Architecture

## 1. Purpose & Scope

The Buckman Wellfield depletion pipeline processes years sequentially, building on prior results. Each year’s MODFLOW simulation requires pumping data accumulated from all previous years (2003–present), and each year’s depletion tables carry forward historical values from prior years’ calculations. This specification documents the year-chaining architecture: how the pipeline connects year N-1 outputs to year N inputs, ensuring continuity of the cumulative record while allowing recalculation of current and future projections.

Year-chaining is critical for regulatory compliance because the OSE annual report contains cumulative depletion values that must be internally consistent across all reporting years. An error in the chaining logic would corrupt the historical record, requiring reprocessing of all subsequent years.

## 2. Scientific Basis

### 2.1 Superposition Principle and Cumulative Pumping

MODFLOW simulates groundwater flow using the principle of superposition: the aquifer response to pumping at time T equals the sum of responses to all pumping from all prior stress periods. Consequently, the well package (.wel) file must contain pumping records for every month from the model start (2003) through the current processing year. Each new year appends 12 months of pumping data to the cumulative record.

### 2.2 WEL File Accumulation

The MODFLOW well package (.wel) file grows by exactly 324 lines per year:

Lines per year = 12 months x 27 lines/month
               = 12 x (1 header + 26 well entries)
               = 12 x (1 + 13 wells x 2 layers)
               = 324 lines

Each well entry specifies: layer, row, column, pumping rate (ft^3/s), well name, month, year.

### 2.3 Key Equations

Unit conversion (acre-feet to ft^3/s per layer):

rate_ft3_per_s = -(AF / N_layers) x 43,560 / (days_in_month x 86,400)

Where:
  AF           = monthly pumping volume (acre-feet)
  N_layers     = 2 (pumping split equally between MODFLOW layers 1 and 2)
  43,560       = ft^3 per acre-foot
  days_in_month = calendar days (28/29 for Feb, 30/31 for others)
  86,400       = seconds per day
  Negative sign = MODFLOW convention (extraction is negative)

### 2.4 Worked Example

Well B1, January 2024 (31 days, non-leap immaterial for January):

Given:  AF = 16.887963 acre-feet, N_layers = 2, days = 31

Step 1: Volume per layer = 16.887963 / 2 = 8.443982 AF
Step 2: Convert to ft^3  = 8.443982 x 43,560 = 367,819.0 ft^3
Step 3: Seconds in Jan   = 31 x 86,400 = 2,678,400 s
Step 4: Rate             = 367,819.0 / 2,678,400 = 0.13730 ft^3/s
Step 5: MODFLOW sign     = -0.13730 ft^3/s

WEL file entry:
         1        13        11  -0.13730  BUCKMAN 1 JAN 2024
         2        13        11  -0.13730  BUCKMAN 1 JAN 2024

Leap year effect (February 2024, 29 days vs. 28 days):

For the same 10 AF pumped in February:

Leap year (29 days):   rate = -(10/2) x 43560 / (29 x 86400) = -0.08695 ft^3/s
Non-leap (28 days):    rate = -(10/2) x 43560 / (28 x 86400) = -0.09006 ft^3/s

Difference: 3.5% -- significant enough to matter for compliance precision

## 3. Assumptions

| # | Assumption | Justification | Risk if Wrong | Mitigation |
| --- | --- | --- | --- | --- |
| 1 | Years must be processed sequentially (2024, 2025, 2026…) | Each year’s WEL file, Table 1, Table 3, and Table 5 depend on prior year’s output | Missing intermediate year corrupts cumulative record | Pipeline checks for prior year output before proceeding |
| 2 | Pumping is split equally between Layer 1 and Layer 2 | Buckman wells are completed across both model layers; equal split is the conventional assumption for this model | Under/overestimates depletion in individual layers | Conservative for total depletion (sum across layers is correct) |
| 3 | Pumping rate is constant within each monthly stress period | MODFLOW96 uses constant-rate stress periods; sub-monthly variation is smoothed | Seasonal peak effects underestimated within high-demand months | Monthly resolution matches OSE reporting requirements |
| 4 | The 2023 baseline WEL file contains correct pumping for 2003-2023 | File was produced by the prior (pre-automation) workflow and validated independently | All subsequent years would inherit the error | SHA-256 hash verification of baseline files in manifest system |
| 5 | Historical depletion values (pre-processing_year) should be preserved exactly from the year they were calculated | Each year’s MODFLOW run is the authoritative source for that year’s depletions | Re-running with different model inputs could change historical values | Chaining logic locks historical rows from prior year’s output |
| 6 | BASELINE_YEAR = 2024 is the first automated year | 2024 uses original 2023 baseline files; 2025+ uses chained outputs | Incorrect baseline year would use wrong input files | Constant is explicitly defined and documented |

## 4. Data Sources & Provenance

| Source | Description | Access Method | Update Frequency | QC Procedure |
| --- | --- | --- | --- | --- |
| input/modflow/2023/thruCY2165.wel | Baseline WEL file (2003-2023 pumping) | Static file, version-controlled | Never (frozen baseline) | SHA-256 hash verification |
| output/modflow/{year-1}/thruCY2165_{year-1}.wel | Prior year’s cumulative WEL file | Generated by step2 for year-1 | Annual (one per processing year) | Line count validation (must be 324 lines longer than prior) |
| validation/Table_1_data_afy_{year}.xlsx | Table 1 template (baseline year only) | Manual provision for 2024 | One-time | Cross-check with OSE records |
| output/ingested_data/{year-1}/{year-1}_Table_1_updated.xlsx | Table 1 template (subsequent years) | Generated by step1 for year-1 | Annual | Row count = prior year + 1 |
| output/depletion/TABLE_3_*_{year-1}.xlsx | Prior year’s Table 3 for chaining | Generated by step4 for year-1 | Annual | Monotonicity check on cumulative values |
| output/depletion/TABLE_5_*_{year-1}.xlsx | Prior year’s Table 5 for chaining | Generated by step4 for year-1 | Annual | Monotonicity check on cumulative values |

## 5. Key Constants & Conversions

| Constant | Value | Units | Derivation | Source |
| --- | --- | --- | --- | --- |
| BASELINE_YEAR | 2024 | year | First year using automated pipeline with 2023 inputs | Project convention |
| ACRE_FT_TO_FT3 | 43,560 | ft^3/AF | 1 acre = 43,560 ft^2, 1 AF = 1 ft depth | USGS definition |
| SECONDS_PER_DAY | 86,400 | s/day | 24 x 60 x 60 | Physical constant |
| NUM_LAYERS | 2 | dimensionless | Buckman wells completed in both model layers | Model design |
| LINES_PER_MONTH | 27 | lines | 1 header + 13 wells x 2 layers | WEL file format |
| LINES_PER_YEAR | 324 | lines | 12 months x 27 lines | WEL file format |
| WELLS_PER_MONTH | 26 | entries | 13 wells x 2 layers | WEL file format |

### Dimensional Analysis

AF to ft^3/s conversion chain:

acre-feet     1 AF       43,560 ft^3      1 day          1
--------- x -------- x ------------- x ------------ = -------
 month      2 layers      1 AF         86,400 sec      ft^3/s

Cancellation:  [AF] x [ft^3/AF] / [layers] / [s/month]
            =  ft^3 / (layers x s)
            =  ft^3/s per layer

## 6. Domain-Specific Constraints

### 6.1 Chaining Architecture by Pipeline Step

#### Step 1: Table 1 Chaining (Annual Pumping History)

Table 1 is a cumulative record of annual pumping by well (1988–present). Each year adds one row.

| Scenario | Template Source | Behavior |
| --- | --- | --- |
| Baseline year (2024) | validation/Table_1_data_afy_2024.xlsx | Uses hand-prepared historical file |
| Subsequent years (2025+) | output/ingested_data/{year-1}/{year-1}_Table_1_updated.xlsx | Uses prior year’s output as template |
| Neither exists | Error | Pipeline halts with diagnostic message |

Chaining rule: Read all existing rows from template, append new row with current year’s annual totals per well. Statistics rows (Mean, Min, Max, Rank) are recalculated for the full series.

#### Step 2: WEL File Chaining (Cumulative MODFLOW Pumping)

The WEL file is the core chained artifact. It grows by exactly 324 lines each year.

| Scenario | Input WEL Source | Behavior |
| --- | --- | --- |
| Baseline year (2024) | input/modflow/2023/thruCY2165.wel | Original 2003-2023 pumping from pre-automation era |
| Subsequent years (2025+) | output/modflow/{year-1}/thruCY2165_{year-1}.wel | Prior year’s output (contains 2003 through year-1) |

Chaining rule: Parse input WEL file to find target year boundaries (searches for “JAN {year}” and “DEC {year}” patterns), replace the target year section with newly generated entries from current year’s Table 2 pumping data.

Structural invariant: After processing year N, the WEL file contains: - Pre-target lines: all pumping from 2003 through year N-1 (unchanged) - Target year lines: 324 lines of year N pumping (newly generated) - Post-target lines: projected pumping for 2165 (unchanged, from baseline)

#### Step 4: Table 3 Chaining (Stream Depletions)

Table 3 reports cumulative stream depletions for Rio Pojoaque-Nambe and Rio Tesuque. Historical values must be preserved from the year they were computed.

| Scenario | Historical Source | Behavior |
| --- | --- | --- |
| First resolution | output/depletion/TABLE_3_*_{year-1}.xlsx | Prior year’s Table 3 output |
| Fallback | validation/2024/expected_outputs/Table_3_expected.xlsx | Frozen 2024 baseline |
| Neither exists | Error | Pipeline halts with diagnostic message |

Chaining rule: For years before processing_year, values are copied verbatim from the prior year’s Table 3. For years >= processing_year, values are recalculated from the current MODFLOW post-processor output. This ensures that 1988-2023 values remain locked to their respective MODFLOW runs while 2024+ values reflect the latest model.

#### Step 4: Table 5 Chaining (La Cienega Springs)

Table 5 follows the same chaining pattern as Table 3 but for La Cienega Springs cumulative depletions.

| Scenario | Historical Source | Behavior |
| --- | --- | --- |
| First resolution | output/depletion/TABLE_5_*_{year-1}.xlsx | Prior year’s Table 5 output |
| Fallback | LA_CIENEGA_CUMULATIVE dict | Hardcoded historical values (2004-2030) |

Chaining rule: Years before processing_year use historical values from prior year’s output. Years >= processing_year are recomputed from current MODFLOW output using cfs-to-AF conversion.

### 6.2 Directory Structure Evolution

The output directory structure changed between the pre-automation era and the automated pipeline:

| Year Range | Structure | Path Pattern |
| --- | --- | --- |
| 2024 and earlier | Nested | output/modflow/{year}/modflow/CY{year}.* |
| 2025 and later | Flat | output/modflow/{year}/CY{year}.* |

All scripts auto-detect both structures. The flat structure was adopted for simplicity; the nested structure is preserved for backward compatibility with the 2024 baseline.

### 6.3 Leap Year Handling

Leap years affect the AF-to-ft^3/s conversion for February and any annual aggregation.

| Component | Leap Year Effect | Implementation |
| --- | --- | --- |
| WEL file rates | February: 29 vs 28 days in denominator | calendar.isleap(year) in get_days_in_month() |
| Table 3 superposition | cfs-to-AF conversion uses actual calendar days | get_days_in_year(year) returns 12-element list |
| Table 5 cumulative | Same cfs-to-AF conversion | calendar.isleap(year) passed to converter |

### 6.4 Physical Bounds

| Parameter | Min | Max | Units | Basis |
| --- | --- | --- | --- | --- |
| Lines added per year | 324 | 324 | lines | WEL file format (invariant) |
| Wells per stress period | 26 | 26 | entries | 13 wells x 2 layers (invariant) |
| Total annual pumping | 0 | ~5,000 | AF | Engineering limit |
| Individual well rate | 0 | ~2 | ft^3/s | Well capacity |
| Pumping sign convention | -inf | 0 | ft^3/s | MODFLOW convention (negative = extraction) |

### Conservation Laws

- Volumetric conservation: Total pumping in WEL file must equal total pumping in Table 2 (within floating-point tolerance)

- Monotonic accumulation: WEL file line count must increase by exactly 324 per year

- Historical preservation: Pre-processing_year rows in Tables 3 and 5 must be value-identical within floating-point precision to prior year’s output (the pipeline reads Excel values as floats and writes them to new Excel files, so byte-level identity is not guaranteed)

### Workflow Pipeline Limits

- Sequential integrity: No year may be skipped; OSE report must show continuous annual record

- Cumulative consistency: Depletion values in Table 3 must be monotonically non-decreasing (you cannot “un-deplete” a stream)

- Baseline immutability: The 2023 baseline files (input/modflow/2023/) must never be modified after initial validation

## 7. References

### Data Sources

- City of Santa Fe daily pumping records (CSV format, annual delivery)

- OSE historical Buckman wellfield reports (1988–present)

## 8. Cross-References

| Related Spec | Relationship |
| --- | --- |
| DS-01 | Pumping data ingestion (Step 1) that feeds into year chaining |
| DS-02 | MODFLOW model structure that consumes the chained WEL file |
| DS-03 | Stream depletion calculations that use chained Tables 3 and 5 |
| DS-05 | Quality assurance tests that verify chaining integrity |
| DS-06 | Provenance logging that documents chain-of-custody for each year |
| IS-01 | Implementation of step1_ingest_buckman_data.py (Table 1 chaining) |
| IS-02 | Implementation of step2_update_modflow.py (WEL file chaining) |
| IS-04 | Implementation of step4_generate_depletion_tables.py (Tables 3/5 chaining) |

Document Maintenance:

Next Review: When a new baseline year is established or the chaining logic is modified

Change Triggers: Addition of new wells, changes to WEL file format, modification of Table 3/5 chaining fallback sources, changes to BASELINE_YEAR constant

# DS-05: Quality Assurance Framework

## 1. Purpose & Scope

The QA framework ensures the Buckman pipeline produces defensible, scientifically reproducible results through a 7-layer verification hierarchy. Each layer catches progressively subtler errors, including outright pipeline crashes (Layer 0) through physics violations (Layer 1), statistical anomalies (Layer 2), known-good regression (Layer 5), and cryptographic chain-of-custody (Layer 6). The framework is designed around the principle that the test harness is the product: the pipeline produces answers; the harness produces confidence. A total of approximately 241 tests across 7 test files and standalone validation scripts enforce this hierarchy.

## 2. Scientific Basis

### 2.1 Testing Philosophy

Groundwater model outputs cannot be validated in the traditional sense because the true state of the aquifer is never fully known. Instead, the framework applies multiple independent lines of evidence:

- Internal consistency – Do outputs obey conservation of mass and known physical constraints?

- Historical plausibility – Are outputs within the envelope of prior observations?

- Reproducibility – Can the pipeline reproduce known-good results from frozen inputs?

- Provenance – Can every output be traced to specific inputs, code versions, and test results?

This multi-layered approach mirrors USGS guidance on model validation (Anderson & Woessner, 2015) and reflects legal standards for admissibility of scientific evidence (Daubert v. Merrell Dow Pharmaceuticals, 1993).

### 2.2 The 7-Layer Verification Hierarchy

| Layer | Name | What It Catches | Hard-fail? |
| --- | --- | --- | --- |
| 0 | Smoke Tests | Import errors, missing files, NaN/null outputs, pipeline crashes | Yes |
| 0.5 | Edge Cases | Boundary conditions, zero pumping wells, leap years, single-day months | Yes |
| 1 | Conservation | Mass balance violations, unit conversion errors, dropped records | Yes |
| 2 | Temporal Consistency | Year-over-year anomalies, stress period misalignment, chaining errors | Flag |
| 3 | Cross-Comparison | Systematic errors invisible to internal checks | Skipped (no external data source) |
| 4 | Perturbation Testing | Fragility – disproportionate output changes from small input changes | Planned (not yet implemented) |
| 5 | Regression (2024 Baseline) | Pipeline drift – code changes that subtly alter results | Flag |
| 6 | Provenance | Missing audit trail, hash mismatches, unreproducible runs | Flag |

### 2.3 Hard-Fail vs. Soft-Flag Policy

The framework distinguishes between conditions that invalidate outputs and conditions that require expert judgment:

| Category | Trigger | Exit Code | Response |
| --- | --- | --- | --- |
| Hard fail | Physics violation (NaN, negative depletion, non-conservation) | 1 | Pipeline halts. Outputs must not be used. |
| Soft flag | Statistical outlier (>2 sigma from historical mean) | 2 | Pipeline continues. Analyst review required before submission. |
| Ballpark critical | Gross physics violation (non-monotonic cumulative depletion) | 3 | Immediate halt. Fundamental model error suspected. |
| Pass | All checks within bounds | 0 | Outputs cleared for use. |
| Skip | No validation baseline for this year | 4 | Ballpark check not applicable (normal for new years). |

Regulatory rationale: Layers 0–1 are hard failures because they detect physically impossible outputs. Layers 2–6 produce flags because they detect conditions that may be unusual but valid. An analyst must review each flag, document their judgment (e.g., “real operational change” vs. “data quality issue”), and sign off with initials and date.

### 2.4 Tolerance Model

The regression testing framework (Layer 5) uses a hybrid tolerance model that accommodates both small and large values:

PASS if |actual - expected| <= absolute_tolerance
  OR if |actual - expected| / |expected| <= relative_tolerance

Where both criteria are checked; passing EITHER one is sufficient.

Rationale: Pure absolute tolerance fails for large values (0.01 AF tolerance is meaningless for an 800 AF depletion). Pure relative tolerance fails for near-zero values (0.1% of 0.01 AF is 0.00001 AF, which is below floating-point noise). The hybrid approach handles both regimes.

Per-table tolerance definitions (from validation/2024/tolerances.yaml):

| Table | Description | Abs. Tolerance | Rel. Tolerance | Precision Basis |
| --- | --- | --- | --- | --- |
| Table 1 | Annual pumping by well (AF/yr) | 0.01 AF | 0.001 (0.1%) | 2 decimal places in report |
| Table 2 | Monthly pumping by well (AF/mo) | 0.01 AF | 0.001 (0.1%) | 2 decimal places in report |
| Table 3 | Stream depletions (cumulative AF) | 0.001 AF | 0.001 (0.1%) | 3 decimal places in report |
| Table 4 | Rio Grande above/below Otowi (AF) | 0.1 AF | 0.01 (1%) | Larger tolerance; may need adjustment |
| Table 5 | La Cienega cumulative (AF) | 0.01 AF | 0.005 (0.5%) | 2 decimal places; only through 2024 |

Global policies (from tolerances.yaml): - nan_handling: "fail" – Any NaN value is an immediate hard failure - empty_cell_handling: "match" – Empty cells must match exactly between actual and expected

### 2.5 Worked Example: Tolerance Check

Table 3, Rio Pojoaque Total Impact, 2024:

Expected value:  60.797 AF
Actual value:    60.798 AF
Difference:      0.001 AF

Table 3 tolerances:
  Absolute: 0.001 AF
  Relative: 0.001 (0.1%)

Check 1 (absolute): |60.798 - 60.797| = 0.001 <= 0.001  --> PASS
Check 2 (relative): 0.001 / 60.797 = 0.0000164 <= 0.001 --> PASS

Result: PASS (satisfies both criteria)

Edge case: near-zero value (Well B5, annual pumping = 0.00 AF):

Expected: 0.00 AF
Actual:   0.001 AF
Difference: 0.001 AF

Table 1 tolerances:
  Absolute: 0.01 AF
  Relative: 0.001 (0.1%)

Check 1 (absolute): 0.001 <= 0.01            --> PASS
Check 2 (relative): 0.001 / 0.00 = undefined --> N/A (division by zero)

Result: PASS (absolute criterion sufficient)

## 3. Assumptions

| # | Assumption | Justification | Risk if Wrong | Mitigation |
| --- | --- | --- | --- | --- |
| 1 | 2024 validation data is the authoritative ground truth | Independently produced by prior (non-automated) workflow and verified by domain expert | Regression tests pass despite pipeline errors | Cross-check against prior-year PDF reports (2022, 2023) |
| 2 | Historical bounds (2022-2024) are representative of future operations | 3-year window captures recent operational patterns | Bounds too tight for changing conditions | Soft flags (not hard fails) for statistical outliers; bounds updated annually |
| 3 | Cumulative depletions are physically monotonic | Groundwater depletion effects propagate outward and do not reverse under continued pumping | Flags valid aquifer recovery as an error | Monotonic tolerance of 0.01 AF allows for floating-point noise |
| 4 | MODFLOW mass-balance discrepancy < 0.1% indicates valid simulation | Standard USGS practice for MODFLOW model acceptance | Convergence errors masked by compensating budget terms | Layer 1 tests parse individual budget components, not just totals |
| 5 | NaN/Inf values are always errors, never valid data | Physical quantities are always finite and defined | None identified | Hard fail on any NaN/Inf in output tables |
| 6 | Layer 3 (cross-comparison) is not applicable to this pipeline | Rejected during P6 development: no independent external data source available | Systematic errors undetected by internal checks | Layers 1, 2, and 5 provide overlapping coverage |
| 7 | The hybrid tolerance model is appropriate for all tables | Both absolute and relative error matter depending on value magnitude | Tolerance too loose for some cells, too tight for others | Per-table tolerance tuning with domain expert review |

## 4. Data Sources & Provenance

| Source | Description | Access Method | Update Frequency | QC Procedure |
| --- | --- | --- | --- | --- |
| validation/historical/bounds.yaml | Physical bounds from 2022-2024 data (2 sigma/3 sigma thresholds) | YAML file, version-controlled | When new baseline years are added | Manual extraction from historical tables, documented in extraction_log.md |
| validation/2024/tolerances.yaml | Tolerance definitions for regression testing (hybrid model) | YAML file, version-controlled | When tolerance policy changes | Reviewed by domain expert |
| validation/2024/expected_outputs/ | Frozen 2024 expected output tables (Tables 1-5) | Excel files, version-controlled | Never (frozen baseline) | SHA-256 hash verification before each regression run |
| validation/2024/inputs/ | Frozen 2024 input files (pumping CSV, MODFLOW templates) | CSV/Excel files, version-controlled | Never (frozen baseline) | SHA-256 hash verification |
| output/test_results/*.json | JSON test reports from pytest-json-report | Generated by run_all_tests.py | Each pipeline run | Automated parsing of pass/fail counts |
| archive/ralph/ | Historical Ralph test automation logs and state snapshots (v0-v6) | Archived from ralph/ | Never (historical) | Preserved for audit trail; active ralph logs remain in ralph/ |
| output/manifests/buckman_manifest_{year}.json | Provenance manifest (Layer 6) | Generated by pipeline_manifest.py | Each pipeline run | SHA-256 hashes of all inputs and outputs |

## 5. Key Constants & Conversions

| Constant | Value | Units | Derivation | Source |
| --- | --- | --- | --- | --- |
| soft_flag_sigma | 2.0 | sigma | Flag if > 2 standard deviations from historical mean | Statistical convention (95.4% coverage) |
| hard_fail_sigma | 3.0 | sigma | Fail if > 3 standard deviations from historical mean | Statistical convention (99.7% coverage) |
| monotonic_tolerance_af | 0.01 | AF | Floating-point noise floor for cumulative values | Empirical (observed rounding in MODFLOW output) |
| min_pumping_af | 0.0 | AF | Cannot extract negative water | Physics |
| max_pumping_af | 5,000 | AF | Engineering limit (all 13 wells at full capacity) | Operational knowledge |
| modflow_budget_tolerance | 0.1 | % | Maximum volumetric budget discrepancy | USGS MODFLOW convention |

### Ballpark Threshold Derivation

The ballpark check uses two tiers derived from the 2022-2024 historical total annual pumping:

Historical total annual pumping:
  2022: 975.47 AF
  2023: 866.48 AF
  2024: 1,372.90 AF
  Mean: 1,071.62 AF
  Std:  266.95 AF
  Max:  1,372.90 AF

Soft flag threshold: 2x historical max = 2,745.80 AF
Hard fail threshold: 3x historical max = 4,118.70 AF
Hard minimum:        0.0 AF (physics floor)

### Dimensional Analysis

All tolerance comparisons operate on values in acre-feet. No unit conversion is required within the tolerance check itself because all output tables report in AF. The tolerance values themselves have units of AF (absolute) or dimensionless ratios (relative).

## 6. Domain-Specific Constraints

### 6.1 Layer 0: Smoke Tests

Purpose: Verify the pipeline does not crash and produces structurally valid output.

What it catches: Missing files, NaN/null values, wrong column counts, empty tables, pipeline crashes.

Tests include:

All 5 output Excel files created and non-empty

No NaN, Inf, or null values in any output cell

Expected columns/rows present in each table

MODFLOW96 listing file contains “NORMAL TERMINATION”

Post-processor output file exists and has expected structure

Policy: Hard fail (exit code 1). If smoke tests fail, no subsequent layers run.

### 6.2 Layer 0.5: Edge Cases

Purpose: Verify boundary conditions and extreme inputs are handled correctly.

What it catches: Zero-pumping wells, first/last month of year, leap year edge cases, wells at grid boundaries.

Tests include:

Well B5 (historically zero pumping) handled correctly

February 29 in leap years produces correct conversion

All 13 wells present in output regardless of pumping volume

Single-day months and 31-day months handled identically

Policy: Hard fail (exit code 1). Edge case failures indicate logic errors.

### 6.3 Layer 1: Conservation and Mass-Balance Checks

Purpose: Verify that water is neither created nor destroyed at pipeline hand-off points.

What it catches: Physics violations such as sign errors, dropped records, unit conversion mistakes, stress period misalignment.

Tests include:

Volumetric budget closure: MODFLOW listing file percent discrepancy < 0.1%

Pumping-in = pumping-used: Sum of WEL file pumping matches Table 2 totals within tolerance

Depletion <= pumping: For every stream reach, cumulative depletion does not exceed cumulative pumping

Table sum integrity: Row/column sums in report tables match expected totals

Prerequisites: Requires MODFLOW outputs to exist (full pipeline must have been run).

Policy: Hard fail (exit code 1). Conservation violations indicate fundamental errors.

### 6.4 Layer 2: Temporal Consistency

Purpose: Flag values that are physically possible but historically implausible.

What it catches: Stress period misalignment (months shifted by one), data entry errors, unusual operational patterns.

Tests include:

Year-over-year rate of change: Flag if annual pumping changes by more than threshold relative to prior year

Seasonal pattern validation: Compare normalized monthly profile against historical mean; flag if Pearson correlation < 0.85

Multi-year trend continuity: Current year within 95% prediction interval of historical linear regression

Monthly Profile Validation:

The historical mean monthly pumping profile (normalized fraction of annual total, from bounds.yaml):

| Month | Mean Fraction | Physical Interpretation |
| --- | --- | --- |
| JAN | 0.005 (0.5%) | Winter minimum – low municipal demand |
| FEB | 0.053 (5.3%) | Late winter |
| MAR | 0.061 (6.1%) | Early spring |
| APR | 0.041 (4.1%) | Spring |
| MAY | 0.120 (12.0%) | Summer ramp-up |
| JUN | 0.162 (16.2%) | Summer peak begins |
| JUL | 0.178 (17.8%) | Summer peak |
| AUG | 0.191 (19.1%) | Annual maximum – peak irrigation + municipal |
| SEP | 0.079 (7.9%) | Fall decline |
| OCT | 0.059 (5.9%) | Autumn |
| NOV | 0.007 (0.7%) | Winter minimum |
| DEC | 0.044 (4.4%) | Winter |

A significantly different seasonal pattern (low Pearson r against this profile) likely indicates stress period misalignment – the most insidious bug where values look reasonable individually but months are shifted.

Policy: Soft flag (exit code 2). Outputs may still be valid but require human review before submission.

### 6.5 Layer 3: Cross-Comparison

Status: Not included during P6 development.

Rationale: No independent external data source for depletion is available for cross-comparison.

### 6.6 Layer 4: Perturbation Testing

Status: Planned but not yet implemented.

Design: Create controlled perturbations of current year’s pumping data:

Add +/-5% uniform noise to all pumping values

Shift one well’s data by one stress period (time offset)

Zero out one well entirely

Double one well’s pumping for one month

Assert that output changes are proportional and physically reasonable. If zeroing out one minor well causes 40% change in depletion at a distant reach, something is wrong.

### 6.7 Layer 5: 2024 Regression Harness

Purpose: Verify the pipeline reproduces known-good results from frozen inputs. This is the single highest-value layer.

What it catches: Workflow “pipeline drift” are code changes that subtly alter results.

Architecture:

- Verify SHA-256 hashes of all 2024 input files match stored values (via validation/historical/hashes.json)

- Run full pipeline on frozen 2024 inputs

- Compare every cell in all 5 output tables against expected values using the hybrid tolerance model 4. Report pass/fail with cell-level detail on any failures

Policy: The regression must pass before any code change is accepted. If the pipeline cannot reproduce 2024, it cannot be trusted for any year.

Extension plan: When independent validation for 2025 or later years becomes available, add them to the regression suite. The harness grows harder to pass and more trustworthy over time.

Integration note: Layer 5 is NOT currently integrated into the run_all_tests.py master test runner. It is run separately via validation/2024/run_regression_2024.py. The master runner orchestrates Layers 0, 0.5, 1, 2, and 6 only.

### 6.8 Layer 6: Provenance Logging

Purpose: Produce an audit trail that can withstand legal challenge.

What it produces (via src/pipeline_manifest.py):

Input manifest: File names, SHA-256 hashes, row counts, date ranges for every input file

Pipeline manifest: Git commit hash, git working tree status (clean/dirty), Python version, script modification dates, MODFLOW/FORTRAN executable metadata

Test results manifest: Pass/fail per layer with specific values tested and thresholds applied

Flag register: Analyst disposition workflow, where  each flag has fields for disposition text, analyst initials, and review date

Run metadata: Machine name, OS, timestamps, total runtime

Manifest schema version: 1.0

Policy: Generated on every pipeline run. Accompanies the 5 report tables in the project file. The manifest transforms “I checked the numbers” into “N automated tests passed, M flags were reviewed and dispositioned, the pipeline reproduced the independently validated 2024 dataset within tolerance, and here is the hash of every input file.”

### 6.9 Physical Bounds from Historical Data (2022-2024)

Extracted from validation/historical/bounds.yaml.

#### Table 1: Annual Pumping by Well

| Well | Min (AF) | Max (AF) | Mean (AF) | Notes |
| --- | --- | --- | --- | --- |
| B1 | 224.06 | 601.27 | 427.82 | Primary production well |
| B2 | 0.01 | 0.01 | 0.01 | Essentially inactive |
| B3/3A | 0.40 | 56.23 | 18.95 | Variable usage |
| B4 | 38.18 | 63.35 | 49.62 | Moderate, steady |
| B5 | 0.00 | 0.00 | 0.00 | Inactive (zero pumping) |
| B6 | 7.92 | 152.22 | 56.02 | Highly variable |
| B7 | 0.31 | 1.00 | 0.54 | Near-zero |
| B8 | 111.46 | 196.50 | 139.81 | Major production |
| B9 | 0.52 | 11.81 | 4.29 | Minor, variable |
| B10 | 45.17 | 151.41 | 80.66 | Significant |
| B11 | 13.62 | 89.00 | 38.75 | Variable |
| B12 | 0.18 | 9.80 | 3.39 | Minor |
| B13 | 127.85 | 212.02 | 152.57 | Major production |
| Total | 866.48 | 1,372.90 | 1,071.62 | Std: 266.95 AF |

Threshold policy for total annual pumping: - Historical range: 866–1,373 AF - Soft flag: > 2x historical max (2,746 AF) - Hard fail: > 3x historical max (4,119 AF) or < 0 AF

#### Table 3: Cumulative Stream Depletions

| Stream | Min (AF) | Max (AF) | Mean (AF) | Std (AF) | Monotonic? |
| --- | --- | --- | --- | --- | --- |
| Rio Pojoaque-Nambe | 59.844 | 60.797 | 60.321 | 0.477 | Yes (required) |
| Rio Tesuque | 33.490 | 33.583 | 33.521 | 0.053 | Yes (required) |

#### Table 4: Rio Grande Depletions

| Location | Min (AF) | Max (AF) | Mean (AF) | Std (AF) |
| --- | --- | --- | --- | --- |
| Above Otowi (annual) | 101.21 | 101.43 | 101.29 | 0.12 |
| Below Otowi (annual) | 783.23 | 842.94 | 809.50 | 30.67 |

#### Table 5: La Cienega Springs

| Parameter | Min (AF) | Max (AF) | Mean (AF) | Std (AF) | Monotonic? |
| --- | --- | --- | --- | --- | --- |
| Cumulative depletion | 3.37 | 3.74 | 3.55 | 0.19 | Yes (required) |

#### Derived Ratios

| Ratio | Min | Max | Mean | Interpretation |
| --- | --- | --- | --- | --- |
| Depletion/Pumping | 0.069 | 0.108 | 0.091 | Varies inversely with pumping intensity |

### 6.10 Test Runner Architecture

The master test runner (run_all_tests.py, 894 lines) orchestrates all layers with subprocess isolation and structured JSON reporting:

run_all_tests.py --year YYYY [--skip-ballpark] [--verbose] [--dry-run]
  |
  +-- [1/6] Ballpark check (validation/ballpark_check.py)
  |     Exit 0 --> pass
  |     Exit 2 --> soft flags, continue
  |     Exit 3 --> STOP (physics violation)
  |     Exit 4 --> skip (no baseline for this year)
  |
  +-- [2/6] Layer 0: Smoke tests (pytest -m layer0, timeout 300s)
  +-- [3/6] Layer 0.5: Edge cases (pytest -m edge_cases, timeout 120s)
  +-- [4/6] Layer 1: Conservation (pytest -m conservation, timeout 120s)
  |     Prerequisite check: MODFLOW outputs must exist
  |
  +-- [5/6] Layer 2: Temporal consistency (timeout 60s)
  |     Produces flags (not hard fails)
  |
  +-- [5.5/6] Layer 3: Cross-comparison (omitted from orchestration sequence; not explicitly skipped at runtime)
  |
  +-- [6/6] Layer 6: Provenance manifest (src/pipeline_manifest.py)
  |
  +-- Summary + workflow log generation (src/workflow_logger.py)

Exit code semantics (master runner run_all_tests.py):

Exit 0: All hard-stop tests passed (flags may exist requiring review)

Exit 1: Hard-stop test failure or script error

Exit 3: Ballpark check critical physics violation

Note: Exit code 2 (soft flag) is used internally by the ballpark_check.py subprocess, not by the master runner itself. When ballpark_check.py returns exit code 2, the master runner records the soft flags but continues execution and returns exit code 0 if no hard failures occurred. The master runner returns only 0, 1, or 3.

Subprocess timeouts:

Ballpark check: 30 seconds

Layer 0 (smoke): 300 seconds (5 minutes)

Layer 0.5 (edge): 120 seconds

Layer 1 (conservation): 120 seconds

Layer 2 (temporal): 60 seconds

Layer 6 (manifest): 60 seconds

JSON reporting: Each pytest layer writes structured results to output/test_results/{layer_name}.json via pytest-json-report, enabling programmatic parsing of pass/fail counts for the manifest and workflow log.

## 7. References

### Data Sources

- validation/historical/bounds.yaml – Physical bounds derived from 2022-2024 data

- validation/2024/tolerances.yaml – Regression tolerance definitions

- validation/historical/extraction_log.md – Documentation of bounds extraction process

## 8. Cross-References

| Related Spec | Relationship |
| --- | --- |
| DS-01 | Well production data – Layer 0 verifies ingestion outputs are valid; Layer 1 checks pumping sums |
| DS-02 | MODFLOW model – Layer 1 checks mass balance from MODFLOW listing file |
| DS-03 | Stream depletions – Layer 5 regression verifies depletion table values against 2024 baseline |
| DS-04 | Year chaining – Layer 2 temporal checks verify chaining consistency across years |
| DS-06 | Regulatory compliance – Layer 6 provenance logging produces audit artifacts required for OSE submission |
| IS-10 | Test implementation – pytest test files implementing Layers 0, 0.5, 1 |
| IS-11 | Validation framework – ballpark_check.py, temporal_consistency.py, bounds.yaml, tolerances.yaml |
| IS-12 | Provenance implementation – pipeline_manifest.py, workflow_logger.py |

Document Maintenance:

Next Review: When historical bounds are updated with new baseline years, or when new test layers are implemented (Layer 4 perturbation)

Change Triggers: Addition of new validation years to regression suite, changes to tolerance definitions, implementation of Layer 3 or Layer 4, modification of exit code semantics, changes to the hybrid tolerance model

# DS-06: Compliance Context

## 1. Purpose & Scope

This specification documents the compliance framework within which the Buckman Wellfield depletion pipeline operates. This workflow pipeline produces deliverables for the New Mexico Office of the State Engineer (OSE). The Buckman Wellfield serves the City of Santa Fe’s municipal water supply. Pumping from the wellfield depletes surface water flows in the Rio Grande system. The pipeline quantifies these depletions annually and produces 5 Excel tables that are copy-pasted into the official OSE report.

## 2. Scientific Basis

### 2.1 The Rio Grande Compact (1938)

The Rio Grande Compact allocates water from the Rio Grande among Colorado, New Mexico, and Texas. Key provisions affecting this workflow pipeline:

- Otowi Gage (USGS 08313000) is the accounting point for New Mexico’s deliveries to Texas

- Water depletions that occur below Otowi are chargeable against New Mexico’s delivery obligation

- Water depletions that occur above Otowi do not directly affect Compact accounting (but are still reported)

The Buckman Wellfield lies below Otowi Bridge. Groundwater pumping at Buckman creates stream depletions in:

The Rio Grande itself (both above and below Otowi, due to aquifer geometry)

Tributary streams (Rio Pojoaque-Nambe, Rio Tesuque)

Springs (La Cienega Springs)

These depletions must be quantified and reported annually.

### 2.2 Depletion Accounting Framework

The total depletion from Buckman pumping is partitioned into components reported in separate tables:

Total Depletion = Table 3 (Tributaries) + Table 4 (Rio Grande) + Table 5 (Springs)

Where:
  Table 3: Rio Pojoaque-Nambe + Rio Tesuque (cumulative AF)
  Table 4: Above Otowi + Below Otowi (annual AF, monthly detail)
  Table 5: La Cienega Springs (cumulative AF)

Compact-chargeable depletions = Table 4 (Below Otowi) + portions of Tables 3 and 5

### 2.3 Superposition + Analytical Residual Framework

Stream depletions are calculated using a two-component approach:

Component 1: MODFLOW Superposition (1988–present)

The MODFLOW96 model simulates aquifer response to pumping from 1988 through the current year using the principle of superposition. The model calculates how much stream flow is reduced by each year’s pumping.

Component 2: Analytical Residuals (pre-1988)

Buckman wellfield pumping began in 1972. The MODFLOW superposition model only covers 1988–present. The effects of 1972–1987 pumping continue to propagate through the aquifer as “residual” depletions. These are calculated using Core (2003) analytical projections extending Spinks (1988) through approximately 2050.

For Table 3, each stream’s total impact is:

Total Impact = Residual (Core 2003) + Superposition (MODFLOW)

Note: Pojoaque residual exhausted in 2015 (projected to zero by Core 2003)
      Tesuque residual continues through ~2050

### 2.4 Worked Example: Depletion Impact

2024 Buckman Wellfield Depletion Impacts:

Total pumping:              1,372.90 AF
Below Otowi depletion:        842.94 AF (Table 4)
Above Otowi depletion:        101.43 AF (Table 4)
Pojoaque depletion:            60.80 AF (Table 3, cumulative)
Tesuque depletion:             33.58 AF (Table 3, cumulative)
La Cienega cumulative:          3.74 AF (Table 5, cumulative since 2004)

Compact-chargeable (Table 4 Below):  842.94 AF
As fraction of NM allocation:        842.94 / 405,000 = 0.21%

Physical interpretation:
  Pumping 1 AF results in approximately 0.61 AF depletion
  the Rio Grande below Otowi (842.94 / 1,372.90 = 0.614).
  The remainder affects tributaries, above-Otowi reaches, and springs.

## 3. Assumptions

| # | Assumption | Justification | Risk if Wrong | Mitigation |
| --- | --- | --- | --- | --- |
| 1 | OSE report tables are copy-pasted directly from pipeline output | Current workflow; tables must match exact format | Format changes require pipeline update | Template-based Excel generation with validation against prior years |
| 2 | MODFLOW superposition is valid for Buckman aquifer system | Tested against 2003 aquifer test data; linear response confirmed | Non-linear effects underestimated in high-pumping years | Depletion/pumping ratio monitored in bounds.yaml (0.069–0.108) |
| 3 | Core (2003) analytical residuals remain valid | Projection based on 1988 aquifer properties; no new calibration data | Residuals over/underestimated | Pojoaque residual already exhausted (2015); Tesuque residual is small and declining |
| 4 | Workflow pipeline outputs must be scientifically correct | Potential for litigation | Inadequate documentation undermines credibility | Provenance logging (Layer 6) documents every input, code version, and test result |
| 5 | Precision requirements match OSE reporting conventions | Tables 1-2: 2 decimal places; Table 3: 3 decimal places; Tables 4-5: 2 decimal places | Rounding differences vs. OSE expectations | Tolerances defined per table in tolerances.yaml |
| 6 | Annual reporting cycle: one report per calendar year | OSE requires annual depletion report for compact accounting | Multi-year processing gap requires sequential catch-up | Year chaining (DS-04) supports sequential processing of any year |

## 4. Data Sources & Provenance

| Source | Description | Access Method | Update Frequency | QC Procedure |
| --- | --- | --- | --- | --- |
| City of Santa Fe pumping records | Daily MGD by well (CSV) | Annual delivery from City | Annual | Three-tier validation (noise, info, error thresholds) |
| MODFLOW96 executable | USGS MODFLOW-96 groundwater flow model | Static binary (1996 release) | Never | SHA-256 hash verification |
| sfmodflx_2245.exe | Fortran stream depletion post-processor | Static binary (custom for Buckman) | Never | SHA-256 hash verification |
| Core (2003) analytical residuals | Pre-1988 pumping depletion projections | Hardcoded in stream_depletions.py | Never (fixed projection) | Values from published report |
| Prior-year OSE reports | Historical pumping and depletion tables | PDF files in docs/reports_prior_year/ | Annual archive | Manual cross-check during Table 1 chaining |

## 5. Key Constants & Conversions

| Constant | Value | Units | Derivation | Source |
| --- | --- | --- | --- | --- |
| NM Compact allocation | ~405,000 | AF/year | Approximate (varies with upstream supply) | Rio Grande Compact (1938) |
| 1 MG to AF | 3.06889 | AF/MG | 1,000,000 gal / 325,851 gal/AF | USGS definition |
| 1 AF to ft^3 | 43,560 | ft^3/AF | 1 acre x 1 ft | USGS definition |
| cfs to AF/day | 1.9835 | AF/day/cfs | 86,400 s/day / 43,560 ft^3/AF | Unit conversion |

### Reporting Precision Requirements

| Table | Content | Precision | Physical Basis |
| --- | --- | --- | --- |
| Table 1 | Annual pumping by well | 2 decimal places (AF) | Well meter accuracy (~1%) |
| Table 2 | Monthly pumping by well | 2 decimal places (AF) | Aggregated from daily readings |
| Table 3 | Tributary depletions | 3 decimal places (AF) | MODFLOW precision at stream cells |
| Table 4 | Rio Grande depletions | 2 decimal places (AF) | Aggregated from monthly values |
| Table 5 | La Cienega cumulative | 2 decimal places (AF) | Small values require resolution |

## 6. Domain-Specific Constraints

### 6.1 The Five Report Tables

Each table serves a specific regulatory purpose. Together they document the complete impact of Buckman pumping on the Rio Grande system.

Table 1: Historical Annual Pumping by Well (AF/year)

Purpose: Shows whether total extraction is increasing, decreasing, or stable over the 1988–present record. Allows identification of changes in pumping patterns.

Structure: Rows = years (1988–present), Columns = wells (B1 through B13) + Total + Rank.

Reporting significance: Establishes the factual basis for how much water was extracted. Pumping data comes from City of Santa Fe production records.

Table 2: Monthly Pumping Detail for Current Year (AF/month)

Purpose: Seasonal pattern documentation. Shows how pumping varies through the year, reflecting municipal demand cycles (low winter, high summer).

Structure: Rows = wells (B1 through B13) + Total, Columns = months (JAN–DEC) + Annual Total.

Reporting significance: Monthly resolution is required because MODFLOW model stress periods are monthly. This table provides the pumping rates used in the model.

Table 3: Tributary Stream Depletions (Cumulative AF)

Purpose: Quantifies cumulative impact on Rio Pojoaque-Nambe and Rio Tesuque from Buckman pumping since 1988.

Structure: Rows = years (1988–present), Columns = Residual Impact, Superposition Impact, Total Impact for each stream.

Reporting significance: Tributary depletions reduce flow available to downstream users and may affect junior water rights. Values are cumulative and generally increasing. For example, the Residual Impact column for Pojoaque decreases from 40.432 AF in 1988 to 0 AF after 2015 as the Core (2003) analytical projection exhausts.

Table 4: Rio Grande Above/Below Otowi (AF)

Purpose: Compact accounting. Separates depletions above and below the Otowi Gage for Rio Grande Compact bookkeeping.

Structure: Cell-level monthly detail + summary rows for Above Otowi and Below Otowi. Below Otowi total is the Compact-chargeable depletion.

Reporting significance: This is the most Compact-critical table. Below Otowi depletions are debited against New Mexico’s delivery obligation to Texas.

Table 5: La Cienega Springs Cumulative Depletions (AF)

Purpose: Track impact on La Cienega Springs, located approximately 15 km south of the wellfield.

Structure: Rows = years (2004–2030), Column = cumulative depletion (AF).

Reporting significance: Spring impacts were first calculated in 2005 (Barroll & Keyes). Values are small (3–4 AF cumulative through 2024) but monitored because springs have ecological and cultural significance.

### 6.2 Provenance Chain-of-Custody

The workflow pipeline produces a provenance manifest (JSON) that documents the complete chain of custody from raw input to report table. This is designed for auditability in regulatory and legal settings.

Input Manifest

For every input file consumed by the pipeline:

File name and full path

SHA-256 cryptographic hash (proves file has not been altered)

File size in bytes

Row count (for CSV/Excel files)

Date range of data (for pumping records)

Source system identification

Purpose: Proves which specific data went into the calculation based upon the hash.

Pipeline Manifest

For the software that produced the results:

Git commit hash (exact code version)

Git working tree status (clean vs. dirty – uncommitted changes flagged)

Modification dates of all pipeline scripts

MODFLOW96 and sfmodflx_2245 executable modification dates and sizes

Python version

Purpose: Proves which specific code version produced the results. Enables exact reproduction of any past calculation.

Test Results Manifest

For every automated test that was run:

Layer name and test name

Pass/fail/skip status

Specific value tested and threshold applied

Timestamp of test execution

Purpose: Documents that quality assurance was performed and what specifically was verified.

Flag Register

For any test that raised a soft flag (not a hard fail):

Test name and flagged value

Threshold that was exceeded

Disposition field: Space for analyst to record: “Reviewed, determined to be [real operational change / data quality issue / within acceptable bounds]. Analyst initials, date.”

Purpose: Creates a documented human-in-the-loop for statistical outliers. Transforms “the analyst reviewed the numbers” into “Flag X was reviewed by [initials] on [date] and determined to be [reason].”

### 6.3 Workflow Log (Audit Trail)

The pipeline generates a comprehensive workflow log in both Markdown and DOCX formats. DOCX generation requires pandoc to be installed (pypi.org, 2026); if pandoc is unavailable, the workflow logger degrades gracefully to Markdown-only output. The log is structured as a formal audit document with the following sections:

| Section | Content | Regulatory Purpose |
| --- | --- | --- |
| 1. Metadata | Timestamp, operator, machine, git commit, branch | Establishes when, where, and by whom the calculation was performed |
| 2. Executive Summary | Total pumping, key depletions, verification status | One-page summary for management review |
| 3. Input Inventory | File names, sizes, SHA-256 hashes, roles | Proves which data was consumed |
| 4. Step-by-Step Execution | Each pipeline step with inputs/outputs | Documents the calculation sequence |
| 5. Output Inventory | Generated files with hashes | Proves what was produced |
| 6. Verification Summary | Test results by layer (pass/fail/skip counts) | Documents QA was performed |
| 7. Physical Interpretation | Pumping analysis, Compact implications, tributary/spring impacts | Provides context for non-technical reviewers |
| 8. Assumptions & Limitations | Model assumptions, data limitations, known caveats | Required disclosure for scientific reporting |
| 9. Approval Block | Prepared by, Reviewed by, Approved by (with signature lines) | Formal sign-off for regulatory submission |

Physical Interpretation Templates

The workflow log includes pre-formatted interpretation sections that contextualize the numbers:

Pumping Analysis: Compares current year pumping to 5-year historical average, characterizes as above/below/normal, and notes active well distribution.

Rio Grande Compact Implications: States the below-Otowi depletion in AF, notes it is chargeable against NM allocation, and computes the fraction of typical annual allocation consumed.

Tributary Impacts: Reports Pojoaque and Tesuque depletions with residual/superposition breakdown, and notes the status of Core (2003) residual projections (Pojoaque exhausted 2015, Tesuque continues).

La Cienega Springs: Reports cumulative and annual increment, and provides context on the springs’ distance from the wellfield and the history of the GHB representation.

### 6.4 Analyst Disposition Workflow

When the test framework raises soft flags (exit code 2), the analyst must review each flag before outputs can be submitted. The disposition workflow is:

1. Test framework raises flag --> recorded in flag register
2. Analyst reviews the flagged value and its context
3. Analyst records disposition:
   - "Reviewed. [Value] is within expected range for [reason]."
   - OR "Reviewed. [Value] reflects [operational change]. No error."
   - OR "Reviewed. Identified [data quality issue]. Corrective action: [action]."
4. Analyst signs with initials and date
5. Flag register is included in provenance manifest

This workflow ensures that unusual values receive explicit human judgment before entering the regulatory record. It also creates a defensible audit trail: if challenged on an outlier value, the response is “This value was flagged by automated testing, reviewed by [analyst] on [date], and determined to be [reason].”

### 6.5 Reporting Limits

| Constraint | Source | Enforcement |
| --- | --- | --- |
| Annual reporting deadline | OSE administrative requirement | Pipeline designed for single-command execution per year |
| Table format must match OSE template | OSE report format | Excel generation uses openpyxl (Gazoni, Eric and Clark, Charlie, 2026) with format-matching styles |
| Below-Otowi depletions charged to NM | Rio Grande Compact Art. IV | Table 4 explicitly separates Above/Below Otowi |
| Cumulative total depletions generally increasing | Physics (net depletion grows under continued pumping, though individual residual components can decline) | Flagged in ballpark_check.py; strict monotonicity not guaranteed for all columns |
| All pumping must be non-negative | Physics (cannot inject water through production wells) | Hard fail for negative values |
| Historical values preserved across years | Regulatory consistency | Year chaining locks historical rows (DS-04) |

## 7. References

### Data Sources

- City of Santa Fe Buckman Wellfield production records (annual CSV delivery)

## 8. Cross-References

| Related Spec | Relationship |
| --- | --- |
| DS-01 | Pumping data ingestion – produces Tables 1 and 2 for regulatory submission |
| DS-02 | MODFLOW model – generates the depletion calculations underlying Tables 3-5 |
| DS-03 | Stream depletion calculations – scientific basis for Table 3-5 values |
| DS-04 | Year chaining – ensures cumulative consistency across annual reports |
| DS-05 | Quality assurance – verification framework that produces audit evidence |
| IS-09 | Implementation of workflow_logger.py (audit trail generation) |
| IS-10 | Implementation of pipeline_manifest.py (provenance manifest) |
| IS-11 | Implementation of run_all_tests.py (test orchestration) |

Document Maintenance:

Next Review: When OSE reporting requirements change, or when new regulatory obligations are identified (e.g., additional spring monitoring, new Compact accounting rules)

Change Triggers: Changes to OSE report format, new Compact litigation requirements, addition of new wells to the wellfield, changes to the depletion accounting framework

# Appendix C: Implementation Specifications

# IS-01: Project Scaffold & Constants

Tier 2 Implementation Specification – A complete Claude Code session-sized prompt. Paste into a fresh session with Tier 1 domain docs available to get a working module back. Sequentially ordered with explicit dependencies.

Status: src/constants.py has been implemented as the centralized constants module. All pipeline scripts import constants from from src.constants import ... (see step1_ingest_buckman_data.py, step2_update_modflow.py, step4_generate_depletion_tables.py, and stream_depletions.py).

## 1. Session Goal

Set up the Buckman Wellfield depletion pipeline project directory structure, requirements.txt, pyproject.toml, and shared constants module (src/constants.py).

## 2. Prerequisites

### Prior Specs (must be complete)

- None – this is the foundational scaffold.

### Input Files (must exist)

| File | Path | Description |
| --- | --- | --- |
| None | N/A | This spec creates the project from scratch |

### Domain Knowledge

- See DS-01 for well field layout and OSE permit numbering

- See DS-02 for unit conversion factors (MG-to-AF, AF-to-ft3)

- See DS-03 for MODFLOW grid cell mapping

## 3. Context for Claude Code

The Buckman Wellfield depletion pipeline processes annual pumping data from the City of Santa Fe’s 13 production wells. Data flows through 5 pipeline steps: CSV ingestion, MODFLOW update, MODFLOW execution, stream depletion post-processing, and depletion table generation.

All pipeline steps share conversion constants, well mappings, and tolerance thresholds. Centralizing these in src/constants.py prevents duplication and ensures consistency.

### Key Equations (Inline)

MG to AF:  AF = MG * 3.06889
           (1 acre-foot = 325,851 gallons; 1,000,000 / 325,851 = 3.06889)

AF to ft3/s:  rate = -(AF / num_layers) * 43560 / (days_in_month * 86400)
              (1 acre-foot = 43,560 ft3; 1 day = 86,400 seconds)

### Key Constants (Inline)

| Constant | Value | Units |
| --- | --- | --- |
| MG_TO_AF_FACTOR | 3.06889 | AF/MG |
| ACRE_FT_TO_FT3 | 43560 | ft3/AF |
| SECONDS_PER_DAY | 86400 | s/day |
| NUM_LAYERS | 2 | dimensionless |
| NOISE_THRESHOLD_MGD | 0.0015 | MGD |
| DAILY_SUM_TOLERANCE_INFO_MGD | 0.001 | MGD |
| DAILY_SUM_TOLERANCE_ERROR_MGD | 0.005 | MGD |
| ANNUAL_SUM_TOLERANCE_MG | 0.01 | MG |
| RATE_TOLERANCE | 0.0001 | ft3/s |

## 4. Requirements (Numbered, Testable)

| Req | Description | Acceptance Criterion |
| --- | --- | --- |
| R1 | Create directory structure with all required directories | ls confirms all directories exist: input/csv/, input/modflow/2023/, output/ingested_data/, output/modflow/, output/depletion/, validation/, tests/, src/, docs/, archive/ (superseded development artifacts) |
| R2 | Create requirements.txt with pinned minimum versions | File contains: pandas>=1.5.0, pint>=0.20.0, openpyxl>=3.0.0, pytest>=7.0.0, ruff>=0.1.0, mypy>=1.0.0. Note: PyYAML and pytest-json-report are not currently in requirements.txt. |
| R3 | Create ruff.toml and mypy.ini with linting/type-checking config | ruff check src/ and mypy src/ run without configuration errors. Note: the project uses ruff.toml and mypy.ini, not pyproject.toml. |
| R4 | Create src/__init__.py (empty) | Python can import from src package |
| R5 | Create src/constants.py with all shared constants | All constants importable: from src.constants import MG_TO_AF_FACTOR – verified working in all pipeline scripts |
| R6 | Define WELL_OSE_MAP: well number (1-13) to OSE permit string | WELL_OSE_MAP[1] == "RG-20516-S-5" and WELL_OSE_MAP[3] == "RG-20516-S" |
| R7 | Define WELL_NAME_MAP: well number (1-13) to MODFLOW name | WELL_NAME_MAP[3] == "BUCKMAN 3A" (not “BUCKMAN 3”) |
| R8 | Define WELL_GRID_MAP: MODFLOW name to (row, col) tuple | WELL_GRID_MAP["BUCKMAN 1"] == (13, 11) and WELL_GRID_MAP["BUCKMAN 13"] == (20, 16) |
| R9 | Define CSV_WELL_COLUMNS list (13 entries) | len(CSV_WELL_COLUMNS) == 13 and CSV_WELL_COLUMNS[0] == "BWell 1\¦Flow Mgd" |
| R10 | Define MONTHS_ABBREV tuple and MONTHS_ORDERED tuple | MONTHS_ABBREV[0] == "JAN" and MONTHS_ORDERED[0] == ("01", "JAN") |
| R11 | Define BASELINE_FILES_TO_COPY list (10 files) | List contains modflow96.exe, sflcs.bcf, sfmodflx_2245.exe, etc. |
| R12 | Define print_error() function with 5-element forensic format | Calling print_error("msg", "loc", "act", "exp", "ctx") prints structured error |
| R13 | Define ABOVE_OTOWI_CELLS and BELOW_OTOWI_CELLS sets | Sets contain MODFLOW cell coordinates for stream depletion grouping |
| R14 | Define CORE_2003_POJOAQUE and CORE_2003_TESUQUE dicts | Dicts map cell coordinates to 2003-baseline depletion fractions |
| R15 | Define LA_CIENEGA_CUMULATIVE list | List contains cumulative La Cienega Springs depletion data |

## 5. Worked Example

### WELL_OSE_MAP (complete mapping)

WELL_OSE_MAP = {
    1: "RG-20516-S-5",
    2: "RG-20516-S-6",
    3: "RG-20516-S",       # Combined wells 3/3A
    4: "RG-20516-S-2",
    5: "RG-20516-S-3",
    6: "RG-20516-S-4",
    7: "RG-20516-S-7",
    8: "RG-20516-S-8",
    9: "RG-20516-S-9",
    10: "RG-20516-S-10",
    11: "RG-20516-S-11",
    12: "RG-20516-S-12",
    13: "RG-20516-S-13",
}

### WELL_GRID_MAP (complete mapping)

WELL_GRID_MAP = {
    "BUCKMAN 1":  (13, 11),
    "BUCKMAN 2":  (14, 11),
    "BUCKMAN 3A": (14, 11),   # Same cell as BUCKMAN 2
    "BUCKMAN 4":  (14, 11),   # Same cell as BUCKMAN 2
    "BUCKMAN 5":  (15, 12),
    "BUCKMAN 6":  (14, 12),
    "BUCKMAN 7":  (13, 11),   # Same cell as BUCKMAN 1
    "BUCKMAN 8":  (13, 11),   # Same cell as BUCKMAN 1
    "BUCKMAN 9":  (14, 12),   # Same cell as BUCKMAN 6
    "BUCKMAN 10": (17, 13),
    "BUCKMAN 11": (19, 14),
    "BUCKMAN 12": (19, 15),
    "BUCKMAN 13": (20, 16),
}

### WELL_NAME_MAP (complete mapping)

WELL_NAME_MAP = {
    1: "BUCKMAN 1",   2: "BUCKMAN 2",   3: "BUCKMAN 3A",
    4: "BUCKMAN 4",   5: "BUCKMAN 5",   6: "BUCKMAN 6",
    7: "BUCKMAN 7",   8: "BUCKMAN 8",   9: "BUCKMAN 9",
    10: "BUCKMAN 10", 11: "BUCKMAN 11", 12: "BUCKMAN 12",
    13: "BUCKMAN 13",
}

### print_error() usage

>>> print_error(
...     "CSV file not found",
...     "./input/csv/Buckman_Well_Prod_2024.csv",
...     "File does not exist",
...     "CSV file with 366 daily records",
...     "Missing source data - cannot process 2024 pumping records"
... )
ERROR: CSV file not found
  Location: ./input/csv/Buckman_Well_Prod_2024.csv
  Actual: File does not exist
  Expected: CSV file with 366 daily records
  Physical context: Missing source data - cannot process 2024 pumping records

### BASELINE_FILES_TO_COPY (complete list)

BASELINE_FILES_TO_COPY = [
    "modflow96.exe",
    "sflcs.bcf",
    "sflcs.sip",
    "thruCY2165.bas",
    "thruCY2165.ghb",
    "thruCY2165.oc",
    "thruCY2165.riv",
    "sfmodflx_2245.exe",
    "verify_modflow_run.py",
    "verify_depletion.py",
]

## 6. Files to Create/Modify

| Action | Path | Description |
| --- | --- | --- |
| Create | src/__init__.py | Empty init file for Python package |
| Create | src/constants.py | All shared constants, mappings, and print_error() |
| Create | requirements.txt | Python dependencies with minimum versions |
| Create | ruff.toml and mypy.ini | Ruff and mypy configuration (not pyproject.toml) |
| Create | tests/__init__.py | Empty init file for test package |
| Create | tests/test_constants.py | Unit tests for constants module. Note: no dedicated tests/test_constants.py exists in the current codebase; constants are tested as part of individual module tests (e.g., tests/test_ingest_buckman_data.py, tests/test_update_modflow.py). |

## 7. Acceptance Criteria

# These commands must all pass:
pytest tests/test_constants.py -v --tb=short    # See note below
ruff check src/constants.py
mypy src/constants.py

Note: No dedicated tests/test_constants.py exists in the current codebase. Constants are tested as part of individual module tests (e.g., tests/test_ingest_buckman_data.py, tests/test_update_modflow.py).

Expected output: - All tests pass (verifying constant values, mapping completeness, print_error output) - No ruff lint errors - No mypy type errors

## 8. Known Gotchas

- ☐ Well 3 maps to “BUCKMAN 3A” (not “BUCKMAN 3”) – historical naming convention for combined wells 3/3A

- ☐ Multiple wells share the same MODFLOW grid cell (e.g., BUCKMAN 1, 7, 8 all at row=13, col=11). This is intentional – they are separate physical wells in the same model cell.

- ☐ CSV_WELL_COLUMNS uses pipe separator: "BWell 1|Flow Mgd" not "BWell 1 Flow Mgd"

- ☐ WELL_OSE_MAP well 3 is "RG-20516-S" (no suffix), while all others have -S-N format

- ☐ BASELINE_FILES_TO_COPY has exactly 10 files – includes both executables (modflow96.exe, sfmodflx_2245.exe) and two Python verification scripts

## 9. Year-Chaining Behavior

| Data Element | Source for Year N | Source for Year N+1 |
| --- | --- | --- |
| Constants | src/constants.py (static) | Same src/constants.py |
| Directory structure | Created once | Reused |

Constants do not change between years. This module is year-independent.

## 10. Verification

Single command to confirm the module works end-to-end:

python3 -c "from src.constants import MG_TO_AF_FACTOR, WELL_OSE_MAP, WELL_GRID_MAP, WELL_NAME_MAP, CSV_WELL_COLUMNS, MONTHS_ABBREV; print(f'MG_TO_AF={MG_TO_AF_FACTOR}, wells={len(WELL_OSE_MAP)}, grid={len(WELL_GRID_MAP)}, cols={len(CSV_WELL_COLUMNS)}')"

Expected result: MG_TO_AF=3.06889, wells=13, grid=13, cols=13

## 11. Cross-References

| Related Spec | Relationship |
| --- | --- |
| DS-01 | Well field layout, OSE permits, physical well locations |
| DS-02 | Unit conversion factors (MG-to-AF, AF-to-ft3) |
| DS-03 | MODFLOW grid cell assignments |
| IS-02 | Uses constants for CSV ingestion (CSV_WELL_COLUMNS, MG_TO_AF_FACTOR, tolerances) |
| IS-03 | Uses constants for Table 1 generation (WELL_OSE_MAP, WELL_NAME_MAP) |
| IS-04 | Uses constants for WEL file management (WELL_GRID_MAP, ACRE_FT_TO_FT3, NUM_LAYERS) |

# IS-02: CSV Ingestion & Table 2

Tier 2 Implementation Specification – A complete Claude Code session-sized prompt. Paste into a fresh session with Tier 1 domain docs available to get a working module back. Sequentially ordered with explicit dependencies.

## 1. Session Goal

Implement step1_ingest_buckman_data.py – CSV parsing, daily data validation, monthly aggregation with pint dimensional analysis, Table 2 generation (CSV and formatted XLSX), and annual sum verification.

## 2. Prerequisites

### Prior Specs (must be complete)

- IS-01: Project Scaffold & Constants (provides src/constants.py)

### Input Files (must exist)

| File | Path | Description |
| --- | --- | --- |
| Source CSV | input/csv/Buckman_Well_Prod_{year}.csv | Daily MGD values for 13 wells, provided by City of Santa Fe |
| Table 1 template | validation/Table_1_data_afy_{year}.xlsx OR output/ingested_data/{year-1}_Table_1_updated.xlsx | Historical AFY data for Table 1 chaining (see IS-03) |

### Domain Knowledge

- See DS-01 for well numbering and CSV column format

- See DS-02 for MG-to-AF conversion and tolerance rationale

## 3. Context for Claude Code

The City of Santa Fe provides a CSV file each year with daily pumping data for the 13 Buckman wells. Each row is one day, each column is one well’s flow in MGD (million gallons per day). The CSV also contains a “Sum” row with annual totals per well (in MG) and a “BWP|Flow Mgd|MGD|Formula” column with daily total production.

This script aggregates daily MGD into monthly MG (dimensional analysis: sum(MGD) * 1 day = MG), converts to acre-feet, and produces Table 2 (monthly AFY grid for 13 wells).

### Key Equations (Inline)

Monthly volume (MG):  MG = sum(daily_MGD_values) * 1 day
                      (pint: sum(MGD * ureg.day) -> million_gallon)

MG to AF:             AF = MG * 3.06889
                      (pint: mg_qty.to(ureg.acre_foot))

Three-tier daily verification:
  Tier 1 (Noise):     BWP > 0 and BWP < 0.0015 MGD -> INFO
  Tier 2 (Formula):   All wells = 0 and BWP >= 0.0015 MGD -> ERROR
  Tier 3 (Rounding):  |calc_sum - BWP| <= 0.001 -> OK
                      |calc_sum - BWP| <= 0.005 -> INFO
                      |calc_sum - BWP| > 0.005  -> ERROR

### Key Constants (Inline)

| Constant | Value | Units | Purpose |
| --- | --- | --- | --- |
| MG_TO_AF_FACTOR | 3.06889 | AF/MG | Volume conversion |
| NOISE_THRESHOLD_MGD | 0.0015 | MGD | Tier 1 database artifact threshold |
| DAILY_SUM_TOLERANCE_INFO_MGD | 0.001 | MGD | Tier 3 INFO threshold |
| DAILY_SUM_TOLERANCE_ERROR_MGD | 0.005 | MGD | Tier 3 ERROR threshold |
| ANNUAL_SUM_TOLERANCE_MG | 0.01 | MG | Annual sum verification tolerance |

## 4. Requirements (Numbered, Testable)

| Req | Description | Acceptance Criterion |
| --- | --- | --- |
| R1 | read_source_csv(csv_path) -> (daily_df, sum_series) parses CSV, separates daily rows from summary rows | Returns DataFrame with 365/366 daily rows and Series with 13 annual MG totals |
| R2 | CSV first column (date range header) renamed to Date; daily rows have valid datetime; BWP total column renamed to BWP_Total | daily_df['Date'].dtype == datetime64, 'BWP_Total' in daily_df.columns |
| R3 | validate_daily_data(daily_df) -> flags_df flags BLANK and NEGATIVE | flags_df has same shape as well columns; zero values are valid (not flagged). Note: NON_NUMERIC is not a separate flag – non-numeric values are converted to NaN via pd.to_numeric(errors='coerce') before validation, so they appear as BLANK. |
| R4 | verify_daily_sums(daily_df) -> verification_df applies three-tier severity | verification_df has columns: Date, Calculated_Sum, BWP_Total, Difference, Severity |
| R5 | aggregate_monthly(daily_df, flags_df) -> dict[str, DataFrame] groups by month | Returns dict with keys “01”-“12”, each DataFrame has Well_Number, MG_Month (pint Quantity), Has_Flagged_Data |
| R6 | Monthly MG uses pint dimensional analysis: sum(MGD * ureg.day) | monthly_data["01"].iloc[0]['MG_Month'].units == ureg.million_gallon |
| R7 | generate_table2_output(monthly_data, year, output_dir) writes CSV and XLSX | CSV has 14 rows (13 wells + Total) x 14 columns (Well, JAN-DEC, Total) + summary rows |
| R8 | Table 2 XLSX uses Aptos font, #,##0.00 number format, medium/hair borders | Visual comparison to validation/Table_2_2024.xlsx matches styling |
| R9 | verify_annual_sums(monthly_data, sum_row) checks 12-month totals vs CSV Sum row | All 13 wells within 0.01 MG tolerance |
| R10 | generate_monthly_csv() writes per-month CSV with OSE number, well name, MG, AF, data quality | 12 CSV files created in output directory |
| R11 | generate_qa_summary() writes input_summary.csv with flagged wells and daily mismatches | File created with two sections (or “No data quality issues found”) |
| R12 | Main function accepts --year argument and orchestrates full pipeline | python3 step1_ingest_buckman_data.py --year 2024 runs successfully |

## 5. Worked Example

### Input: Buckman #1, January 2024

Daily MGD values for 31 days (from CSV):
Day 1:  0.2057 MGD    Day 11: 0.1773 MGD    Day 21: 0.1832 MGD    Day 31: 0.1654 MGD
Day 2:  0.1925 MGD    Day 12: 0.0000 MGD    Day 22: 0.1745 MGD
...
(31 total daily values)

### Calculation Steps

Step 1: Sum daily MGD values for January
  sum(daily_mgd) = 5.503 MGD*days = 5.503 MG
  (Using pint: sum(0.2057 + 0.1925 + ... ) * ureg.MGD * ureg.day = 5.503 * ureg.MG)

Step 2: Convert MG to AF
  AF = 5.503 MG * 3.06889 AF/MG = 16.887963 AF
  (Using pint: (5.503 * ureg.million_gallon).to(ureg.acre_foot) = 16.887963 AF)

Step 3: Verify daily sum for January 1
  Calculated_Sum = BWell1 + BWell2 + ... + BWell13 = 0.2057 + 0 + ... = 0.4523 MGD
  BWP_Total = 0.4523 MGD (from CSV formula column)
  Difference = |0.4523 - 0.4523| = 0.0000 MGD
  Severity = OK (within 0.001 MGD)

### Expected Output: Table 2 CSV (2024_Table_2_output.csv)

Well,JAN,FEB,MAR,APR,MAY,JUN,JUL,AUG,SEP,OCT,NOV,DEC,Total
1,16.887963,38.805796,41.370397,...,162.488610,...,0.000000,601.276042
2,0.000000,0.006135,0.000000,...,0.000000,...,0.000000,0.009202
...
13,18.099940,0.000000,4.293040,...,55.001150,...,0.000000,212.020480
Total,19.393860,39.388656,41.629337,...,302.058090,...,65.677300,1372.954123

Wells 10-13,386.860000,28.2%
Wells 1,7,8,713.750000,52.0%

### Three-Tier Daily Verification Example

Tier 1 (Noise): 2024-06-23 BWP=0.0009 MGD, all wells=0 -> INFO (below 0.0015
threshold)
Tier 2 (Formula): 2024-07-15 BWP=0.05 MGD, all wells=0 -> ERROR (formula 
inconsistency)
Tier 3 (Rounding): 2024-03-10 calc=4.523, BWP=4.524, diff=0.001 -> OK (within 0.001)

## 6. Files to Create/Modify

| Action | Path | Description |
| --- | --- | --- |
| Create | step1_ingest_buckman_data.py | Main ingestion script with all functions |
| Create | tests/test_ingest_buckman_data.py | Unit tests for CSV parsing, validation, aggregation |

### Output files generated at runtime

| File | Path | Description |
| --- | --- | --- |
| Monthly CSVs | output/ingested_data/{year}_{MM}_{MON}.csv | 12 monthly per-well production files |
| Table 2 CSV | output/ingested_data/{year}_Table_2_output.csv | Monthly AFY grid |
| Table 2 XLSX | output/ingested_data/{year}_Table_2_output.xlsx | Formatted Excel version |
| QA summary | output/ingested_data/input_summary.csv | Flagged data and mismatches |

## 7. Acceptance Criteria

# These commands must all pass:
pytest tests/test_ingest_buckman_data.py -v --tb=short
ruff check step1_ingest_buckman_data.py
mypy step1_ingest_buckman_data.py

# End-to-end run (requires input CSV):
python3 step1_ingest_buckman_data.py --year 2024

Expected output: - 2024_Table_2_output.csv with 14 data rows + 3 summary rows - 2024_Table_2_output.xlsx with Aptos font formatting - 12 monthly CSV files - Annual verification: 13 wells OK - Total annual pumping: ~1372.95 AFY (2024 data)

## 8. Known Gotchas

- ☐ Pint unit registry must define custom units: million_gallon = 1e6 * gallon = MG and million_gallon_per_day = million_gallon / day = MGD. These are not built into pint.

- ☐ CSV first column header contains a date range (e.g., “1/1/2024-12/31/2024”), not “Date”. Must rename to “Date” after reading.

- ☐ Summary rows (Sum, Avg, Max, Min) are identified by non-parseable dates. Use pd.to_datetime(errors='coerce') to separate daily rows from summary rows.

- ☐ The BWP formula column header varies: "BWP|Flow Mgd|MGD|Formula". Must use CSV_TOTAL_COLUMN constant.

- ☐ Zero values (0.0 MGD) are valid – they mean the well was not pumping. Only NaN, negative, and non-numeric values get flagged.

- ☐ Table 2 XLSX: Well column contains integers (1-13), not strings. Summary rows use “Wells 10-13” string in Well column.

- ☐ The Table 2 XLSX summary area (Wells 10-13, Wells 1,7,8) is placed in columns Q-T, rows 14-15 – separate from the main data grid.

- ☐ MG-to-AF conversion: use pint’s .to(ureg.acre_foot) for dimensional safety, not manual multiplication. Both approaches should give the same result.

## 9. Year-Chaining Behavior

| Data Element | Source for Year N | Source for Year N+1 |
| --- | --- | --- |
| Input CSV | input/csv/Buckman_Well_Prod_{N}.csv | input/csv/Buckman_Well_Prod_{N+1}.csv |
| Table 2 output | Computed from year N CSV | Computed from year N+1 CSV |
| Table 1 template | validation/Table_1_data_afy_{N}.xlsx | output/ingested_data/{N}_Table_1_updated.xlsx (output from year N) |

Table 2 is computed fresh each year from that year’s CSV data. No chaining is needed for Table 2 itself. Table 1 chaining is handled by IS-03.

## 10. Verification

Single command to confirm the module works end-to-end:

python3 step1_ingest_buckman_data.py --year 2024 && \
  python3 -c "
import pandas as pd
df = pd.read_csv('output/ingested_data/2024_Table_2_output.csv')
wells = df[df['Well'].apply(lambda x: str(x).isdigit())]
total_row = df[df['Well'] == 'Total']
print(f'Wells: {len(wells)}, Total AFY: {float(total_row.iloc[0][\"Total\"]):.2f}')
assert len(wells) == 13, f'Expected 13 wells, got {len(wells)}'
print('PASS')
"

Expected result: Wells: 13, Total AFY: 1372.95 followed by PASS

## 11. Cross-References

| Related Spec | Relationship |
| --- | --- |
| DS-01 | Well numbering, CSV column format |
| DS-02 | MG-to-AF conversion factor, tolerance thresholds |
| IS-01 | Constants module (MG_TO_AF_FACTOR, CSV_WELL_COLUMNS, WELL_OSE_MAP, tolerances) |
| IS-03 | Table 1 chaining consumes Table 2 annual totals |
| IS-04 | WEL file generation reads Table 2 CSV output |

# IS-03: Table 1 Chaining

Tier 2 Implementation Specification – A complete Claude Code session-sized prompt. Paste into a fresh session with Tier 1 domain docs available to get a working module back. Sequentially ordered with explicit dependencies.

## 1. Session Goal

Implement Table 1 generation – load a prior-year XLSX template containing the historical annual pumping record (1988-present), append a new row with the current year’s well-by-well annual AF totals from Table 2 output, preserve all historical rows unchanged, add statistics and ranking rows, and export to both CSV and formatted XLSX.

## 2. Prerequisites

### Prior Specs (must be complete)

- IS-01: Project Scaffold & Constants (provides src/constants.py with WELL_OSE_MAP, WELL_NAME_MAP)

- IS-02: CSV Ingestion & Table 2 (produces {year}_Table_2_output.csv with monthly AFY grid and annual totals per well)

### Input Files (must exist)

| File | Path | Description |
| --- | --- | --- |
| Table 1 template (primary) | validation/Table_1_data_afy_{year}.xlsx | Historical AFY data Excel file (1988-{year-1}) for first-time processing |
| Table 1 template (fallback) | output/ingested_data/{year-1}_Table_1_updated.xlsx | Prior year’s Table 1 output (used for year-chaining when no validation file exists). Known issue: The write path is flat (output/ingested_data/{year}_Table_1_updated.xlsx), but the fallback lookup path in code uses a subdirectory (output/ingested_data/{year-1}/{year-1}_Table_1_updated.xlsx). This path discrepancy is a known code bug. |
| Table 2 output | output/ingested_data/{year}_Table_2_output.csv | Current year’s monthly AFY grid – the Total column provides annual well totals consumed here |

### Domain Knowledge

- See DS-01 for well numbering conventions (Well 3 is labeled “3/3A” in Table 1)

- See DS-02 for AFY volume units and conversion factors

- See DS-04 Section 6.1 for Table 1 chaining architecture and year-chaining rules

## 3. Context for Claude Code

Table 1 is the historical annual pumping record for the Buckman Well Field, spanning 1988 to the present year. It is the single authoritative registry of how much water each well pumped in each year. Every year, the pipeline loads the prior year’s Table 1 output, appends one new row for the current year, recalculates statistics, and writes updated CSV and XLSX files.

Structure: - Data rows: Years 1988 through current year (one row per year, one column per well) - Average rows: “Average, 1988-{year}” and “Average, 2022-{year}” (using Excel AVERAGE formulas, not static values) - Statistics rows: Per-well percentage of annual total, Wells 10-13 group sum and percentage, Wells 1,7,8 group sum and percentage - Sort column: All years’ totals sorted descending (column T), with ranks for 2013+ years (column U) since Wells 10-13 came online in 2013

Template resolution order: 1. Primary: validation/Table_1_data_afy_{year}.xlsx (hand-prepared file for baseline year 2024) 2. Fallback: output/ingested_data/{year-1}_Table_1_updated.xlsx (chained from prior year’s run) 3. Neither: pipeline halts with forensic error message

Current-year data source: Annual AFY totals per well are computed by summing all 12 monthly MG values from the IS-02 monthly aggregation, converting each to AF via pint, and passing the 13-well dict to generate_table1_output().

### Key Equations (Inline)

Well percentage:      pct_i = (well_i_AFY / total_AFY) * 100
                      where total_AFY = sum of all 13 wells for current year

Wells 10-13 sum:      sum_10_13 = well_10 + well_11 + well_12 + well_13
Wells 10-13 pct:      pct_10_13 = sum_10_13 / total_AFY * 100

Wells 1,7,8 sum:      sum_1_7_8 = well_1 + well_7 + well_8
Wells 1,7,8 pct:      pct_1_7_8 = sum_1_7_8 / total_AFY * 100

Year rank:            Rank all years by Total AFY ascending (1 = lowest pumping year)
                      All years get a rank; Sort column T displays totals descending;
                      Column U shows rank only for years >= 2013

### Key Constants (Inline)

| Constant | Value | Notes |
| --- | --- | --- |
| Well 3 column header | "3/3A" | String, not integer 3 – historical convention for combined wells 3 and 3A |
| Year column header | "Well:" | First column in Table 1 template DataFrame |
| Wells 10-13 start year | 2013 | Wells 10-13 came online in 2013; ranking in column U is limited to 2013+ |
| MG_TO_AF_FACTOR | 3.06889 | AF/MG – used upstream in IS-02 for the conversion that feeds Table 1 |
| Total column | "Total" | Annual sum across all 13 wells |
| Sort column | "Total, Sort" | Rank index for year ordering |

## 4. Requirements (Numbered, Testable)

| Req | Description | Acceptance Criterion |
| --- | --- | --- |
| R1 | generate_table1_output(year_afy_data, year, output_dir) loads Table 1 template from validation file (primary) or prior year output (fallback) | Template loads successfully; table1_df has all historical year rows; function returns None on success, 1 on failure |
| R2 | Read annual totals per well from year_afy_data dict (13 wells, AFY values) | year_afy_data = {1: 601.276042, 2: 0.009202, ..., 13: 212.020480} with all 13 keys present |
| R3 | Append new row for current year with Well 3 mapped to column "3/3A" | New row has Well: = year; row_2024["3/3A"] contains Well 3 value; all 13 well columns populated |
| R4 | If current year already exists in template, update in place (no duplicates) | Running twice for same year does not create duplicate rows; second run updates values |
| R5 | Preserve all historical rows exactly as they appear in the template | Compare rows 1988-{year-1}: values unchanged within floating-point precision (no rounding, no truncation) |
| R6 | Calculate Total column as sum of all 13 well values | row_2024["Total"] == sum(year_afy_data.values()) rounded to 6 decimal places |
| R7 | Detect and recalculate missing Total values (NaN) in historical rows from well columns | Any year with NaN Total gets recalculated as sum(wells 1-13) with warning printed |
| R8 | Calculate Sort column: rank all years by Total AFY (1 = lowest pumping) | Year with lowest total gets rank 1; highest gets rank N; all years ranked |
| R9 | Add “% of Total” row: each well’s percentage of current year’s annual total | sum(all_well_pcts) == "100.0%"; row label is "Pecent" in XLSX (matching validation typo) |
| R10 | Add “Wells 10-13” sum row and “Wells 10-13 %” percentage row | Sum placed in well 10 column; percentage placed in well 10 column |
| R11 | Add “Wells 1,7,8” sum row and “Wells 1,7,8 %” percentage row | Sum placed in well 1 column; percentage placed in well 1 column |
| R12 | Write CSV: {year}_Table_1_updated.csv | CSV readable by pd.read_csv(); contains year data rows + statistics rows |
| R13 | Write XLSX: {year}_Table_1_updated.xlsx with Aptos font, medium/hair borders, #,##0.00 number format | Visual comparison to validation/Table_1_data_afy_2024.xlsx matches styling |
| R14 | XLSX includes Average rows with Excel formulas (not static values) | “Average, 1988-{year}” uses =AVERAGE(B2:B{last_row}); “Average, 2022-{year}” uses last 3 rows |
| R15 | XLSX includes repeat header row and percentage row with Excel formulas | Percentage row formulas: =B{last_year_row}/$O${last_year_row} referencing current year data |
| R16 | XLSX column T: all years’ totals sorted descending; column U: rank for years >= 2013 only | Sorted list is independent of data row order; column U populated only for 2013+ entries |
| R17 | Row count validation: output has exactly (prior year rows + 1) data rows | If template has 36 year rows (1988-2023), output has 37 year rows (1988-2024) |

## 5. Worked Example

### Input: Table 1 Template (1988-2023, 36 year rows)

Well:   1        2      3/3A     4      5      6      7       8       9      10      11      12     13     Total
1988    0.00     0.00   0.00     0.00   0.00   0.00   598.44  326.41  323.87 0.00    0.00    0.00   0.00   1248.72
1989    0.00     0.00   0.00     0.00   0.00   0.00   1142.91 585.99  326.80 0.00    0.00    0.00   0.00   2055.70
...
2023    451.22   0.00   77.80    0.00   0.00   0.00   17.22   0.37    53.57  211.08  185.44  103.37 195.30 1295.48

### Input: Table 2 Annual Totals for 2024 (from IS-02 output)

year_afy_data = {
    1: 601.276042,   2: 0.009202,   3: 0.033721,
    4: 0.000000,     5: 0.000000,   6: 0.000000,
    7: 57.637710,    8: 54.836285,  9: 60.279885,
    10: 101.200850,  11: 73.639045, 12: 0.000000,
    13: 212.020480
}
total_afy = sum(year_afy_data.values())  # = 1372.954123 AF

### Calculation Steps

Step 1: Load template
  Read validation/Table_1_data_afy_2024.xlsx
  -> 36 year rows (1988-2023), columns: Well:, 1, 2, 3/3A, 4-13, Total

Step 2: Build new row
  row_2024 = {
      "Well:": 2024,
      1: 601.276042,  2: 0.009202,  "3/3A": 0.033721,
      4: 0.000000,    5: 0.000000,  6: 0.000000,
      7: 57.637710,   8: 54.836285, 9: 60.279885,
      10: 101.200850, 11: 73.639045, 12: 0.000000,
      13: 212.020480,
      "Total": 1372.954123
  }
  Note: Well 3 value goes to column "3/3A" (not column 3)

Step 3: Check for duplicate
  2024 not in existing years -> append (not update)
  Result: 37 year rows (1988-2024)

Step 4: Calculate Sort column (rank by Total ascending, 1 = lowest)
  Rank all 37 years by Total AFY:
  Rank 1 = lowest total (~748 AF, e.g., 2020)
  ...
  Rank 37 = highest total (~2648 AF, e.g., 2002)
  2024 rank ~= 21 of 37

Step 5: Calculate statistics for 2024
  % of Total:  Well 1 = 601.28 / 1372.95 * 100 = 43.8%
               Well 2 = 0.01 / 1372.95 * 100 = 0.0%
               ...
               Well 13 = 212.02 / 1372.95 * 100 = 15.4%
  Wells 10-13: 101.20 + 73.64 + 0.00 + 212.02 = 386.860375 AF (28.2%)
  Wells 1,7,8: 601.28 + 57.64 + 54.84 = 713.750037 AF (52.0%)

### Expected Output: Table 1 CSV (2024_Table_1_updated.csv)

Well:,1,2,3/3A,4,5,6,7,8,9,10,11,12,13,Total,"Total, Sort"
1988,0.00,0.00,0.00,0.00,0.00,0.00,598.44,326.41,323.87,0.00,0.00,0.00,0.00,1248.72,1
...
2023,451.22,0.00,77.80,0.00,0.00,0.00,17.22,0.37,53.57,211.08,185.44,103.37,195.30,1295.48,20
2024,601.276042,0.009202,0.033721,0.0,0.0,0.0,57.63771,54.836285,60.279885,101.20085,73.639045,0.0,212.02048,1372.954123,21
% of Total,43.8%,0.0%,0.0%,0.0%,0.0%,0.0%,4.2%,4.0%,4.4%,7.4%,5.4%,0.0%,15.4%,100.0%,
Wells 10-13,,,,,,,,,,386.860375,,,,
Wells 10-13 %,,,,,,,,,,28.2%,,,,
"Wells 1,7,8",713.750037,,,,,,,,,,,,,
"Wells 1,7,8 %",52.0%,,,,,,,,,,,,,

### XLSX Average Row Formulas

Row 39 (Average, 1988-2024):
  Cell B39 = =AVERAGE(B2:B38)      -- averages well 1 across all 37 years
  Cell O39 = =AVERAGE(O2:O38)      -- averages Total column

Row 40 (Average, 2022-2024):
  Cell B40 = =AVERAGE(B36:B38)     -- averages well 1 across last 3 years
  Cell O40 = =AVERAGE(O36:O38)     -- averages Total column

## 6. Files to Create/Modify

| Action | Path | Description |
| --- | --- | --- |
| Modify | step1_ingest_buckman_data.py | Contains generate_table1_output() and write_table1_xlsx() functions |
| Create | tests/test_ingest_buckman_data.py | Unit tests for Table 1 chaining, row preservation, statistics, and duplicate handling. Note: Table 1 tests are in tests/test_ingest_buckman_data.py, not a separate tests/test_table1.py. |

### Output files generated at runtime

| File | Path | Description |
| --- | --- | --- |
| Table 1 CSV | output/ingested_data/{year}_Table_1_updated.csv | Historical AFY data with new year appended + statistics rows |
| Table 1 XLSX | output/ingested_data/{year}_Table_1_updated.xlsx | Formatted Excel with Aptos font, borders, formulas |

## 7. Acceptance Criteria

# These commands must all pass:
pytest tests/test_ingest_buckman_data.py -v --tb=short
ruff check step1_ingest_buckman_data.py
mypy step1_ingest_buckman_data.py

# End-to-end run (requires IS-02 output and template file):
python3 step1_ingest_buckman_data.py --year 2024

Expected output: - Table 1 contains all historical years (1988-2023) plus 2024 = 37 data rows - 2024 row total = 1372.95 AFY (matching Table 2 annual total) - Statistics rows: Wells 10-13 = 386.86 AF (28.2%), Wells 1,7,8 = 713.75 AF (52.0%) - XLSX formatting matches validation file styling (Aptos font, medium/hair borders, #,##0.00) - Sort column ranks 37 years; column U shows ranks only for 2013+ (12 years) - Chaining test: process year 2024, then process year 2025 using 2024’s output as template

## 8. Known Gotchas

- ☐ Well 3 column header is "3/3A" (string), not integer 3. The DataFrame has mixed column types: "Well:" (string), 1, 2 (int), "3/3A" (string), 4-13 (int), "Total" (string). This requires careful handling when building row_2024 dict: use col_name = '3/3A' if well_num == 3 else well_num.

- ☐ The validation file’s “Pecent” row label is a deliberate match to the original hand-built file’s typo. Do not correct to “Percent” – the XLSX must match the validation file exactly.

- ☐ Average rows use Excel formulas (=AVERAGE(...)), not pre-computed values. When openpyxl writes these formulas, Excel must open the file and recalculate before the values appear. The CSV contains the computed static values from Python.

- ☐ When the template already has the current year (re-run scenario), the code detects the existing row via lambda x: isinstance(x, (int, float)) and not pd.isna(x) and int(x) == year and updates in place. This prevents duplicate year rows.

- ☐ NaN values in the Total column for historical years can occur when the template was exported from Excel without recalculating formulas. The code detects these and recalculates from well columns with a warning.

- ☐ The XLSX Sort column (column T) contains all years’ totals sorted descending, independently of the data row order. Column U contains rank integers only for the last N entries corresponding to years >= 2013 (when wells 10-13 came online). Rank is “position from bottom” in the sorted list.

- ☐ The XLSX average rows reference dynamic ranges: “Average, 1988-{year}” uses B2:B{last_year_row} (the full range), while “Average, 2022-{year}” uses only the last 3 year rows: B{last_year_row-2}:B{last_year_row}.

- ☐ The year_afy_data dict passed to generate_table1_output() must have AFY values rounded to 6 decimal places to match the precision of Table 2 output. The Total is calculated as sum(year_afy_data.values()), also rounded to 6 places.

## 9. Year-Chaining Behavior

| Data Element | Source for Year N | Source for Year N+1 |
| --- | --- | --- |
| Table 1 template | validation/Table_1_data_afy_{N}.xlsx (first time) OR output/ingested_data/{N-1}_Table_1_updated.xlsx (chained) | output/ingested_data/{N}_Table_1_updated.xlsx |
| Current year AFY | Computed from year N’s Table 2 monthly aggregation | Computed from year N+1’s Table 2 monthly aggregation |
| Historical rows (1988 through N-1) | Preserved verbatim from template | Preserved verbatim from year N output |
| Statistics rows | Recalculated for year N’s totals | Recalculated for year N+1’s totals |
| Sort/rank column | All years re-ranked including year N | All years re-ranked including year N+1 |
| Average row ranges | Extend to include year N | Extend to include year N+1 |

Critical chaining logic:

1. For the baseline year (2024): try validation/Table_1_data_afy_2024.xlsx (contains 1988-2023, 36 rows)

2. For subsequent years (2025+): try validation/Table_1_data_afy_2025.xlsx first, then fall back to output/ingested_data/2024_Table_1_updated.xlsx (contains 1988-2024, 37 rows)

3. Each year’s output becomes the next year’s input template, accumulating the full historical record 4. Row count invariant: output_rows = template_year_rows + 1 (unless template already had the current year, in which case output_rows = template_year_rows)

## 10. Verification

Single command to confirm the module works end-to-end:

python3 step1_ingest_buckman_data.py --year 2024 && \
python3 -c "
import pandas as pd
df = pd.read_csv('output/ingested_data/2024_Table_1_updated.csv')

# Count year rows (filter out stats rows)
year_rows = df[df['Well:'].apply(lambda x: str(x).replace('.0','').isdigit())]
print(f'Historical years: {len(year_rows)}')
assert len(year_rows) == 37, f'Expected 37 years, got {len(year_rows)}'

# Check 2024 row
row_2024 = year_rows[year_rows['Well:'].astype(float).astype(int) == 2024]
assert len(row_2024) == 1, f'Expected 1 row for 2024, got {len(row_2024)}'
total = float(row_2024.iloc[0]['Total'])
print(f'2024 Total: {total:.2f} AFY')
assert abs(total - 1372.95) < 1.0, f'Total mismatch: {total}'

# Check no duplicate years
years = year_rows['Well:'].astype(float).astype(int).tolist()
assert len(years) == len(set(years)), 'Duplicate year rows found'

print('PASS')
"

Expected result: Historical years: 37, 2024 Total: 1372.95 AFY, PASS

## 11. Cross-References

| Related Spec | Relationship |
| --- | --- |
| DS-01 | Well numbering conventions, “3/3A” historical naming |
| DS-02 | AFY volume units, MG-to-AF conversion factor |
| DS-04 | Year-chaining architecture (Section 6.1: Table 1 chaining rules) |
| IS-01 | Constants module (WELL_OSE_MAP, WELL_NAME_MAP for well numbering) |
| IS-02 | Table 2 output provides annual totals per well consumed by Table 1 |
| IS-04 | WEL file reads Table 2 CSV directly (not Table 1), but Table 1 provides historical pumping context |

# IS-04: WEL File Management

Tier 2 Implementation Specification – A complete Claude Code session-sized prompt. Paste into a fresh session with Tier 1 domain docs available to get a working module back. Sequentially ordered with explicit dependencies.

## 1. Session Goal

Implement step2_update_modflow.py – WEL file parsing, pumping rate conversion (acre-feet to ft3/s), stress period well entry generation, WEL file assembly, NAM file generation, and baseline file copying.

## 2. Prerequisites

### Prior Specs (must be complete)

- IS-01: Project Scaffold & Constants (provides src/constants.py with WELL_GRID_MAP, WELL_NAME_MAP, conversion constants)

- IS-02: CSV Ingestion & Table 2 (produces {year}_Table_2_output.csv)

### Input Files (must exist)

| File | Path | Description |
| --- | --- | --- |
| Table 2 CSV | output/ingested_data/{year}_Table_2_output.csv | Monthly AFY grid from step 1 |
| Source WEL file (baseline) | input/modflow/2023/thruCY2165.wel | Original MODFLOW well file (for year 2024) |
| Source WEL file (chained) | output/modflow/{year-1}/thruCY2165_{year-1}.wel | Prior year’s output WEL file (for year 2025+) |
| Source NAM file | input/modflow/2023/CY2023.nam | Baseline MODFLOW name file |
| Baseline MODFLOW files | input/modflow/2023/ | 10 static files required by MODFLOW96 |

### Domain Knowledge

- See DS-02 for AF-to-ft3/s conversion and MODFLOW conventions

- See DS-03 for MODFLOW grid cell assignments

## 3. Context for Claude Code

MODFLOW96 uses a WEL (well) package file to specify pumping rates at each well for each stress period (month). The WEL file is a large text file spanning years 1947-2165, with each month containing 26 well entries (13 wells x 2 model layers). This script reads the existing WEL file, replaces the target year’s 324 lines (12 months x 27 lines/month) with new pumping data converted from Table 2’s acre-feet values to MODFLOW’s ft3/s convention, and writes the updated file.

Pumping rates are negative in MODFLOW convention (extraction from aquifer). Each well’s monthly pumping volume in acre-feet is split equally between two model layers and converted to a continuous rate in ft3/s.

### Key Equations (Inline)

AF to ft3/s (per layer):
  rate = -(AF / num_layers) * ACRE_FT_TO_FT3 / (days_in_month * SECONDS_PER_DAY)
  rate = -(AF / 2) * 43560 / (days * 86400)

Example: Well 1, JAN 2024 (16.887963 AF, 31 days):
  rate = -(16.887963 / 2) * 43560 / (31 * 86400)
  rate = -8.443982 * 43560 / 2678400
  rate = -367,819.86 / 2678400
  rate = -0.13733 ft3/s

### Key Constants (Inline)

| Constant | Value | Units | Purpose |
| --- | --- | --- | --- |
| ACRE_FT_TO_FT3 | 43560 | ft3/AF | Volume conversion |
| SECONDS_PER_DAY | 86400 | s/day | Time conversion |
| NUM_LAYERS | 2 | dimensionless | Pumping split between 2 model layers |
| LINES_PER_MONTH | 27 | lines | 1 header + 26 well entries per stress period |
| WELLS_PER_MONTH | 26 | entries | 13 wells x 2 layers |
| BASELINE_YEAR | 2024 | year | First year that uses 2023 baseline input |
| RATE_TOLERANCE | 0.0001 | ft3/s | Validation tolerance for rate comparison |

## 4. Requirements (Numbered, Testable)

| Req | Description | Acceptance Criterion |
| --- | --- | --- |
| R1 | YearConfig dataclass with all year-specific paths | config.target_year == 2024, config.source_year == 2023, paths resolve correctly |
| R2 | get_year_config(target_year) resolves input paths: baseline (2024) vs chained (2025+) | For 2024: input_wel_path == "input/modflow/2023/thruCY2165.wel"; for 2025: input_wel_path == "output/modflow/2024/thruCY2165_2024.wel" |
| R3 | parse_wel_file(wel_path, target_year) -> WelFileData splits into pre/target/post sections | len(target_year_lines) == 324 (12 months x 27 lines) |
| R4 | find_year_boundaries() searches for “JAN {year}” and “DEC {year}” patterns | Start index points to header line before first JAN entry; end index is exclusive |
| R5 | WEL file validation: each month has header “26”, first entry is BUCKMAN 1, last is BUCKMAN 13 | ValueError raised if structure doesn’t match |
| R6 | read_table2_pumping_data(csv_path) -> dict[int, dict[str, float]] reads Table 2 | Returns {1: {"JAN": 16.887963, "FEB": 38.805796, ...}, ...} for all 13 wells |
| R7 | convert_af_to_ft3s(acre_feet, days_in_month, num_layers=2) -> float returns negative rate | convert_af_to_ft3s(16.887963, 31, 2) returns -0.13733 (within 0.00001) |
| R8 | generate_well_entries(pumping_data, target_year) -> list[str] produces 324 lines | Well order: BUCKMAN 1 through 13; Layer 1 before Layer 2 for each well |
| R9 | Well entry line format: "{layer:10d}{row:10d}{col:10d} {rate:8.5f} {well_name} {month} {year}\r\n" | Exact format match with CRLF line endings |
| R10 | write_updated_wel_file() assembles pre-target + new target + post-target | Total line count = original line count; pre and post sections byte-identical |
| R11 | generate_nam_file(target_year, output_dir, output_filename) writes MODFLOW name file | NAM file references correct year-specific filenames (WEL, LST, flux files) |
| R12 | copy_baseline_files(output_dir) copies 10 static files from input/modflow/2023/ | All 10 files exist in output directory after copy |
| R13 | verify_nam_file_references() checks all NAM-referenced files exist | Returns True when all referenced files are present |
| R14 | CRLF line endings (\r\n) throughout generated WEL file content | Matches MODFLOW96 Windows convention |
| R15 | Zero pumping rates formatted as -0.00000 (negative zero) | Matches validation file format for inactive wells |
| R16 | Leap year handling: February has 29 days in leap years, 28 in non-leap | convert_af_to_ft3s(10.0, 29) uses 29 days for February 2024 (leap year) |

## 5. Worked Example

### Input: Table 2 CSV for Well 1, January 2024

Well,JAN,FEB,MAR,...,DEC,Total
1,16.887963,38.805796,41.370397,...,0.000000,601.276042

### Calculation Steps

Step 1: Read pumping data
  Well 1, JAN = 16.887963 AF

Step 2: Get days in month
  JAN 2024 = 31 days (2024 is a leap year, but January is unaffected)

Step 3: Convert AF to ft3/s per layer
  rate = -(16.887963 / 2) * 43560 / (31 * 86400)
  rate = -(8.4439815) * 43560 / (2678400)
  rate = -367,819.83 / 2678400
  rate = -0.13733 ft3/s

Step 4: Generate well entry lines (Layer 1 and Layer 2)
  BUCKMAN 1 is at grid position (row=13, col=11)

  Layer 1: "         1        13        11  -0.13733  BUCKMAN 1 JAN 2024\r\n"
  Layer 2: "         2        13        11  -0.13733  BUCKMAN 1 JAN 2024\r\n"

Step 5: Generate month header
  "        26\r\n"

Step 6: Assemble January block (27 lines)
  Line 1: "        26\r\n"                                          (header)
  Line 2: "         1        13        11  -0.13733  BUCKMAN 1 JAN 2024\r\n"  (Well 1, Layer 1)
  Line 3: "         2        13        11  -0.13733  BUCKMAN 1 JAN 2024\r\n"  (Well 1, Layer 2)
  Line 4: "         1        14        11  -0.00000  BUCKMAN 2 JAN 2024\r\n"  (Well 2, Layer 1)
  Line 5: "         2        14        11  -0.00000  BUCKMAN 2 JAN 2024\r\n"  (Well 2, Layer 2)
  ...
  Line 26: "         1        20        16  -0.14716  BUCKMAN 13 JAN 2024\r\n" (Well 13, Layer 1)
  Line 27: "         2        20        16  -0.14716  BUCKMAN 13 JAN 2024\r\n" (Well 13, Layer 2)

### Additional Conversion Examples

Well 1, FEB 2024 (38.805796 AF, 29 days -- leap year):
  rate = -(38.805796 / 2) * 43560 / (29 * 86400)
  rate = -(19.402898) * 43560 / (2505600)
  rate = -845,190.24 / 2505600
  rate = -0.33732 ft3/s

Well 13, JUL 2024 (55.001150 AF, 31 days):
  rate = -(55.001150 / 2) * 43560 / (31 * 86400)
  rate = -(27.500575) * 43560 / (2678400)
  rate = -1,197,925.05 / 2678400
  rate = -0.44725 ft3/s

Well 2, JAN 2024 (0.000000 AF, 31 days):
  rate = -(0.0 / 2) * 43560 / (31 * 86400)
  rate = -0.00000 ft3/s  (formatted as negative zero)

Note: Values shown use rounded inputs; actual pipeline results may differ slightly.

### NAM File Content

# MODFLOW Name File for Buckman Depletion Model - Year 2024
# Automatically generated by Python script on 2026-02-20 10:30:00
# File Type      Unit File Name
#------------------------------------
LIST            23    CY2024.lst
BAS             21    thruCY2165.bas
BCF             11    sflcs.bcf
OC              10    thruCY2165.oc
RIV             14    thruCY2165.riv
GHB             15    thruCY2165.ghb
SIP             17    sflcs.sip
WEL             12    thruCY2165_2024.wel
DATA(BINARY)    24    CY2024_riv.flx
DATA(BINARY)    34    CY2024_ghb.flx

## 6. Files to Create/Modify

| Action | Path | Description |
| --- | --- | --- |
| Create | step2_update_modflow.py | Main MODFLOW update script with all functions |
| Create | tests/test_update_modflow.py | Unit tests for conversion, parsing, entry generation |

### Output files generated at runtime

| File | Path | Description |
| --- | --- | --- |
| Updated WEL | output/modflow/{year}/thruCY2165_{year}.wel | WEL file with target year’s pumping data |
| NAM file | output/modflow/{year}/CY{year}.nam | MODFLOW name file for target year |
| Baseline copies | output/modflow/{year}/*.{bcf,sip,bas,ghb,oc,riv,exe,py} | 10 static MODFLOW support files |

## 7. Acceptance Criteria

# These commands must all pass:
pytest tests/test_update_modflow.py -v --tb=short
ruff check step2_update_modflow.py
mypy step2_update_modflow.py

# End-to-end run (requires step1 output):
python3 step2_update_modflow.py --year 2024

Expected output: - thruCY2165_2024.wel with same total line count as input - CY2024.nam with correct file references - 10 baseline files copied to output/modflow/2024/ - Validation against validation/modflow/2024/ (if validation files exist): - Pre-2024 section: byte-identical - 2024 pumping rates: within 0.0001 ft3/s tolerance - Post-2024 section: byte-identical

## 8. Known Gotchas

- ☐ Well 3 maps to “BUCKMAN 3A” (not “BUCKMAN 3”). The WELL_NAME_MAP handles this: WELL_NAME_MAP[3] == "BUCKMAN 3A".

- ☐ Line format uses fixed-width fields: {layer:10d}{row:10d}{col:10d}  {rate:8.5f}  {well_name} {month} {year}. Two spaces separate col from rate, and rate from well_name.

- ☐ Zero pumping must be formatted as -0.00000 (negative zero), not 0.00000 or -0.0. Use rate = -0.0 when abs(rate) < 1e-10.

- ☐ CRLF line endings (\r\n) are critical. MODFLOW96 is a DOS/Windows program. Use newline="" when writing to prevent Python from adding extra CR.

- ☐ The WEL file spans 1947-2165. The target year section is a small portion in the middle. Pre-target and post-target sections must be preserved byte-for-byte.

- ☐ Multiple wells share grid cells (e.g., BUCKMAN 1, 7, 8 all at row=13, col=11). This is correct – different physical wells in the same MODFLOW cell.

- ☐ Leap year handling is automatic via calendar.isleap(). February 2024 has 29 days, February 2025 has 28 days. This affects the ft3/s rate conversion.

- ☐ Rate tolerance for validation is 0.0001 ft3/s (not 0.00002 as originally stated in PRD). Actual differences can reach 0.00008 due to rounding between CSV precision and validation file.

- ☐ The find_year_boundaries() function searches for the first occurrence of “JAN {year}” and the last occurrence of “DEC {year}”. The WEL file may contain these strings in well comments, so the search must find the correct structural boundaries.

- ☐ NAM file comment lines start with # and are ignored during validation comparison. Only non-comment lines must match.

## 9. Year-Chaining Behavior

| Data Element | Source for Year N (2024) | Source for Year N+1 (2025) |
| --- | --- | --- |
| Input WEL file | input/modflow/2023/thruCY2165.wel (baseline) | output/modflow/2024/thruCY2165_2024.wel (prior year output) |
| Input NAM file | input/modflow/2023/CY2023.nam (baseline) | output/modflow/2024/CY2024.nam (prior year output) |
| Table 2 CSV | output/ingested_data/2024_Table_2_output.csv | output/ingested_data/2025_Table_2_output.csv |
| Output WEL file | output/modflow/2024/thruCY2165_2024.wel | output/modflow/2025/thruCY2165_2025.wel |
| Baseline files | Copied from input/modflow/2023/ | Copied from input/modflow/2023/ (always same source) |

Critical chaining logic: - BASELINE_YEAR = 2024 uses original 2023 files from input/modflow/2023/ - Years > 2024 use prior year’s output: output/modflow/{year-1}/thruCY2165_{year-1}.wel - The WEL file grows cumulatively: 2024 output contains actual pumping for 2024 (replacing placeholder data), and 2025 output contains actual data for both 2024 and 2025 - Baseline support files (BCF, SIP, BAS, etc.) are always copied fresh from the 2023 directory – they do not change between years

## 10. Verification

Single command to confirm the module works end-to-end:

python3 step2_update_modflow.py --year 2024 && \
  python3 -c "
from pathlib import Path
wel = Path('output/modflow/2024/thruCY2165_2024.wel')
nam = Path('output/modflow/2024/CY2024.nam')
assert wel.exists(), 'WEL file not created'
assert nam.exists(), 'NAM file not created'
lines = wel.read_text().splitlines()
jan_lines = [l for l in lines if 'BUCKMAN 1 JAN 2024' in l]
assert len(jan_lines) == 2, f'Expected 2 BUCKMAN 1 JAN lines (L1+L2), got {len(jan_lines)}'
rate = float(jan_lines[0].split()[3])
assert abs(rate - (-0.13733)) < 0.001, f'Rate mismatch: {rate}'
print(f'WEL: {len(lines)} lines, BUCKMAN 1 JAN rate: {rate:.5f} ft3/s')
baseline_count = sum(1 for f in Path('output/modflow/2024').iterdir() if f.is_file())
print(f'Files in output dir: {baseline_count}')
print('PASS')
"

Expected result: WEL: XXXXX lines, BUCKMAN 1 JAN rate: -0.13733 ft3/s, Files in output dir: 12, PASS

## 11. Cross-References

| Related Spec | Relationship |
| --- | --- |
| DS-02 | AF-to-ft3/s conversion, MODFLOW pumping conventions |
| DS-03 | MODFLOW grid cell assignments (WELL_GRID_MAP) |
| IS-01 | Constants (WELL_GRID_MAP, WELL_NAME_MAP, ACRE_FT_TO_FT3, NUM_LAYERS, BASELINE_FILES_TO_COPY) |
| IS-02 | Table 2 CSV output consumed as input |
| IS-05 | MODFLOW execution reads the WEL and NAM files produced here |
| IS-06 | Stream depletion post-processing depends on MODFLOW run completing successfully |

# IS-05: MODFLOW Execution Wrapper

Tier 2 Implementation Specification — A complete Claude Code session-sized prompt. Paste into a fresh session with Tier 1 domain docs available to get a working module back. Sequentially ordered with explicit dependencies.

## 1. Session Goal

Implement step3_run_modflow.sh — a bash script that wraps MODFLOW96 execution via Wine with enhanced logging, timing, and automatic post-run verification.

## 2. Prerequisites

### Prior Specs (must be complete)

- IS-04: MODFLOW Input Update (step2_update_modflow.py must have generated output/modflow/{year}/CY{year}.nam and all referenced input files)

### Input Files (must exist)

| File | Path | Description |
| --- | --- | --- |
| NAM file | output/modflow/{year}/CY{year}.nam | MODFLOW name file listing all input/output file paths |
| MODFLOW96 exe | output/modflow/{year}/modflow96.exe | MODFLOW96 executable (Win32, runs via Wine) |
| Verification script | output/modflow/{year}/verify_modflow_run.py | Post-run convergence/output checker (optional) |

### Domain Knowledge

- See DS-03 for MODFLOW model structure and stress period configuration

- MODFLOW96 is a legacy Fortran groundwater model (McDonald & Harbaugh, 1988)

- Runs under Wine on Linux/WSL because it is a 32-bit Windows executable

## 3. Context for Claude Code

MODFLOW96 is a finite-difference groundwater flow simulator. It reads a “name file” (.nam) that lists all input packages (well pumping, river boundaries, GHB boundaries, etc.) and output file paths. The model solves the groundwater flow equation iteratively for each stress period (month).

For the Buckman model: - 50 stress periods = 50 months of simulation (varying by year configuration) - Runtime: 30-45 minutes on typical hardware - Input method: MODFLOW96 reads the NAM filename from stdin (not command-line argument) - Output: Binary flux files (CY{year}_riv.flx, CY{year}_ghb.flx) consumed by the FORTRAN post-processor in Step 4

### Key Equations (Inline)

Runtime reporting:
  RUNTIME = END_TIME - START_TIME (epoch seconds)
  MINUTES = RUNTIME / 60
  SECONDS = RUNTIME % 60

### Key Constants (Inline)

| Constant | Value | Units |
| --- | --- | --- |
| Typical runtime | 30-45 | minutes |
| Exit code success | 0 | - |
| NAM file pattern | CY{YEAR}.nam | - |
| Log file pattern | {YEAR}_modflow_run.log | - |

## 4. Requirements (Numbered, Testable)

| Req | Description | Acceptance Criterion |
| --- | --- | --- |
| R1 | Parse --year YYYY argument (required), validate is numeric | ./step3_run_modflow.sh without --year prints usage and exits 1; ./step3_run_modflow.sh --year abc prints error about non-numeric value and exits 1 |
| R2 | Support -h/--help flag showing usage, description, and what the script does | ./step3_run_modflow.sh --help prints help text and exits 0 |
| R3 | Check prerequisite: output/modflow/{year}/ directory exists | Missing directory prints error with hint to run step2 first, exits 1 |
| R4 | Check prerequisite: Wine is installed (command -v wine) | Missing Wine prints install instructions, exits 1 |
| R5 | Check prerequisite: CY{year}.nam file exists in MODFLOW directory | Missing NAM file prints error with expected location, exits 1 |
| R6 | Execute MODFLOW96: echo "CY{year}.nam" \¦ wine ./modflow96.exe piping NAM filename to stdin | MODFLOW96 receives filename and begins solving |
| R7 | Capture all output to {year}_modflow_run.log using tee (stdout visible in real-time AND logged) | Log file created with MODFLOW output; user sees convergence info in terminal |
| R8 | Track runtime using epoch seconds (date +%s before and after), report minutes/seconds | Summary shows runtime in Xm Ys format |
| R9 | Check exit code via ${PIPESTATUS[1]} (Wine/MODFLOW exit code from 3-command pipe). Known Bug: Both this spec and step3_run_modflow.sh (line 117) incorrectly use PIPESTATUS[0], which captures echo’s exit code (always 0) instead of wine’s exit code. See Section 8 warning for details. | Non-zero exit code from MODFLOW reported as failure; script exits with that code |
| R10 | Auto-run verify_modflow_run.py if present; skip with warning if absent | Verification runs after successful MODFLOW completion; missing script produces warning, not error |
| R11 | Report next step: python3 step4_generate_depletion_tables.py --year {YEAR} | Success message includes exact command for next pipeline step |
| R12 | Use set -euo pipefail for strict error handling | Script fails fast on undefined variables, command errors, or pipe failures |

## 5. Worked Example

### Input

./step3_run_modflow.sh --year 2025

Working directory: project root (/home/user/projects/rg/santafe/buckman/)

### Execution Flow

Step 1: Parse arguments
  --year 2025 → YEAR="2025"
  Validate: "2025" =~ ^[0-9]+$ → passes

Step 2: Check prerequisites
  [[ -d "output/modflow/2025" ]] → true
  command -v wine → /usr/bin/wine (exists)
  cd output/modflow/2025/
  [[ -f "CY2025.nam" ]] → true

Step 3: Print banner
  "MODFLOW96 Execution - Year 2025"
  "Started: Fri Feb 20 10:30:00 MST 2026"

Step 4: Run MODFLOW96
  START_TIME=$(date +%s) → 1740069000
  echo "CY2025.nam" | wine ./modflow96.exe 2>&1 | tee "2025_modflow_run.log"
  EXIT_CODE=${PIPESTATUS[0]} → 0 (BUG: captures echo's exit code, not wine's; see Section 8)
  END_TIME=$(date +%s) → 1740071400
  RUNTIME = 1740071400 - 1740069000 = 2400 seconds

Step 5: Report runtime
  2400 / 60 = 40 minutes
  2400 % 60 = 0 seconds
  "Runtime: 40 minutes 0 seconds"

Step 6: Check exit code
  EXIT_CODE=0 → "MODFLOW96 completed successfully"

Step 7: Run verification
  python3 verify_modflow_run.py → exit 0
  "All checks passed!"

Step 8: Report next step
  "cd ../../.."
  "python3 step4_generate_depletion_tables.py --year 2025"

### Expected Output

==============================================================
MODFLOW96 Execution - Year 2025
Started: Fri Feb 20 10:30:00 MST 2026
==============================================================

Working directory: /home/user/projects/rg/santafe/buckman/output/modflow/2025

NAM file: CY2025.nam
Executable: modflow96.exe

Running MODFLOW96 (this will take 30-45 minutes)...
You can watch convergence behavior in real-time.
Press Ctrl+C to abort if you see problems.

--------------------------------------------------------------
[... less than one minute of MODFLOW convergence output ...]
--------------------------------------------------------------

==============================================================
MODFLOW96 Execution Complete
Runtime: 40 minutes 0 seconds
Exit code: 0
==============================================================

MODFLOW96 completed successfully

Running post-execution verification...
[... verification output ...]

==============================================================
SUMMARY
==============================================================
MODFLOW runtime: 40m 0s
Log file: 2025_modflow_run.log

All checks passed!

Next Step:
  cd ../../..
  python3 step4_generate_depletion_tables.py --year 2025

## 6. Files to Create/Modify

| Action | Path | Description |
| --- | --- | --- |
| Create | step3_run_modflow.sh | Bash script wrapping MODFLOW96 execution via Wine |

## 7. Acceptance Criteria

# These commands must all pass:

# 1. Script is executable
test -x step3_run_modflow.sh

# 2. Help flag works
./step3_run_modflow.sh --help

# 3. Missing --year produces error
./step3_run_modflow.sh 2>&1 | grep -q "ERROR.*--year is required"

# 4. Non-numeric year produces error
./step3_run_modflow.sh --year abc 2>&1 | grep -q "ERROR.*must be a number"

# 5. Unknown option produces error
./step3_run_modflow.sh --foo 2>&1 | grep -q "ERROR.*Unknown option"

# 6. Shellcheck passes (if installed)
shellcheck step3_run_modflow.sh

Expected output: All checks exit 0 (help text displayed, errors captured correctly).

Note: Full execution testing requires Wine + MODFLOW96 + prepared model files, which takes 30-45 minutes. Dry-run testing validates argument parsing and prerequisite checks.

## 8. Known Gotchas

- ☐ WARNING – PIPESTATUS[0] bug (spec and code): The command pipe is echo "$NAM_FILE" | wine ./modflow96.exe 2>&1 | tee .... In this 3-command pipe, PIPESTATUS[0] captures echo’s exit code (always 0), PIPESTATUS[1] captures wine’s exit code (the desired one), and PIPESTATUS[2] captures tee’s exit code. Both this spec and the code (step3_run_modflow.sh line 117) incorrectly use PIPESTATUS[0], meaning MODFLOW failures would never be detected via the PIPESTATUS check. The correct index is PIPESTATUS[1]. However, with set -e and pipefail, the script would still exit on wine failure before reaching the PIPESTATUS line.

- ☐ PIPESTATUS must be read immediately — ${PIPESTATUS[...]} must be captured on the very next line after the pipe. Any intervening command between the pipe and the PIPESTATUS read will overwrite it.

- ☐ set -euo pipefail and pipes — With pipefail, the pipe returns the exit code of the last non-zero command. PIPESTATUS gives element-by-element access: [0] = echo, [1] = wine, [2] = tee. The correct element for Wine’s exit code is PIPESTATUS[1].

- ☐ Wine stderr — Wine may produce its own diagnostic messages on stderr (e.g., “fixme: …” warnings). These are piped via 2>&1 and will appear in the log file. This is intentional for debugging.

- ☐ Working directory — The script cds into output/modflow/{year}/ and stays there. The verify script runs from that directory. The “next step” instructions tell the user to cd ../../.. back to the project root.

- ☐ 30-45 minute runtime — This is a long-running process. The tee ensures the user can watch convergence in real-time. Ctrl+C will abort. There is no timeout; the modeler is expected to monitor convergence.

- ☐ MODFLOW96 stdin — MODFLOW96 reads the NAM filename from stdin, NOT from a command-line argument. This is why we use echo "CY{year}.nam" | wine ./modflow96.exe instead of passing it as an argument.

## 9. Year-Chaining Behavior

| Data Element | Source for Year N | Source for Year N+1 |
| --- | --- | --- |
| NAM file | output/modflow/N/CYN.nam (from step2) | output/modflow/N+1/CY{N+1}.nam (from step2) |
| MODFLOW exe | Same modflow96.exe (copied by step2) | Same modflow96.exe (copied by step2) |
| Output flux files | Generated by this step: CYN_riv.flx, CYN_ghb.flx | Generated by this step: CY{N+1}_riv.flx, CY{N+1}_ghb.flx |

No chaining between years at this step. Each year’s MODFLOW run is fully independent given the input files from step2.

## 10. Verification

Single command to confirm the module works end-to-end:

# Dry-run test (no Wine/MODFLOW needed):
./step3_run_modflow.sh --year 9999 2>&1 | head -20
# Expected: "ERROR: Directory not found: output/modflow/9999"

# Full test (requires Wine + prepared model):
./step3_run_modflow.sh --year 2025
# Expected: 30-45 min runtime, log file created, verification passes

Expected result: Dry-run exits 1 with directory-not-found error. Full run creates {year}_modflow_run.log and reports success.

## 11. Cross-References

| Related Spec | Relationship |
| --- | --- |
| DS-03 | MODFLOW model structure, stress periods, boundary packages |
| IS-04 | Upstream dependency: generates MODFLOW input files and directory structure |
| IS-06 | Downstream dependency: post-processor reads flux files generated by this step |

# IS-06: Post-Processor & Output Parsing

Tier 2 Implementation Specification — A complete Claude Code session-sized prompt. Paste into a fresh session with Tier 1 domain docs available to get a working module back. Sequentially ordered with explicit dependencies.

## 1. Session Goal

Implement step4_generate_depletion_tables.py — the orchestrator script that copies flux files, runs the FORTRAN post-processor (sfmodflx_2245.exe), parses the output, and calls stream_depletions.py to generate Tables 3, 4, and 5.

## 2. Prerequisites

### Prior Specs (must be complete)

- IS-05: MODFLOW Execution (step3_run_modflow.sh must have produced flux files)

- IS-07: Stream Depletion Library (core functions called by this script)

- IS-08: Table 3 Generation (called during table generation phase)

### Input Files (must exist)

| File | Path | Description |
| --- | --- | --- |
| RIV flux file | output/modflow/{year}/[modflow/]CY{year}_riv.flx | MODFLOW river boundary binary flux output (~31 MB) |
| GHB flux file | output/modflow/{year}/[modflow/]CY{year}_ghb.flx | MODFLOW general head boundary binary flux output (~31 MB) |
| Post-processor | output/modflow/{year}/[depletions/]sfmodflx_2245.exe | FORTRAN post-processor executable (Win32) |
| GHB package | input/modflow/2023/thruCY2165.ghb | GHB cell definitions for geometry validation |

### Domain Knowledge

- See DS-03 for MODFLOW model structure

- See docs/MODFLOW_CELL_MAPPING.md for FORTRAN cell extraction logic and La Cienega Springs cell mapping

- The post-processor reads binary flux files and produces a formatted text file with monthly cfs values per cell and per stream

## 3. Context for Claude Code

The MODFLOW post-processor (sfmodflx_2245.exe) is a legacy FORTRAN program that reads binary MODFLOW flux output files and calculates stream depletions. It:

- Reads river (RIV) package flux — contains flows for Rio Pojoaque, Rio Tesuque, and Rio Grande cells

- Reads general head boundary (GHB) package flux — contains flows for La Cienega Springs cells

- Assigns cells to streams using hardcoded row/column ranges (see docs/MODFLOW_CELL_MAPPING.md)

- Outputs a formatted text file (CY{year}) with monthly cfs values for each cell and stream summary rows

### Directory Structure Evolution

The project underwent a directory restructuring between 2024 and 2025:

2024 (Legacy nested structure):
  output/modflow/2024/
    modflow/           ← MODFLOW output (flux files here)
      CY2024_riv.flx
      CY2024_ghb.flx
    depletions/        ← Post-processor working directory
      sfmodflx_2245.exe
      CY2024             ← Post-processor output

2025+ (Flat structure):
  output/modflow/2025/
    CY2025_riv.flx     ← Flux files here
    CY2025_ghb.flx
    sfmodflx_2245.exe   ← Post-processor here too
    CY2025              ← Output here too

### Key Constants (Inline)

| Constant | Value | Description |
| --- | --- | --- |
| POST_PROCESSOR_EXE | sfmodflx_2245.exe | FORTRAN post-processor executable name |
| OUTPUT_DIR | ./output/depletion/ | Directory for generated XLSX tables |
| VALIDATION_DIR | ./validation/ | Directory for validation reference files |
| DEFAULT_YEAR | 2024 | Default processing year if not specified |

### Post-Processor Input Protocol

The post-processor reads three lines from stdin:

CY{year}_riv.flx      ← River flux file name
CY{year}_ghb.flx      ← GHB flux file name
CY{year}               ← Output file prefix

### Post-Processor Output Format

The output file CY{year} contains year-blocks like:

1 PUMPAGE EFFECT ON BOUNDARY FLOW (CFS)    CY2024_riv.flx
YEAR:      2024     jan         feb         mar    ...
LAY ROW COL
+_________________________________...
    1   9  14    0.025737    0.025725    0.025738  ...
    1   9  15    0.019555    0.019547    0.019556  ...
    ...
0  R POJOAQUE    0.083581    0.083486    0.083596  ...
0  R TESUQUE     0.078939    0.078843    0.078957  ...
0  RIO GRANDE    0.178098    0.177948    0.178121  ...
0  RIV  TOTAL    0.340618    0.340277    0.340674  ...

[GHB section follows with similar format]
0  LC SPRINGS    0.005137    0.005134    0.005137  ...

## 4. Requirements (Numbered, Testable)

| Req | Description | Acceptance Criterion |
| --- | --- | --- |
| R1 | get_modflow_output_dir(year) — Returns correct directory based on year: 2024 and earlier: output/modflow/{year}/modflow/; 2025+: output/modflow/{year}/ | get_modflow_output_dir(2024) returns ./output/modflow/2024/modflow/; get_modflow_output_dir(2025) returns ./output/modflow/2025/ |
| R2 | get_depletions_dir(year) — Returns post-processor working directory: 2024 and earlier: output/modflow/{year}/depletions/; 2025+: output/modflow/{year}/ | get_depletions_dir(2024) returns ./output/modflow/2024/depletions/; get_depletions_dir(2025) returns ./output/modflow/2025/ |
| R3 | copy_flux_files(year) — Copies CY{year}_riv.flx and CY{year}_ghb.flx from modflow dir to depletions dir; skips copy if source and destination resolve to same directory (flat structure) | For 2024: files copied from modflow/ to depletions/; for 2025+: skip message printed, no copy |
| R4 | run_post_processor(year) — Executes sfmodflx_2245.exe via Wine, pipes stdin with riv file, ghb file, and output prefix; verifies output file created | Post-processor creates CY{year} text file; returns True on success |
| R5 | check_prerequisites(year) — Verifies flux files, depletions directory, and post-processor exe exist before processing | Missing any prerequisite prints forensic error and returns False |
| R6 | check_wine_installed() — Verifies Wine is installed and accessible | Returns True when wine --version succeeds; returns False otherwise |
| R7 | main(year) orchestrates full pipeline: check_prerequisites -> copy_flux_files -> validate GHB geometry -> run_post_processor -> parse output -> extract depletions -> generate Tables 3, 4, 5 -> validate | Returns 0 on success, 1 on any failure |
| R8 | --year YYYY command-line argument with argparse; defaults to DEFAULT_YEAR (2024) | python3 step4_generate_depletion_tables.py --year 2025 processes year 2025 |
| R9 | GHB geometry validation — call sd.validate_ghb_cells_in_fortran_range() before running post-processor to ensure all GHB cells fall within FORTRAN extraction rectangle | Geometry mismatch produces forensic error referencing docs/MODFLOW_CELL_MAPPING.md |
| R10 | Forensic error messages using 5-element format: what failed, location, actual, expected, physical context | All error messages follow print_error() format |

## 5. Worked Example

### Input

python3 step4_generate_depletion_tables.py --year 2024

### Calculation Steps

Step 1: check_prerequisites(2024)
  - MODFLOW dir: ./output/modflow/2024/modflow/ → exists
  - RIV flux: CY2024_riv.flx → exists (31,484,640 bytes)
  - GHB flux: CY2024_ghb.flx → exists (31,484,640 bytes)
  - Depletions dir: ./output/modflow/2024/depletions/ → exists
  - Post-processor: sfmodflx_2245.exe → exists
  Result: True

Step 2: copy_flux_files(2024)
  - Source: ./output/modflow/2024/modflow/
  - Dest: ./output/modflow/2024/depletions/
  - Source != Dest (nested structure) → copy both files
  - CY2024_riv.flx: 31,484,640 bytes -> depletions/
  - CY2024_ghb.flx: 31,484,640 bytes -> depletions/
  Result: True

Step 3: validate_ghb_cells_in_fortran_range()
  - Parse input/modflow/2023/thruCY2165.ghb → 6 cells
  - All cells in rows 30-32, cols 12-15
  - FORTRAN rectangle: rows 28-35, cols 10-20
  - All 6 cells within rectangle → passes

Step 4: run_post_processor(2024)
  - stdin_input = "CY2024_riv.flx\nCY2024_ghb.flx\nCY2024\n"
  - wine ./sfmodflx_2245.exe with cwd=depletions/
  - Output: CY2024 created (text file with depletion data)
  Result: True

Step 5: parse_post_processor_output(year=2024)
  - File: output/modflow/2024/depletions/CY2024
  - Years parsed: 37 (1988-2024)
  - Sample: parsed_data[2024]["R POJOAQUE"]["jan"] = 0.083581

Step 6-13: Generate and write Tables 3, 4, 5 (see IS-07, IS-08)
  - TABLE_3_Rio_Pojoaque_Tesuque_2024.xlsx
  - TABLE_4_Rio_Grande_Otowi_2024.xlsx
  - TABLE_5_La_Cienega_Springs_2024.xlsx

Step 14: Validate against reference files
  - Table 3: OK (all 6 fields within 0.001 AF)
  - Table 4: OK (annual totals within 0.01 AF)
  - Table 5: OK (cumulative within 0.01 AF)
  Overall: OK

### Expected Output

=== Stream Depletion Table Generator for 2024 ===

[prerequisite checklist output]

Copying flux files to post-processor directory for year 2024...
  CY2024_riv.flx: 31,484,640 bytes -> output/modflow/2024/depletions/
  CY2024_ghb.flx: 31,484,640 bytes -> output/modflow/2024/depletions/
Flux files copied successfully.

=== US-001 Complete ===

=== Validating MODFLOW Geometry ===
All 6 GHB cells within FORTRAN rectangle (rows 28-35, cols 10-20)

Running post-processor via Wine for year 2024...
  Wine version: wine-8.0
  Command: wine sfmodflx_2245.exe
  Working directory: output/modflow/2024/depletions/
  Inputs: CY2024_riv.flx, CY2024_ghb.flx, CY2024
Post-processor completed successfully.
  Output file: CY2024 (1,234,567 bytes)

[... parsing, extraction, table generation output ...]

FINAL SUMMARY
Year processed: 2024
Generated files:
  - output/depletion/TABLE_3_Rio_Pojoaque_Tesuque_2024.xlsx
  - output/depletion/TABLE_4_Rio_Grande_Otowi_2024.xlsx
  - output/depletion/TABLE_5_La_Cienega_Springs_2024.xlsx

Validation status: OK
All validations passed!

## 6. Files to Create/Modify

| Action | Path | Description |
| --- | --- | --- |
| Create | step4_generate_depletion_tables.py | Orchestrator: copy flux files, run post-processor, parse output, generate tables |

Dependencies imported at runtime: - stream_depletions (IS-07) — unit conversions, parsing, table generation, validation - shutil — file copying - subprocess — Wine execution - argparse — CLI argument parsing

## 7. Acceptance Criteria

# These commands must all pass:
pytest tests/test_generate_depletion_tables.py -v --tb=short
ruff check step4_generate_depletion_tables.py
mypy step4_generate_depletion_tables.py

# Unit tests for path functions:
python3 -c "
from step4_generate_depletion_tables import get_modflow_output_dir, get_depletions_dir
assert get_modflow_output_dir(2024) == './output/modflow/2024/modflow/'
assert get_modflow_output_dir(2025) == './output/modflow/2025/'
assert get_depletions_dir(2024) == './output/modflow/2024/depletions/'
assert get_depletions_dir(2025) == './output/modflow/2025/'
print('Path function tests passed')
"

# Full end-to-end (requires Wine + flux files):
python3 step4_generate_depletion_tables.py --year 2024
# Expected: 3 XLSX files created, validation passes

Expected output: Ruff/mypy clean; path function tests pass; end-to-end generates three XLSX files with validation status OK.

## 8. Known Gotchas

- ☐ Directory structure varies by year — The 2024/pre-2024 nested structure (modflow/ + depletions/ subdirectories) differs from the 2025+ flat structure (everything in one directory). Both get_modflow_output_dir() and get_depletions_dir() use the year <= 2024 threshold to select the correct path.

- ☐ Same-directory skip for flat structure — When source and destination resolve to the same directory (2025+), copy_flux_files() must skip the copy entirely. It compares source_dir.resolve() == dest_dir.resolve() to handle this correctly.

- ☐ Post-processor stdin protocol — sfmodflx_2245.exe reads three lines from stdin: (1) RIV flux filename, (2) GHB flux filename, (3) output prefix. The \n separators are critical. Using subprocess.run(input=...) with text mode handles this correctly.

- ☐ Post-processor working directory — The FORTRAN executable expects flux files in its working directory (the cwd parameter). It does NOT accept full paths; it opens files by name only.

- ☐ Wine timeout — The post-processor timeout is 300 seconds (5 minutes). This is generous for the small flux files; sfmodflx_2245 typically completes in seconds. If it hangs, it is likely a Wine configuration issue.

- ☐ GHB geometry validation — The geometry check (validate_ghb_cells_in_fortran_range()) runs BEFORE the post-processor to catch cell mismatches early. If the FORTRAN code’s hardcoded rectangle doesn’t contain all GHB cells, depletion values would be silently wrong.

- ☐ Output file has no extension — The post-processor output file is named CY{year} with NO file extension. This is the FORTRAN convention. Python reads it as a text file.

- ☐ Two different parsing functions exist — parse_post_processor_output() (with underscores: post_processor) in step4_generate_depletion_tables.py is used by the step4 pipeline. parse_postprocessor_output() (no underscore: postprocessor) in stream_depletions.py is used by the stream depletion library. These are separate implementations with slightly different regex patterns but produce equivalent results. Be careful not to confuse them when importing or testing.

## 9. Year-Chaining Behavior

| Data Element | Source for Year N | Source for Year N+1 |
| --- | --- | --- |
| Flux files | Generated by step3 for year N | Generated by step3 for year N+1 |
| Post-processor output | CY{N} in depletions dir | CY{N+1} in depletions dir |
| Table 3 historical baseline | validation/2024/expected_outputs/Table_3_expected.xlsx or prior year output | Year N’s TABLE_3_Rio_Pojoaque_Tesuque_N.xlsx |
| Table 5 historical baseline | LA_CIENEGA_CUMULATIVE dict or prior year output | Year N’s TABLE_5_La_Cienega_Springs_N.xlsx |
| Validation files | validation/2024/expected_outputs/Table_3_expected.xlsx + validation/TABLE 4 - Rio Grande...xlsx | Same files (2024-specific); Table 4 validation skipped for non-2024 years |

Key chaining logic: - Table 3: For years < processing_year, values come from the prior year’s output Table 3 (chaining). Falls back to 2024 validation baseline. - Table 5: For years < processing_year, cumulative values come from the prior year’s output Table 5. Falls back to LA_CIENEGA_CUMULATIVE dict.

## 10. Verification

Single command to confirm the module works end-to-end:

python3 step4_generate_depletion_tables.py --year 2024

Expected result: - Three XLSX files in output/depletion/: - TABLE_3_Rio_Pojoaque_Tesuque_2024.xlsx - TABLE_4_Rio_Grande_Otowi_2024.xlsx - TABLE_5_La_Cienega_Springs_2024.xlsx - Validation status: OK (all tables match reference within tolerances) - Exit code: 0

## 11. Cross-References

| Related Spec | Relationship |
| --- | --- |
| DS-03 | MODFLOW model structure, boundary packages |
| IS-05 | Upstream dependency: generates flux files consumed by post-processor |
| IS-07 | Dependency: unit conversions, parsing, table data generation functions |
| IS-08 | Dependency: Table 3 generation logic |
| docs/MODFLOW_CELL_MAPPING.md | FORTRAN cell extraction logic, geometry validation rationale |

# IS-07: Stream Depletion Library

Tier 2 Implementation Specification — A complete Claude Code session-sized prompt. Paste into a fresh session with Tier 1 domain docs available to get a working module back. Sequentially ordered with explicit dependencies.

## 1. Session Goal

Implement stream_depletions.py core functions — unit conversions (cfs to acre-feet), Core (2003) analytical residual lookups, post-processor output parsing, Otowi depletion extraction, and supporting constants.

## 2. Prerequisites

### Prior Specs (must be complete)

- IS-06: Post-Processor & Output Parsing (produces the CY{year} file that this library parses)

### Input Files (must exist)

| File | Path | Description |
| --- | --- | --- |
| Post-processor output | output/modflow/{year}/[depletions/]CY{year} | Text file from sfmodflx_2245.exe with monthly cfs values per cell and stream |

### Domain Knowledge

- See DS-01 for unit conversion derivations and validation

- See DS-02 for Core (2003) analytical model residuals

- See docs/MODFLOW_CELL_MAPPING.md for stream cell identification and FORTRAN extraction logic

## 3. Context for Claude Code

This library is the computational core of the Buckman depletion pipeline. It converts raw MODFLOW post-processor output (monthly cfs flow rates for individual model cells) into acre-feet depletion values for compliance reporting.

### Superposition Principle

MODFLOW calculates depletions using superposition: the difference between a “pumping” scenario and a “no pumping” baseline, both starting in 1988. The residual effects of pre-1988 pumping (1972-1987) are added analytically from Core (2003) lookup tables.

Total Depletion = Analytical Residual (pre-1988) + Superposition 
(1988-present)

### Key Equations (Inline)

Unit conversion (cfs to acre-feet):
  AF = cfs * days * 86400 / 43560
     = cfs * days * 1.983471...

  Where:
    86400 = seconds per day (60 * 60 * 24)
    43560 = cubic feet per acre-foot
    1.983471 = 86400 / 43560 (conversion factor per day)

  Example: 1.0 cfs * 30 days = 1.0 * 30 * 86400 / 43560 = 59.5041... AF

Annual total from monthly values:
  Annual_AF = SUM(cfs[i] * days[i] * 86400/43560) for i=0..11

### Tesuque Residual Coverage

CORE_2003_TESUQUE contains explicit values for all years 1988-2050 (63 entries). No extrapolation formula is needed. For years beyond 2050, get_analytical_residual() uses .get(year, 0.0) to return 0.0, representing exhaustion of the pre-1988 pumping effect.

Example: year 2035 → CORE_2003_TESUQUE[2035] = 7.479 AF (dict lookup)
Example: year 2050 → CORE_2003_TESUQUE[2050] = 0.117 AF (dict lookup)
Example: year 2060 → CORE_2003_TESUQUE.get(2060, 0.0) = 0.0 AF (beyond table)

### Key Constants (Inline)

| Constant | Value | Units |
| --- | --- | --- |
| Seconds per day | 86,400 | s/day |
| Cubic feet per acre-foot | 43,560 | ft^3/AF |
| Conversion factor | 1.983471… | AF/(cfs*day) |
| Days (non-leap Feb) | 28 | days |
| Days (leap Feb) | 29 | days |
| Total days (non-leap) | 365 | days |
| Total days (leap) | 366 | days |

## 4. Requirements (Numbered, Testable)

| Req | Description | Acceptance Criterion |
| --- | --- | --- |
| R1 | cfs_to_acre_feet(cfs, days) — Convert cfs flow rate to acre-feet for a period of days | cfs_to_acre_feet(1.0, 30) = 59.504132… AF; raises ValueError for negative cfs or days < 1 |
| R2 | cfs_to_af(cfs_value, month_index, year, use_leap_year) — Month-specific conversion using appropriate days | cfs_to_af(0.1, 0) = 6.148760… AF (0.1 cfs * 31 Jan days); cfs_to_af(0.1, 1, use_leap_year=True) uses 29 days |
| R3 | cfs_monthly_to_af_annual(cfs_list, year, use_leap_year) — Sum 12 monthly cfs values to annual AF total | cfs_monthly_to_af_annual([0.1]*12) = 72.397… AF (non-leap); cfs_monthly_to_af_annual([0.1]*12, use_leap_year=True) = 72.596… AF (leap) |
| R4 | get_analytical_residual(stream, year) — Lookup Core (2003) residual for Pojoaque or Tesuque | get_analytical_residual("pojoaque", 2024) = 0.0; get_analytical_residual("tesuque", 2024) = 12.877; get_analytical_residual("tesuque", 2035) = 7.479 (dict lookup); get_analytical_residual("tesuque", 2060) = 0.0 (beyond table) |
| R5 | parse_postprocessor_output(file_path) — Parse CY{year} output file into nested dict {year: {identifier: {month: cfs}}} | Correctly parses year headers, cell data rows, and stream summary rows; normalizes “RIV TOTAL” to “RIV TOTAL” |
| R6 | extract_otowi_depletions(parsed_data, year) — Sum model cells above and below Otowi Gage | Returns (above_cfs, below_cfs) each as list of 12 monthly values; raises KeyError for missing cells |
| R7 | ABOVE_OTOWI_CELLS and BELOW_OTOWI_CELLS — Coordinate lists for Otowi classification | 10 cells above Otowi, 16 cells below Otowi; coordinates match MODFLOW model grid |
| R8 | LA_CIENEGA_CUMULATIVE — Historical cumulative depletion dict (2004-2030) | LA_CIENEGA_CUMULATIVE[2024] = 3.74; LA_CIENEGA_CUMULATIVE[2004] = 0.45 |
| R9 | DAYS_VALIDATION — Non-leap year days list; DAYS_2024 — Leap year days list | DAYS_VALIDATION[1] = 28 (Feb non-leap); DAYS_2024[1] = 29 (Feb leap) |
| R10 | CORE_2003_POJOAQUE — Analytical residual lookup dict for Pojoaque (1988-2015) | Returns 0 after 2015 (residual exhausted); 40.432 for 1988 |
| R11 | CORE_2003_TESUQUE — Analytical residual lookup dict for Tesuque (1988-2050, 63 entries) | 21.015 for 1988; 9.933 for 2030; 0.117 for 2050; .get(year, 0.0) returns 0.0 for years beyond 2050 |

## 5. Worked Example

### Example 1: Unit Conversion — cfs to acre-feet

Input: cfs=0.083581, days=31 (January)

Calculation:
  AF = 0.083581 * 31 * 86400 / 43560
     = 0.083581 * 31 * 1.983471
     = 0.083581 * 61.4876
     = 5.13857... AF

Verification:
  0.083581 * 31 = 2.590011 (cfs-days)
  2.590011 * 86400 = 223,777,000 (ft^3)
  223,777,000 / 43560 = 5138.57... wait, this is wrong

Correct: 0.083581 * 31 * 86400 / 43560
  = 0.083581 * 31 * 1.9835
  = 0.083581 * 61.4885
  = 5.1393 AF (approximately)

### Example 2: Annual Total from Monthly CFS

Input: R POJOAQUE 2024 monthly cfs (from post-processor):
  jan=0.083581, feb=0.083486, mar=0.083596, apr=0.083635,
  may=0.083692, jun=0.083753, jul=0.083791, aug=0.083829,
  sep=0.083862, oct=0.083886, nov=0.083905, dec=0.083916

Days (leap year 2024): [31, 29, 31, 30, 31, 30, 31, 31, 30, 31, 30, 31]

Step-by-step:
  Jan: 0.083581 * 31 * 86400/43560 = 5.139 AF
  Feb: 0.083486 * 29 * 86400/43560 = 4.801 AF
  Mar: 0.083596 * 31 * 86400/43560 = 5.140 AF
  ...
  Dec: 0.083916 * 31 * 86400/43560 = 5.160 AF

  Annual total = SUM = ~61.05 AF (approximate; actual depends on precise values)

### Example 3: Analytical Residual Lookup

Input: stream="tesuque", year=2024
  CORE_2003_TESUQUE[2024] = 12.877 AF

Input: stream="pojoaque", year=2024
  CORE_2003_POJOAQUE.get(2024, 0.0) → 0.0 (not in dict, exhausted after 2015)

Input: stream="tesuque", year=2035 (in lookup table)
  CORE_2003_TESUQUE[2035] = 7.479 AF (direct dict lookup)

Input: stream="tesuque", year=2050 (last entry in lookup table)
  CORE_2003_TESUQUE[2050] = 0.117 AF (direct dict lookup)

Input: stream="tesuque", year=2060 (beyond lookup table)
  CORE_2003_TESUQUE.get(2060, 0.0) = 0.0 AF (not in dict, residual exhausted)

### Example 4: Post-Processor Output Parsing

Input file fragment (CY2024):
  "YEAR:      2024     jan         feb     ..."
  "    1   9  14    0.025737    0.025725    0.025738 ..."
  "0  R POJOAQUE    0.083581    0.083486    0.083596 ..."
  "0  RIV  TOTAL    0.340618    0.340277    0.340674 ..."

Parsing results:
  Year header regex: r"YEAR:\s+(\d{4})\s+jan" → matches "2024"
  Cell regex: r"\s+(\d+)\s+(\d+)\s+(\d+)\s+([\d.]+(?:\s+[\d.]+){11})" → (1, 9, 14) + 12 values
  Stream regex: r"0\s+(R POJOAQUE|R TESUQUE|RIO GRANDE|RIV\s+TOTAL|LC SPRINGS)\s+(12 floats)"

  "RIV  TOTAL" → normalized via re.sub(r"\s+", " ", ...) → "RIV TOTAL"

Output:
  parsed_data[2024]["1 9 14"]["jan"] = 0.025737
  parsed_data[2024]["R POJOAQUE"]["jan"] = 0.083581
  parsed_data[2024]["RIV TOTAL"]["jan"] = 0.340618

### Example 5: Otowi Depletion Extraction

ABOVE_OTOWI_CELLS = [
  (1,1,16), (1,2,16), (1,3,16), (1,4,16), (1,5,15),
  (1,6,14), (1,7,14), (1,8,13), (1,9,13), (1,10,12)
]

For month "jan", year 2024:
  above_jan = sum of parsed_data[2024]["1 1 16"]["jan"] +
              parsed_data[2024]["1 2 16"]["jan"] + ... +
              parsed_data[2024]["1 10 12"]["jan"]

BELOW_OTOWI_CELLS = [
  (1,11,11), (1,12,11), (1,13,11), (1,14,10), (1,15,9),
  (1,15,10), (1,16,9), (1,17,8), (1,18,6), (1,18,7),
  (1,19,6), (1,20,5), (1,21,4), (1,21,5), (1,22,4), (1,23,3)
]

For month "jan", year 2024:
  below_jan = sum of 16 cells

Returns: ([above_jan, ..., above_dec], [below_jan, ..., below_dec])

## 6. Files to Create/Modify

| Action | Path | Description |
| --- | --- | --- |
| Create | stream_depletions.py | Core library: constants, unit conversions, residual lookups, parsing, Otowi extraction |

## 7. Acceptance Criteria

# These commands must all pass:
pytest tests/test_stream_depletions.py -v --tb=short -k "cfs_to or residual or parse or otowi"
ruff check stream_depletions.py
mypy stream_depletions.py

Key test cases:

# R1: cfs_to_acre_feet
assert abs(cfs_to_acre_feet(1.0, 30) - 59.50413223) < 0.001
assert abs(cfs_to_acre_feet(0.0, 30) - 0.0) < 0.001
# ValueError for negative cfs
# ValueError for days < 1

# R2: cfs_to_af (month-specific)
assert abs(cfs_to_af(0.1, 0) - 6.14885...) < 0.001  # Jan, 31 days, non-leap
assert abs(cfs_to_af(0.1, 1, use_leap_year=True) - 5.75207...) < 0.001  # Feb, 29 days

# R3: cfs_monthly_to_af_annual
assert abs(cfs_monthly_to_af_annual([0.1]*12) - 72.397...) < 0.01  # non-leap
assert abs(cfs_monthly_to_af_annual([0.1]*12, use_leap_year=True) - 72.596...) < 0.01  # leap

# R4: get_analytical_residual
assert get_analytical_residual("pojoaque", 1988) == 40.432
assert get_analytical_residual("pojoaque", 2024) == 0.0  # exhausted
assert get_analytical_residual("tesuque", 2024) == 12.877
assert get_analytical_residual("tesuque", 2035) == 7.479  # dict lookup

# R8: LA_CIENEGA_CUMULATIVE
assert LA_CIENEGA_CUMULATIVE[2024] == 3.74
assert LA_CIENEGA_CUMULATIVE[2004] == 0.45
assert LA_CIENEGA_CUMULATIVE[2030] == 4.80

## 8. Known Gotchas

- ☐ Two days lists serve different purposes — DAYS_VALIDATION (28 for Feb, 365 total) is used by Table 4, which matches the validation file’s non-leap assumption. DAYS_2024 (29 for Feb, 366 total) is used by Table 3, which uses actual calendar days. The use_leap_year parameter selects between them. For future years, use calendar.isleap(year) to determine the correct list.

- ☐ Cell key format — Parsed cell identifiers use space-separated coordinates: "1 9 14" (layer, row, col). This matches the post-processor output format. When constructing keys for Otowi extraction, use f-strings: f"{lay} {row} {col}".

- ☐ Stream name normalization — The FORTRAN post-processor writes “RIV TOTAL” with two spaces. The parser normalizes this to “RIV TOTAL” with a single space using re.sub(r"\s+", " ", stream_name). All downstream code uses the normalized form.

- ☐ Pojoaque residual exhaustion — CORE_2003_POJOAQUE has no entry for years after 2015. The get_analytical_residual() function uses .get(year, 0.0) to return 0 for these years, representing the exhaustion of the pre-1988 pumping effect.

- ☐ Tesuque residual exhaustion — CORE_2003_TESUQUE has explicit values through 2050 (63 entries, 1988-2050). The residual reaches 0.117 AF in 2050 and is 0.0 for years beyond 2050 via .get(year, 0.0). No extrapolation formula is used.

- ☐ Post-processor output has no file extension — The file is named CY2024, not CY2024.txt. Python reads it as a text file with path.read_text() or open(file_path).

- ☐ Year header regex — The regex r"YEAR:\s+(\d{4})\s+jan" requires “jan” after the year number to distinguish year headers from other lines that might contain “YEAR:”. This is intentional.

- ☐ Stream regex is explicitly enumerated — The stream match regex uses (R POJOAQUE|R TESUQUE|RIO GRANDE|RIV\s+TOTAL|LC SPRINGS) rather than a generic pattern. This prevents false matches on unexpected lines and makes the expected stream names explicit in the code.

## 9. Year-Chaining Behavior

| Data Element | Source for Year N | Source for Year N+1 |
| --- | --- | --- |
| CORE_2003_POJOAQUE | Static lookup dict | Same dict (returns 0 after 2015) |
| CORE_2003_TESUQUE | Static lookup dict (1988-2050) | Same dict (returns 0.0 after 2050) |
| LA_CIENEGA_CUMULATIVE | Static lookup dict | Same dict (extends to 2030) |
| DAYS_VALIDATION / DAYS_2024 | Static constants | Use calendar.isleap(year) for dynamic selection |
| Post-processor output | Parsed fresh each year | Parsed fresh each year |

No chaining at this level. All functions are stateless and operate on the data passed to them. Year-chaining for historical preservation happens at the Table writing layer (IS-08 for Table 3, IS-09 for Table 5).

## 10. Verification

Single command to confirm the module works end-to-end:

python3 -c "
from stream_depletions import (
    cfs_to_acre_feet, cfs_to_af, cfs_monthly_to_af_annual,
    get_analytical_residual, ABOVE_OTOWI_CELLS, BELOW_OTOWI_CELLS,
    LA_CIENEGA_CUMULATIVE, CORE_2003_POJOAQUE, CORE_2003_TESUQUE,
    DAYS_VALIDATION, DAYS_2024
)

# Unit conversion
assert abs(cfs_to_acre_feet(1.0, 30) - 59.5041) < 0.01
assert abs(cfs_monthly_to_af_annual([0.1]*12) - 72.3978) < 0.01

# Residuals
assert get_analytical_residual('pojoaque', 2024) == 0.0
assert get_analytical_residual('tesuque', 2024) == 12.877
assert get_analytical_residual('pojoaque', 1988) == 40.432

# Constants
assert len(ABOVE_OTOWI_CELLS) == 10
assert len(BELOW_OTOWI_CELLS) == 16
assert LA_CIENEGA_CUMULATIVE[2024] == 3.74
assert DAYS_VALIDATION[1] == 28
assert DAYS_2024[1] == 29

print('All stream_depletions.py verification checks passed')
"

Expected result: All assertions pass, “All stream_depletions.py verification checks passed” printed.

## 11. Cross-References

| Related Spec | Relationship |
| --- | --- |
| DS-01 | Unit conversion derivations, validation values |
| DS-02 | Core (2003) analytical model residual tables |
| DS-03 | MODFLOW model grid, cell coordinates, boundary packages |
| IS-06 | Upstream dependency: produces CY{year} output file parsed by this library |
| IS-08 | Downstream dependency: Table 3 generation uses residuals and unit conversions |
| docs/MODFLOW_CELL_MAPPING.md | Stream cell identification, FORTRAN extraction ranges |

# IS-08: Table 3 Generation

Tier 2 Implementation Specification — A complete Claude Code session-sized prompt. Paste into a fresh session with Tier 1 domain docs available to get a working module back. Sequentially ordered with explicit dependencies.

## 1. Session Goal

Implement Table 3 generation — combining MODFLOW superposition results with Core (2003) analytical residuals for Rio Pojoaque-Nambe and Rio Tesuque, preserving historical years via chaining from prior year’s output, and exporting to a formatted XLSX file.

## 2. Prerequisites

### Prior Specs (must be complete)

- IS-06: Post-Processor & Output Parsing (produces parsed post-processor output)

- IS-07: Stream Depletion Library (provides cfs_to_acre_feet(), cfs_to_af(), cfs_monthly_to_af_annual(), get_analytical_residual(), and the CORE_2003_POJOAQUE/CORE_2003_TESUQUE lookup tables)

### Input Files (must exist)

| File | Path | Description |
| --- | --- | --- |
| Post-processor output | output/modflow/{year}/[depletions/]CY{year} | Parsed text with monthly cfs values per stream |
| Prior year’s Table 3 | output/depletion/TABLE_3_Rio_Pojoaque_Tesuque_{year-1}.xlsx | Historical depletion values for chaining |
| Validation baseline | validation/2024/expected_outputs/Table_3_expected.xlsx | Fallback if prior year’s Table 3 not found |

### Domain Knowledge

- See DS-03 §2 for the superposition principle and analytical residual science

- See DS-04 §2 for year-chaining architecture and historical preservation rules

- See DS-03 §5 for the cfs-to-AF conversion factor derivation

## 3. Context for Claude Code

Table 3 reports annual stream depletion impacts to two tributaries: Rio Pojoaque-Nambe and Rio Tesuque. For each stream, the total impact is the sum of two components:

### Superposition Principle

Total_Impact_AF = Residual_AF + Superposition_AF

Where:
  Residual_AF     = Core (2003) analytical model value for pre-1988 pumping 
effects
  Superposition_AF = MODFLOW-calculated depletion from 1988-{year} pumping

- Residual Impact (Analytical): Pre-1988 pumping effects still propagating through the aquifer. Values decrease over time. Rio Pojoaque residuals reached zero by 2016; Rio Tesuque residuals continue through 2050 (0.117 AF), then 0.0.

- Superposition Impact: MODFLOW model calculates depletions from all pumping since 1988. Monthly cfs values are converted to annual AF.

### Historical Preservation (Year Chaining)

Years before the current processing year use values preserved verbatim from the prior year’s Table 3 output. Only the current year and future projections are recalculated from the current MODFLOW run. This ensures historical values remain stable across annual updates.

Chaining resolution order:

1. Prior year’s output: output/depletion/TABLE_3_Rio_Pojoaque_Tesuque_{year-1}.xlsx

2. Fallback: validation/2024/expected_outputs/Table_3_expected.xlsx

### Key Equations (Inline)

cfs to AF conversion:

AF = cfs × days × 86400 / 43560

Where:
  cfs   = cubic feet per second (monthly average from post-processor)
  days  = days in the month (accounts for leap years)
  86400 = seconds per day
  43560 = cubic feet per acre-foot

Simplified: AF = cfs × days × 1.9835

Monthly-to-annual aggregation:

Annual_AF = SUM(cfs_month[i] × days[i] × 1.9835)  for i = Jan..Dec

### Key Constants (Inline)

| Constant | Value | Units |
| --- | --- | --- |
| CFS_TO_AF factor | 86400/43560 = 1.9835 | AF/(cfs·day) |
| Year range | 1988–2050 | inclusive (Tesuque residuals extend through 2050) |
| Number format | 0.000 | 3 decimal places for AF |

### Core (2003) Analytical Residuals

Rio Pojoaque-Nambe (decreasing, exhausted after 2015):

CORE_2003_POJOAQUE = {
    1988: 40.432, 1989: 39.244, 1990: 37.971, 1991: 36.557, 1992: 34.928,
    1993: 33.112, 1994: 31.185, 1995: 29.226, 1996: 27.296, 1997: 25.439,
    1998: 23.678, 1999: 22.028, 2000: 20.491, 2001: 19.068, 2002: 17.753,
    2003: 16.543, 2004: 15.429, 2005: 14.404, 2006: 13.462, 2007: 12.595,
    2008: 11.797, 2009: 11.061, 2010: 10.383, 2011: 6.151, 2012: 4.693,
    2013: 3.234, 2014: 1.775, 2015: 0.316,
    # 2016+: 0 (residual effect exhausted)
}

Rio Tesuque (longer-lasting, continues through 2050; 63 entries):

CORE_2003_TESUQUE = {
    1988: 21.015, 1989: 22.333, 1990: 23.391, 1991: 24.227, 1992: 24.868,
    1993: 25.327, 1994: 25.615, 1995: 25.747, 1996: 25.737, 1997: 25.608,
    1998: 25.378, 1999: 25.067, 2000: 24.691, 2001: 24.265, 2002: 23.800,
    2003: 23.308, 2004: 22.797, 2005: 22.273, 2006: 21.743, 2007: 21.212,
    2008: 20.683, 2009: 20.157, 2010: 19.639, 2011: 19.258, 2012: 18.767,
    2013: 18.276, 2014: 17.785, 2015: 17.295, 2016: 16.804, 2017: 16.313,
    2018: 15.822, 2019: 15.331, 2020: 14.841, 2021: 14.350, 2022: 13.859,
    2023: 13.368, 2024: 12.877, 2025: 12.387, 2026: 11.896, 2027: 11.405,
    2028: 10.914, 2029: 10.424, 2030: 9.933, 2031: 9.442, 2032: 8.951,
    2033: 8.460, 2034: 7.970, 2035: 7.479, 2036: 6.988, 2037: 6.497,
    2038: 6.006, 2039: 5.516, 2040: 5.025, 2041: 4.534, 2042: 4.043,
    2043: 3.552, 2044: 3.062, 2045: 2.571, 2046: 2.080, 2047: 1.589,
    2048: 1.098, 2049: 0.608, 2050: 0.117,
    # 2051+: 0.0 (residual effect exhausted)
}

## 4. Requirements (Numbered, Testable)

| Req | Description | Acceptance Criterion |
| --- | --- | --- |
| R1 | generate_table3_data() shall accept parsed post-processor output and a year, and return a dict with pojoaque and tesuque sub-dicts each containing residual_af, superposition_af, and total_impact_af | Return structure matches documented schema |
| R2 | Superposition values shall be computed by extracting monthly cfs from R POJOAQUE and R TESUQUE stream labels, then converting to annual AF via cfs_monthly_to_af_annual() | Annual AF matches hand calculation within 0.001 AF |
| R3 | Residual values shall be looked up from CORE_2003_POJOAQUE and CORE_2003_TESUQUE via get_analytical_residual() | Pojoaque returns 0.0 for years > 2015; Tesuque returns dict value through 2050, then 0.0 |
| R4 | Total impact shall equal residual_af + superposition_af exactly (no rounding) | total_impact_af == residual_af + superposition_af |
| R5 | load_historical_table3() shall parse a prior year’s Table 3 XLSX and return a nested dict keyed by year | Returns correct values for years 1988–2023 from baseline |
| R6 | Historical values (years < processing_year) shall be preserved verbatim from prior year’s Table 3 | Byte-level comparison of historical rows |
| R7 | write_table3_xlsx() shall produce a formatted XLSX with merged headers, column structure, and 0.000 number format | File opens in Excel with correct formatting |
| R8 | Leap year handling: February shall use 29 days for leap years (2024, 2028, etc.) and 28 days for non-leap years | Feb cfs→AF conversion uses correct day count |
| R9 | Zero residuals shall render as empty cells (not “0.000”) matching the validation file pattern | Cells for years > 2015 (Pojoaque) show blank |
| R10 | Column headers shall include dynamic year range: "Impact of 1988–{processing_year} Pumping (Superposition)" | Header text matches processing year |

## 5. Worked Example

### Input

Parsed post-processor output for 2025 (hypothetical monthly cfs values for R POJOAQUE):

jan: 0.08, feb: 0.08, mar: 0.08, apr: 0.08, may: 0.08,
jun: 0.08, jul: 0.08, aug: 0.08, sep: 0.08, oct: 0.08,
nov: 0.08, dec: 0.08

### Calculation Steps

Step 1: Convert monthly cfs to AF (2025 is non-leap year, Feb = 28 days)
  Jan: 0.08 * 31 * 86400 / 43560 = 0.08 * 31 * 1.9835 = 4.919 AF
  Feb: 0.08 * 28 * 86400 / 43560 = 0.08 * 28 * 1.9835 = 4.443 AF
  Mar: 0.08 * 31 * 1.9835 = 4.919 AF
  Apr: 0.08 * 30 * 1.9835 = 4.760 AF
  ... (remaining months)

Step 2: Sum monthly AF to annual superposition
  Annual_superposition_AF = SUM(all 12 months) ≈ 57.918 AF

Step 3: Look up analytical residual for Pojoaque 2025
  CORE_2003_POJOAQUE.get(2025, 0.0) = 0.0 (exhausted after 2015)

Step 4: Calculate total impact
  Total = 0.0 + 57.918 = 57.918 AF

Step 5: Look up analytical residual for Tesuque 2025
  CORE_2003_TESUQUE[2025] = 12.387 AF

Step 6: (Repeat steps 1-2 for R TESUQUE cfs values to get Tesuque superposition)

### Expected Output

{
    "pojoaque": {
        "residual_af": 0.0,
        "superposition_af": 57.918,  # from monthly cfs conversion
        "total_impact_af": 57.918
    },
    "tesuque": {
        "residual_af": 12.387,
        "superposition_af": <from MODFLOW>,
        "total_impact_af": 12.387 + <superposition>
    }
}

## 6. Files to Create/Modify

| Action | Path | Description |
| --- | --- | --- |
| Modify | stream_depletions.py | Add/maintain generate_table3_data(), load_historical_table3(), write_table3_xlsx(), print_table3_verification() |
| Modify | step4_generate_depletion_tables.py | Call write_table3_xlsx() with proper chaining arguments |

### Function Inventory

| Function | Lines | Purpose |
| --- | --- | --- |
| load_historical_table3() | ~100 | Parse prior year’s Table 3 XLSX into nested dict |
| generate_table3_data() | ~90 | Combine residuals + superposition for one year |
| print_table3_verification() | ~20 | Console output for verification |
| write_table3_xlsx() | ~200 | Full XLSX generation with formatting and chaining |

## 7. Acceptance Criteria

# These commands must all pass:
pytest tests/test_stream_depletions.py -v --tb=short -k "table3"
pytest tests/test_generate_depletion_tables.py -v --tb=short
ruff check stream_depletions.py step4_generate_depletion_tables.py
mypy stream_depletions.py step4_generate_depletion_tables.py

Expected output: - All table3-related tests pass - Table 3 XLSX matches validation baseline within tolerance (±0.001 AF) - No ruff or mypy errors

## 8. Known Gotchas

- ☐ Annotated cell values: Historical Table 3 files may contain annotated strings like "57.182** (57.185)" with asterisk markers. The parser must extract the first numeric value via regex.

- ☐ NaN residuals: In the baseline XLSX, residual cells for years after 2015 (Pojoaque) may be NaN or empty. Treat as 0.0 when loading, and render as empty cells when writing.

- ☐ Leap year awareness: generate_table3_data() uses calendar.isleap(year) to select the correct days-per-month list. The cfs_monthly_to_af_annual() function’s use_leap_year parameter must match.

- ☐ Year range 1988–2050: Table 3 includes projections through 2050 (matching the extent of CORE_2003_TESUQUE), even when processing year is earlier. Future years use MODFLOW projections from the same run. Tesuque residuals reach 0.117 AF in 2050 and are 0.0 beyond.

- ☐ Chaining fallback order: Try prior year’s output first (TABLE_3_..._2024.xlsx), then fall back to validation baseline (Table_3_expected.xlsx). This two-tier fallback is critical for first-time runs.

- ☐ Zero vs empty: Pojoaque residual = 0 renders as an empty cell (no value), not as “0.000”. This matches the validation file’s visual pattern.

- ☐ Dynamic column headers: The “Impact of 1988–{year} Pumping” header must reflect the processing year, not a hardcoded value.

- ☐ Excel styling: Font is Aptos, headers use 12pt bold, data uses 11pt, totals are bold. Number format is 0.000. Borders use medium for headers and hair for data rows.

## 9. Year-Chaining Behavior

| Data Element | Source for Year N | Source for Year N+1 |
| --- | --- | --- |
| Historical rows (1988 to N-1) | Prior year’s Table 3 XLSX (verbatim) | Year N’s Table 3 XLSX (includes N’s fresh row) |
| Current year row (year N) | generate_table3_data() from MODFLOW run | generate_table3_data() from new MODFLOW run |
| Projection rows (N+1 to 2030) | generate_table3_data() from same MODFLOW run | generate_table3_data() from new MODFLOW run |
| Residual values | CORE_2003_POJOAQUE / CORE_2003_TESUQUE dicts | Same dicts (static) |
| Column header year range | “1988–N” | “1988–(N+1)” |

Key principle: Historical rows are never recalculated. They are loaded from the prior year’s output and written verbatim. Only the processing year and future projections use current MODFLOW results.

## 10. Verification

Single command to confirm the module works end-to-end:

python3 -c "
from stream_depletions import parse_postprocessor_output, generate_table3_data, write_table3_xlsx
data = parse_postprocessor_output('output/modflow/2024/modflow/depletions/CY2024')
t3 = generate_table3_data(data, 2024)
print(f'Pojoaque total: {t3[\"pojoaque\"][\"total_impact_af\"]:.3f} AF')
print(f'Tesuque total:  {t3[\"tesuque\"][\"total_impact_af\"]:.3f} AF')
"

Expected result: - Pojoaque total ≈ 60.797 AF (for 2024) - Tesuque total ≈ values matching validation/2024/expected_outputs/Table_3_expected.xlsx

## 11. Cross-References

| Related Spec | Relationship |
| --- | --- |
| DS-03 | Stream depletion science: superposition principle, Core (2003) residuals, cfs→AF conversion |
| DS-04 | Year-chaining architecture: historical preservation rules, chaining resolution order |
| IS-06 | Upstream dependency: provides parsed post-processor output (parse_postprocessor_output()) |
| IS-07 | Library dependency: provides cfs_to_acre_feet(), cfs_to_af(), cfs_monthly_to_af_annual(), get_analytical_residual() |
| IS-09 | Sibling: Tables 4 & 5 follow similar chaining and XLSX export patterns |
| IS-10 | Downstream: test suite includes Table 3 verification tests |

# IS-09: Tables 4 & 5 Generation

Tier 2 Implementation Specification – A complete Claude Code session-sized prompt. Paste into a fresh session with Tier 1 domain docs available to get a working module back. Sequentially ordered with explicit dependencies.

## 1. Session Goal

Implement Table 4 (Rio Grande depletions above/below Otowi Gage) and Table 5 (La Cienega Springs cumulative depletions) data generation and XLSX writing in stream_depletions.py.

## 2. Prerequisites

### Prior Specs (must be complete)

- IS-07: Stream depletion parsing and Table 3 generation (provides parse_postprocessor_output(), cfs_to_af(), cfs_monthly_to_af_annual(), and Otowi cell constants)

- IS-08: Table 3 XLSX writing (provides write_table3_xlsx() and the chaining pattern)

### Input Files (must exist)

| File | Path | Description |
| --- | --- | --- |
| Post-processor output | output/modflow/{year}/depletions/CY{year} | FORTRAN sfmodflx output with cell-level cfs |
| Validation Table 4 | validation/TABLE 4 - Rio Grande, above below Otowi.xlsx | Expected 2024 Table 4 output |
| Validation Table 5 | validation/Table 5 - La Cienega Spring.jpg | Expected 2024 Table 5 image |
| Prior year Table 5 | output/depletion/TABLE_5_La_Cienega_Springs_{year-1}.xlsx | Chaining source for historical years |

### Domain Knowledge

- See DS-03 Section 3 for Otowi cell classification (above/below)

- See DS-03 Section 4 for La Cienega Springs GHB cell mapping

- See DS-02 Section 2 for cfs-to-AF conversion factor derivation

## 3. Context for Claude Code

Table 4 reports Rio Grande stream depletions caused by Buckman wellfield pumping, classified as above or below Otowi Gage. Otowi Gage is the accounting point for New Mexico’s Rio Grande Compact delivery obligations to Texas – depletions below Otowi are chargeable against NM’s allocation.

Table 5 reports cumulative La Cienega Springs depletions since 2004. These are small values (3-5 AF total) representing long-distance aquifer propagation effects. Values are cumulative, meaning each year’s value equals the previous year’s cumulative plus the annual increment.

### Key Equations (Inline)

CFS to AF (monthly):  AF = cfs * days_in_month * 86400 / 43560
                       (1 cfs-day = 1.9835 AF)

Annual AF from monthly cfs:  total_af = SUM over 12 months of (cfs_i * days_i * 1.9835)

Table 5 annual increment:  increment = cumulative(year) - cumulative(year-1)

### Key Constants (Inline)

| Constant | Value | Units |
| --- | --- | --- |
| DAYS_VALIDATION | [31,28,31,30,31,30,31,31,30,31,30,31] | days (non-leap) |
| ABOVE_OTOWI_CELLS | 10 cells: (1,1,16) through (1,10,12) | (layer, row, col) |
| BELOW_OTOWI_CELLS | 16 cells: (1,11,11) through (1,23,3) | (layer, row, col) |
| BUCKMAN_WELLS_CELL | (1, 13, 11) | (layer, row, col) |
| Key counter start | 2089 | integer |
| Stream summary keys | [2135, 2133, 2137, 2134, 2136] | for RIO GRANDE, R POJOAQUE, LC SPRINGS, R TESUQUE, RIV TOTAL |

## 4. Requirements (Numbered, Testable)

| Req | Description | Acceptance Criterion |
| --- | --- | --- |
| R1 | generate_table4_data(parsed_data, year) returns cell-level + aggregated Rio Grande depletions | Returns dict with cell_data (44 entries), stream_summaries (5 streams), above/below AF totals, and Buckman cell data |
| R2 | write_table4_xlsx(parsed_data, output_path, year) writes complex Excel with cell data, stream summaries, AF calculations, and cross-check formulas | Output file has: Row 1 headers, Rows 2-45 cell data (6-decimal cfs), Rows 46-50 stream summaries, days row, cfs section, AF section, cross-check formula rows. Font: Aptos 11. |
| R3 | generate_table5_data(parsed_data, year) returns La Cienega Springs cumulative depletion | Returns dict with monthly_cfs (12 values), annual_af, previous_cumulative_af, cumulative_af |
| R4 | write_table5_xlsx(output_path, parsed_data, processing_year, years, historical_baseline) writes Table 5 with year-chaining | Years < processing_year from prior year’s Table 5; years >= processing_year from parsed_data. Cross-check columns: annual delta, cumulative check, delta B-D (should be 0). Format: 0.00. |
| R5 | load_historical_table5(baseline_path) loads prior year’s Table 5 cumulative values | Returns dict mapping year (int) to cumulative AF (float). Falls back to LA_CIENEGA_CUMULATIVE dict if file missing. |

## 5. Worked Example

### R1: generate_table4_data

#### Input

# parsed_data from parse_postprocessor_output("output/modflow/2024/depletions/CY2024")
# Contains year_data[2024] with cell keys like "1 9 14" and stream names like "RIO GRANDE"

#### Calculation Steps

Step 1: Collect all cells from parsed_data[2024], excluding stream names
        -> 44 cells total

Step 2: Classify cells by Otowi status
        - above_keys = {"1 1 16", "1 2 16", ..., "1 10 12"}  (10 cells)
        - below_keys = {"1 11 11", "1 12 11", ..., "1 23 3"}  (16 cells)
        - non_otowi = remaining 18 cells

Step 3: Sort: non-Otowi by (row,col), then above by (row,col), then below by (row,col)
        -> ordered_cells list of 44 tuples

Step 4: Assign keys starting at 2089
        - Cell 0 -> key 2089, Cell 1 -> key 2090, ..., Cell 43 -> key 2132

Step 5: Extract stream summaries for RIO GRANDE, R POJOAQUE, LC SPRINGS, R TESUQUE, RIV TOTAL
        - Each has 12 monthly cfs values

Step 6: Sum above-Otowi cells per month -> above_cfs[12]
        Sum below-Otowi cells per month -> below_cfs[12]

Step 7: Convert to AF:
        above_af[0] = above_cfs[0] * 31 * 86400 / 43560  (January)
        2024 example: above_otowi_annual_af ~ 101.43 AF
                      below_otowi_annual_af ~ 842.94 AF

#### Expected Output

{
    "cell_data": [44 dicts with key, year, lay, row, col, monthly_cfs, otowi],
    "stream_summaries": {"RIO GRANDE": [12 floats], ...},
    "days_per_month": [31, 28, 31, 30, 31, 30, 31, 31, 30, 31, 30, 31],
    "above_otowi_cfs": [12 floats],
    "below_otowi_cfs": [12 floats],
    "above_otowi_af": [12 floats],
    "below_otowi_af": [12 floats],
    "above_otowi_annual_af": 101.43,   # approximate
    "below_otowi_annual_af": 842.94,  # approximate
    "total_rg_af": [12 floats],
    "total_rg_annual_af": 944.37,     # approximate
    "buckman_cfs": [12 floats],
    "buckman_af": [12 floats],
    "buckman_annual_af": float,
}

### R3: generate_table5_data

#### Input

# parsed_data[2024]["LC SPRINGS"] contains monthly cfs values
# e.g., {"jan": 0.000052, "feb": 0.000053, ..., "dec": 0.000058}

#### Calculation Steps

Step 1: Extract LC SPRINGS monthly cfs -> lc_springs_cfs = [0.000052, 0.000053, ...]

Step 2: Convert to annual AF using calendar days for 2024 (leap year):
        cumulative_af = sum(cfs_i * days_i * 86400 / 43560 for i in 0..11)
        = sum(cfs_i * days_i * 1.9835)
        ~ 3.74 AF (this is CUMULATIVE from all pumping since 1988)

Step 3: Get previous cumulative from LA_CIENEGA_CUMULATIVE[2023] = 3.54

Step 4: annual_af = cumulative_af - previous_cumulative = 3.74 - 3.54 = 0.20 AF

#### Expected Output

{
    "year": 2024,
    "monthly_cfs": [0.000052, 0.000053, ..., 0.000058],
    "annual_af": 0.20,
    "previous_cumulative_af": 3.54,
    "cumulative_af": 3.74,
}

## 6. Files to Create/Modify

| Action | Path | Description |
| --- | --- | --- |
| Modify | stream_depletions.py | Add generate_table4_data(), write_table4_xlsx(), generate_table5_data(), write_table5_xlsx(), load_historical_table5() |

## 7. Acceptance Criteria

# These commands must all pass:
pytest tests/test_stream_depletions.py -v --tb=short -k "table4 or table5"
ruff check stream_depletions.py
mypy stream_depletions.py

Expected output: - generate_table4_data() returns dict with 44 cell entries and correct above/below AF totals - write_table4_xlsx() creates XLSX with ~70 rows including cell data, summaries, AF sections, and cross-check formulas - generate_table5_data() returns correct cumulative AF matching LA_CIENEGA_CUMULATIVE[year] - write_table5_xlsx() creates XLSX with years 2004-2030 and cross-check columns

## 8. Known Gotchas

- ☐ Table 4 uses DAYS_VALIDATION (non-leap year: Feb=28) for the days-per-month row, NOT the actual calendar days for the processing year. This matches the validation file convention.

- ☐ Stream name splits in Table 4 XLSX are cosmetic artifacts of FORTRAN fixed-width output. “RIO GRANDE” becomes “0 RI” / “O GRA” / “NDE” across columns 3-5. These must be reproduced exactly.

- ☐ Table 5 cumulative values from MODFLOW are already cumulative from all pumping since 1988, not annual increments. The annual_af is computed by subtraction from the prior year.

- ☐ Table 5 chaining: years before processing_year are preserved from the prior year’s output, not recomputed. This locks in historical values at the time they were produced.

- ☐ Buckman wells cell (1,13,11) is also a BELOW_OTOWI cell – it appears both in the cell_data list and is extracted separately for the Buckman row.

- ☐ Cross-check formula rows use Calibri 9pt italic gray font, not Aptos 11pt.

## 9. Year-Chaining Behavior

| Data Element | Source for Year N | Source for Year N+1 |
| --- | --- | --- |
| Table 4 cell data | Computed from current MODFLOW run | Computed fresh (no chaining) |
| Table 5 years < N | Prior year’s Table 5 XLSX | Prior year’s Table 5 XLSX |
| Table 5 year >= N | Current MODFLOW run via cfs_monthly_to_af_annual() | Current MODFLOW run |
| Table 5 fallback | LA_CIENEGA_CUMULATIVE dict | LA_CIENEGA_CUMULATIVE dict |

Table 4 does NOT chain – it is fully computed from the current year’s MODFLOW output. Table 5 chains cumulative values to ensure historical years remain locked.

## 10. Verification

python step4_generate_depletion_tables.py --year 2024
diff <(python -c "
import openpyxl
wb = openpyxl.load_workbook('output/depletion/TABLE_4_Rio_Grande_Otowi_2024.xlsx', data_only=True)
ws = wb.active
for row in ws.iter_rows(min_row=56, max_row=58, min_col=18, max_col=18, values_only=True):
    print(f'{row[0]:.3f}')
") <(echo -e "101.430\n842.940\n944.370")

Expected result: Above/below/total Otowi AF values within 0.01 AF of validation reference.

## 11. Cross-References

| Related Spec | Relationship |
| --- | --- |
| DS-03 | Otowi cell classification and La Cienega GHB cell mapping |
| DS-02 | CFS-to-AF conversion factor derivation |
| IS-07 | Provides parse_postprocessor_output() and extract_otowi_depletions() |
| IS-08 | Table 3 XLSX writing pattern (chaining, styling) |
| IS-10 | Test coverage for generate_table4_data and generate_table5_data |

# IS-10: Test Suite (Layers 0-1)

Tier 2 Implementation Specification – A complete Claude Code session-sized prompt. Paste into a fresh session with Tier 1 domain docs available to get a working module back. Sequentially ordered with explicit dependencies.

## 1. Session Goal

Implement the pytest test suite covering Layer 0 (smoke tests), Layer 0.5 (edge cases), and Layer 1 (conservation/mass-balance checks) across 7 test files with approximately 240 tests total.

## 2. Prerequisites

### Prior Specs (must be complete)

- IS-01: Project scaffold and constants

- IS-03: CSV ingestion (step1_ingest_buckman_data.py)

- IS-05: MODFLOW update (step2_update_modflow.py)

- IS-07: Stream depletion parsing (stream_depletions.py)

- IS-09: Tables 4 & 5 generation

### Input Files (must exist)

| File | Path | Description |
| --- | --- | --- |
| pytest.ini | pytest.ini | Marker registration (layer0, edge_cases, conservation) |
| Ingestion module | step1_ingest_buckman_data.py | CSV ingestion functions |
| MODFLOW update module | step2_update_modflow.py | WEL file update functions |
| Stream depletions | stream_depletions.py | Depletion calculation functions |
| Step 4 module | step4_generate_depletion_tables.py | Table generation orchestrator |

### Domain Knowledge

- See DS-02 for unit conversion factors and hand-calculable examples

- See DS-03 for MODFLOW cell geometry and Otowi classification

- See DS-04 for FORTRAN post-processor extraction rectangles

## 3. Context for Claude Code

The test suite implements the first three layers of the 8-prompt testing framework:

Layer 0 (Smoke Tests): Verify code runs without crashing. Tests should complete in < 30 seconds with no MODFLOW dependency. These catch import errors, missing functions, and basic execution failures. All tests marked @pytest.mark.layer0.

Layer 0.5 (Edge Cases): Verify the pipeline handles unexpected inputs correctly – either processing them gracefully or failing with clear diagnostic error messages. Tests cover zero pumping, leap years, empty months, malformed CSV, missing files. Marked @pytest.mark.edge_cases.

Layer 1 (Conservation): Verify physics constraints hold. Mass balance, pumping conservation, depletion upper bounds, and table sum integrity. These require pipeline outputs to exist. Marked @pytest.mark.conservation.

### Key Equations (Inline)

MG to AF:       AF = MG * 3.06889
AF to ft3/s:    rate = -(AF / num_layers) * 43560 / (days * 86400)
CFS to AF:      AF = cfs * days * 86400 / 43560  (= cfs * days * 1.9835)
CFS per day:    1 cfs = 86400 ft3/day = 1.9835 AF/day

### Key Constants (Inline)

| Constant | Value | Units |
| --- | --- | --- |
| MG_TO_AF_FACTOR | 3.06889 | AF/MG |
| ACRE_FT_TO_FT3 | 43560 | ft3/AF |
| SECONDS_PER_DAY | 86400 | s/day |
| BUDGET_CLOSURE_TOLERANCE | 0.1 | percent |
| PUMPING_CONSERVATION_TOLERANCE | 0.1 | percent relative |
| TABLE_SUM_TOLERANCE | 0.01 | acre-feet |

## 4. Requirements (Numbered, Testable)

| Req | Description | Acceptance Criterion |
| --- | --- | --- |
| R1 | tests/conftest.py with shared fixtures | Provides sample_valid_daily_csv, sample_valid_table2_csv, parametrized month/days fixtures. All 7 test files can import fixtures. Note: tests/conftest.py does not currently exist; fixtures like sample_valid_daily_csv and sample_valid_table2_csv are defined directly in test_edge_cases.py. |
| R2 | tests/test_ingest_buckman_data.py – CSV ingestion tests | Tests: module imports, constants defined, 13-well map, MG-to-AF factor, monthly ordering, CSV read/validate/aggregate/convert. ~20 tests. All marked layer0. |
| R3 | tests/test_update_modflow.py – MODFLOW WEL operations | Tests: module imports, convert_af_to_ft3s() hand-calculated, zero pumping, leap year February, WEL file parsing, well entry generation. ~67 tests. All marked layer0. |
| R4 | tests/test_stream_depletions.py – depletion calculations | Tests: module imports, cfs_to_acre_feet() and cfs_to_af(), Core 2003 residuals exist, parse_postprocessor_output(), generate_table3_data(), generate_table4_data(), generate_table5_data(). ~71 tests. All marked layer0. |
| R5 | tests/test_conservation.py – mass balance checks | Tests: MODFLOW budget closure, pumping-in equals pumping-used, depletion <= pumping, table row/column sum integrity. ~4 test functions (file contains many helper functions). Marked conservation. |
| R6 | tests/test_edge_cases.py – boundary conditions | Tests: missing file handling, empty CSV, zero pumping wells, leap year, single-day month, malformed columns, NaN handling, WEL file CRLF line endings. ~30 tests. Marked edge_cases. |
| R7 | tests/test_modflow_geometry.py – cell coordinate validation | Tests: GHB file parsing, La Cienega cells within FORTRAN rectangle, Otowi cell classification, above/below cell count, BUCKMAN_WELLS_CELL in BELOW_OTOWI. ~16 tests. Note: test_modflow_geometry.py does NOT have a pytestmark = pytest.mark.layer0 assignment; running pytest -m layer0 will not pick up these tests unless the marker is added. |
| R8 | pytest markers registered and functional | pytest -m layer0, pytest -m edge_cases, pytest -m conservation each select the correct subset |

## 5. Worked Example

### R2: test_mg_to_af_conversion

#### Input

# Known conversion: 1 MG = 3.06889 AF
# Test input: 100 MG
mg_value = 100.0

#### Calculation Steps

Step 1: Apply conversion factor
        af = 100.0 * 3.06889 = 306.889 AF

Step 2: Verify range bounds (smoke test style)
        assert 306.0 < af < 307.0

#### Expected Output

# 306.889 AF (within tolerance)

### R3: test_convert_af_to_ft3s_known_answer

#### Input

# Well 1 JAN 2024: 16.887963 AF, 31 days, 2 layers
acre_feet = 16.887963
days_in_month = 31
num_layers = 2

#### Calculation Steps

Step 1: Per-layer volume
        per_layer_af = 16.887963 / 2 = 8.443982 AF

Step 2: Convert to ft3
        ft3 = 8.443982 * 43560 = 367,803.0 ft3

Step 3: Convert to rate (ft3/s)
        rate = 367,803.0 / (31 * 86400) = 367,803.0 / 2,678,400 = 0.13730 ft3/s

Step 4: Apply pumping convention (negative)
        result = -0.13730 ft3/s

#### Expected Output

# -0.13730 ft3/s (within [-0.15, -0.12] smoke test range)

### R4: test_cfs_to_af_sanity

#### Input

# 0.1 cfs for 31 days (January)
cfs = 0.1
days = 31

#### Calculation Steps

Step 1: Daily AF = 0.1 * 86400 / 43560 = 0.19835 AF/day
Step 2: Monthly AF = 0.19835 * 31 = 6.149 AF

#### Expected Output

# 6.149 AF (within [5.0, 8.0] range)

### R5: test_depletion_does_not_exceed_pumping

#### Input

# 2024 data: total_pumping ~ 1372.90 AF
# Rio Pojoaque total ~ 60.8 AF, Rio Tesuque total ~ 33.6 AF
# Total depletion ~ 94.4 AF

#### Calculation Steps

Step 1: Sum depletions from all streams
        total_depletion = pojoaque_total + tesuque_total = 60.8 + 33.6 = 94.4 AF

Step 2: Compare to pumping
        94.4 AF < 1372.90 AF -> PASS (depletion < pumping)

Step 3: Verify ratio is physically reasonable
        ratio = 94.4 / 1372.90 = 0.069 -> within [0.05, 0.15] expected range

## 6. Files to Create/Modify

| Action | Path | Description |
| --- | --- | --- |
| Create/Modify | tests/conftest.py | Shared fixtures for all test files |
| Create/Modify | tests/test_ingest_buckman_data.py | Step 1 ingestion smoke tests |
| Create/Modify | tests/test_update_modflow.py | Step 2 MODFLOW update smoke tests |
| Create/Modify | tests/test_stream_depletions.py | Stream depletion calculation tests |
| Create/Modify | tests/test_conservation.py | Layer 1 conservation/mass-balance checks |
| Create/Modify | tests/test_edge_cases.py | Layer 0.5 edge case and boundary tests |
| Create/Modify | tests/test_modflow_geometry.py | MODFLOW cell geometry validation |
| Modify | pytest.ini | Register markers: layer0, edge_cases, conservation |

## 7. Acceptance Criteria

# Run all layers independently:
pytest tests/ -m layer0 -v --tb=short           # ~120 smoke tests
pytest tests/ -m edge_cases -v --tb=short        # ~50 edge case tests
pytest tests/ -m conservation -v --tb=short      # ~20 conservation tests (requires pipeline outputs)

# Run full suite:
pytest tests/ -v --tb=short

# Code quality:
ruff check tests/
mypy tests/ --ignore-missing-imports

Expected output: All Layer 0 and edge case tests pass without MODFLOW dependency. Conservation tests pass if pipeline has been run for the target year. Total: ~240 tests.

## 8. Known Gotchas

- ☐ conftest.py fixtures must not depend on MODFLOW outputs. Layer 0 and 0.5 tests must run on any machine without pipeline execution. Use mock data and tmp_path fixtures.

- ☐ Conservation tests (Layer 1) require output/modflow/{year}/ to exist. These tests are skipped gracefully if pipeline outputs are missing. Use pytest.importorskip or skipIf patterns.

- ☐ The pytestmark = pytest.mark.layer0 module-level marker applies to ALL tests in a file. Do not mix markers within a single file – use separate files for each layer.

- ☐ step1_ingest_buckman_data.py is in project root, not src/. Import paths must account for this: import step1_ingest_buckman_data (not from src import ...).

- ☐ Edge case tests that create temporary files must use tmp_path fixture to avoid polluting the working directory.

- ☐ The test_conservation.py module doubles as a standalone script (python tests/test_conservation.py --year 2024) and a pytest module. Both entry points must work.

- ☐ Well numbers are 1-indexed (1-13), not 0-indexed. All well iteration should use range(1, 14).

## 9. Year-Chaining Behavior

Not directly applicable to tests, but test fixtures should cover chaining scenarios:

| Data Element | Test Fixture Approach |
| --- | --- |
| Table 3 historical | Mock XLSX with 3 years of data |
| Table 5 cumulative | Use LA_CIENEGA_CUMULATIVE dict values directly |
| Prior year outputs | Create minimal fixtures in tmp_path |

## 10. Verification

Single command to confirm the test suite works end-to-end:

pytest tests/ -v --tb=short -m "layer0 or edge_cases" 2>&1 | tail -5

Expected result: ~170 passed with no failures. Conservation tests are separate because they require pipeline outputs.

## 11. Cross-References

| Related Spec | Relationship |
| --- | --- |
| DS-02 | Unit conversion factors used in hand-calculated test values |
| DS-03 | Cell geometry constants verified by test_modflow_geometry |
| IS-03 | step1_ingest_buckman_data.py – target of test_ingest_buckman_data |
| IS-05 | step2_update_modflow.py – target of test_update_modflow |
| IS-07 | stream_depletions.py – target of test_stream_depletions |
| IS-09 | Tables 4 & 5 generation – tested by test_stream_depletions |
| IS-11 | Validation framework consumes test results from this suite |

## Appendix: Test Distribution by File

| File | Layer | Est. Tests | Key Focus Areas |
| --- | --- | --- | --- |
| test_ingest_buckman_data.py | 0 | ~20 | Constants, CSV parsing, validation flags, MG-to-AF |
| test_update_modflow.py | 0 | ~67 | AF-to-ft3/s, WEL parsing, well entries, leap year |
| test_stream_depletions.py | 0 | ~71 | CFS conversion, Core 2003, parsing, Tables 3-5 data |
| test_generate_depletion_tables.py | 0 | ~32 | Step 4 orchestrator, constants, print_error |
| test_conservation.py | 1 | ~4 | Budget closure, pumping conservation, depletion bounds (many helpers) |
| test_edge_cases.py | 0.5 | ~30 | Missing files, zero pumping, leap year, NaN, CRLF |
| test_modflow_geometry.py | 0 | ~16 | GHB parsing, Otowi cells, FORTRAN rectangle |
| Total |  | ~240 |  |

### Marker Configuration (pytest.ini)

[pytest]
markers =
    layer0: Layer 0 smoke tests (fast, no MODFLOW dependency)
    edge_cases: Layer 0.5 edge case and boundary condition tests
    conservation: Layer 1 conservation and mass-balance checks (requires pipeline outputs)

# IS-11: Validation Framework (Layers 2-5)

Tier 2 Implementation Specification – A complete Claude Code session-sized prompt. Paste into a fresh session with Tier 1 domain docs available to get a working module back. Sequentially ordered with explicit dependencies.

## 1. Session Goal

Implement the validation framework covering Layer 2 (temporal consistency), Layer 4 (ballpark/perturbation checks), Layer 5 (2024 regression harness), and the run_all_tests.py orchestrator that coordinates all layers.

## 2. Prerequisites

### Prior Specs (must be complete)

- IS-01: Project scaffold (directory structure, constants)

- IS-09: Tables 4 & 5 generation (provides depletion data for validation)

- IS-10: Test suite Layers 0-1 (provides pytest infrastructure)

### Input Files (must exist)

| File | Path | Description |
| --- | --- | --- |
| bounds.yaml | validation/historical/bounds.yaml | Historical bounds for 2022-2024 |
| tolerances.yaml | validation/2024/tolerances.yaml | Per-table tolerance definitions |
| Frozen 2024 inputs | validation/2024/inputs/ | Regression test input files |
| Expected 2024 outputs | validation/2024/expected_outputs/ | Regression test expected outputs |
| hashes.json | validation/2024/inputs/hashes.json | SHA-256 hashes of frozen inputs |

### Domain Knowledge

- See DS-02 for unit definitions and conversion factors

- See DS-05 for historical data ranges (2022-2024 pumping and depletions)

## 3. Context for Claude Code

The validation framework provides multi-layer defense against errors in the compliance-critical pipeline:

Layer 2 (Temporal Consistency): Year-over-year checks that flag statistical anomalies. All checks produce soft flags requiring human review – not hard failures. Thresholds are data-driven from 3 years of baseline (2022-2024).

Layer 4 (Ballpark Check): Fast (<5 second) sanity check against historical bounds. Hard fails on physics violations (negative values, non-monotonic depletions). Soft flags on statistical outliers (>2 sigma from mean).

Layer 5 (2024 Regression): Frozen-input regression test. Runs pipeline on known-good 2024 data and compares cell-by-cell against expected outputs. Highest-value test – catches any code change that alters compliance numbers.

run_all_tests.py: Master orchestrator that runs all layers in priority order, collects results into structured dataclasses, generates JSON reports, and produces a provenance manifest.

### Key Equations (Inline)

Envelope bounds:  buffer = max(20%, 1.5 * CV)
                  lower = min(historical) * (1 - buffer)
                  upper = max(historical) * (1 + buffer)

Hybrid tolerance:  PASS if |actual - expected| <= absolute_tolerance
                   OR    if |actual - expected| / |expected| <= relative_
                        tolerance

Pearson correlation:  r = cov(current_profile, historical_mean) / 
                          (std_c * std_h)
                     Low r (< 0.75) indicates possible stress period 
                     misalignment

### Key Constants (Inline)

| Constant | Value | Source |
| --- | --- | --- |
| PUMPING_CHANGE_THRESHOLD_PCT | 65.0% | max observed (58.5%) + 10% buffer |
| RATIO_CHANGE_THRESHOLD_PCT | 45.0% | max observed (36.5%) + 10% buffer |
| SEASONAL_CORRELATION_THRESHOLD | 0.75 | month-shift detection simulation |
| ENVELOPE_MIN_BUFFER | 0.20 | measurement uncertainty floor |
| ENVELOPE_CV_MULTIPLIER | 1.5 | scales with observed variability |
| Table 3 absolute tolerance | 0.001 AF | 3-decimal place precision |
| Table 4 absolute tolerance | 0.1 AF | FLAG: may need adjustment |
| Table 5 absolute tolerance | 0.01 AF | 2-decimal place precision |

## 4. Requirements (Numbered, Testable)

| Req | Description | Acceptance Criterion |
| --- | --- | --- |
| R1 | validation/ballpark_check.py – fast sanity check | Loads bounds.yaml, checks pumping within soft_max/hard_max, checks per-well bounds, checks depletion monotonicity. Exit codes: 0=pass, 2=soft flags, 3=hard fail, 4=skip. Completes in < 5 seconds. |
| R2 | validation/temporal_consistency.py – year-over-year checks | Checks: YoY pumping change rate (65% threshold), YoY depletion/pumping ratio change (45%), seasonal correlation (r >= 0.75), envelope bounds (CV-adjusted). All soft flags. Exit codes: 0=pass, 1=flags. |
| R3 | validation/2024/run_regression_2024.py – frozen 2024 regression | Step 1: verify input SHA-256 hashes. Step 2: run pipeline on frozen inputs. Step 3: compare all 5 tables cell-by-cell with hybrid tolerance. Reports pass/fail with cell-level detail. Exit codes: 0=pass, 1=fail. |
| R4 | validation/2024/tolerances.yaml format | Per-table entries with tolerance_type: hybrid, absolute_tolerance, relative_tolerance. Global: nan_handling: fail, empty_cell_handling: match. |
| R5 | validation/historical/bounds.yaml format | Sections: table1_annual_pumping (total + per-well bounds), table3_stream_depletions (per-stream bounds), table4_rio_grande_depletions (above/below Otowi bounds), table5_la_cienega (cumulative depletion bounds), derived_ratios (depletion/pumping ratios), time_series (raw values for regression), monthly_profile (12-month normalized distribution), thresholds (sigma multipliers, monotonic tolerance). |
| R6 | run_all_tests.py orchestration | Dataclasses: TestResult, Flag, TestSuite. Runs: ballpark check -> Layer 0 pytest -> Layer 0.5 pytest -> Layer 1 pytest -> temporal consistency -> manifest generation. JSON report parsing. Summary output. Exit codes: 0=pass, 1=hard fail, 3=physics violation. |

## 5. Worked Example

### R1: ballpark_check – pumping bounds

#### Input

# From bounds.yaml:
table1_annual_pumping:
  total_annual:
    min: 866.48
    max: 1372.90
    mean: 1071.62
    std: 266.95
    soft_max: 2745.80   # 2x max
    hard_max: 4118.70   # 3x max
thresholds:
  soft_flag_sigma: 2.0

#### Calculation Steps

Step 1: Load 2024 total pumping from Table 1 -> 1372.90 AF

Step 2: Check negative: 1372.90 >= 0 -> PASS

Step 3: Check hard_max: 1372.90 <= 4118.70 -> PASS

Step 4: Check soft_max: 1372.90 <= 2745.80 -> PASS

Step 5: Check 2-sigma range:
        upper = 1071.62 + 2.0 * 266.95 = 1605.52
        lower = 1071.62 - 2.0 * 266.95 = 537.72
        1372.90 in [537.72, 1605.52] -> PASS

Result: Exit code 0 (all checks passed)

### R2: temporal_consistency – seasonal pattern

#### Input

# Current year monthly pumping (AF): JAN=7.5, FEB=73.0, MAR=83.4, ...
# Historical mean profile from bounds.yaml: JAN=0.5%, FEB=5.3%, ...

#### Calculation Steps

Step 1: Normalize current year to fractions
        total = sum(monthly) = 1372.90 AF
        JAN_frac = 7.5 / 1372.90 = 0.0055
        FEB_frac = 73.0 / 1372.90 = 0.0532
        ...

Step 2: Compute Pearson correlation with historical mean profile
        r = pearsonr(current_fractions, mean_profile)

Step 3: Compare to threshold
        If r >= 0.75 -> PASS (monthly pattern consistent)
        If r < 0.75 -> FLAG (possible stress period misalignment)

### R3: run_regression_2024 – hybrid tolerance

#### Input

# From tolerances.yaml:
Table_3:
  absolute_tolerance: 0.001   # 3-decimal AF
  relative_tolerance: 0.001   # 0.1%

#### Calculation Steps

Step 1: Load expected Table 3, row for 2024
        Expected: Pojoaque total = 60.797 AF

Step 2: Load actual Table 3 from pipeline output
        Actual: Pojoaque total = 60.796 AF

Step 3: Hybrid tolerance check
        |60.796 - 60.797| = 0.001
        Absolute: 0.001 <= 0.001 -> PASS
        (Would also check relative: 0.001 / 60.797 = 0.0000164 <= 0.001 -> PASS)

Step 4: Either condition PASS -> cell PASSES

### R6: run_all_tests.py – orchestration flow

#### Execution Order

1. Validate year parameter (MIN=2022, MAX=2100)
2. Ballpark check (exit 3 -> abort immediately)
3. Layer 0 pytest -m layer0 (JSON report)
4. Layer 0.5 pytest -m edge_cases (JSON report)
5. Layer 1 pytest -m conservation (JSON report, skip if no outputs)
6. Temporal consistency (collect flags)
7. Generate provenance manifest (Layer 6)
8. Print summary, return exit code

## 6. Files to Create/Modify

| Action | Path | Description |
| --- | --- | --- |
| Create/Modify | validation/ballpark_check.py | Fast sanity check against historical bounds |
| Create/Modify | validation/temporal_consistency.py | Year-over-year temporal consistency flags |
| Create/Modify | validation/2024/run_regression_2024.py | Frozen 2024 regression harness |
| Create/Modify | validation/2024/tolerances.yaml | Per-table hybrid tolerance definitions |
| Create/Modify | validation/historical/bounds.yaml | Historical bounds and time series data |
| Create/Modify | run_all_tests.py | Master test orchestrator |

## 7. Acceptance Criteria

# Ballpark check (requires validation outputs):
python validation/ballpark_check.py --year 2024
# Expected: Exit code 0 or 4 (pass or skip)

# Temporal consistency (requires pipeline outputs):
python validation/temporal_consistency.py --year 2024
# Expected: Exit code 0 (no flags for baseline year)

# Full test orchestrator:
python run_all_tests.py --year 2024 --dry-run
# Expected: Prints execution plan without running

# Full run (requires pipeline completion):
python run_all_tests.py --year 2024
# Expected: Exit code 0, manifest generated in output/manifests/

## 8. Known Gotchas

- ☐ Ballpark exit code 1 is reserved for Python crashes, not intentional failures. Intentional failures use 2 (soft) or 3 (hard). The orchestrator must distinguish crash (code 1) from expected exit codes (0, 2, 3, 4).

- ☐ Temporal consistency thresholds are derived from only 3 data points (2022-2024). Traditional 95% prediction intervals require t-distribution with df=1, yielding useless bounds. The CV-adjusted envelope method is explicitly documented as “honest about n=3 limitations.”

- ☐ The regression harness runs the FULL pipeline including MODFLOW96 via Wine. It takes 30-45 minutes. It should NOT be included in run_all_tests.py default execution – only via --include-regression flag or standalone.

- ☐ bounds.yaml monthly_profile values must sum to 1.0. A profile that sums to 0.98 or 1.02 will produce incorrect correlation calculations. Validate on load.

- ☐ tolerances.yaml hybrid tolerance uses OR logic, not AND. A cell passes if EITHER the absolute OR relative tolerance is met. This handles both near-zero values (where relative tolerance is meaningless) and large values (where absolute tolerance is too tight).

- ☐ pytest-json-report plugin is required for run_all_tests.py JSON report parsing. Install via pip install pytest-json-report. The orchestrator falls back to exit-code-only parsing if the plugin is missing.

- ☐ TestResult.passed can be -1 (unknown count) when JSON report generation fails. Handle this in summary calculations.

- ☐ Layer 3 (cross-comparison) was scientifically rejected in the P6 review and is explicitly skipped with a log message.

## 9. Year-Chaining Behavior

| Data Element | Source for Year N Validation | Notes |
| --- | --- | --- |
| Historical bounds | bounds.yaml (2022-2024 baseline) | Static file, updated annually |
| Temporal consistency | bounds.yaml time_series + Table 2 output | Compares against all historical years |
| Regression baseline | validation/2024/ frozen inputs/outputs | Never changes (2024 locked) |
| Run-time validation | Current pipeline outputs | Checked against bounds.yaml |

## 10. Verification

# Quick verification (no pipeline dependency):
python run_all_tests.py --year 2024 --dry-run

# Full verification:
python run_all_tests.py --year 2024 -v

Expected result for --dry-run:

DRY RUN: Would execute for year 2024
  1. Ballpark check: validation/ballpark_check.py
  2. Layer 0 (smoke): pytest -m layer0
  3. Layer 0.5 (edge): pytest -m edge_cases
  4. Layer 1 (conservation): pytest -m conservation
  5. Layer 2 (temporal): validation/temporal_consistency.py
  6. Layer 6 (manifest): src/pipeline_manifest.py

## 11. Cross-References

| Related Spec | Relationship |
| --- | --- |
| DS-05 | Historical data ranges used in bounds.yaml |
| IS-09 | Tables 4 & 5 outputs validated by ballpark check |
| IS-10 | Layer 0-1 tests invoked by run_all_tests.py orchestrator |
| IS-12 | Provenance manifest generated as Layer 6 of orchestration |

## Appendix A: bounds.yaml Structure

metadata:
  generated: "2026-02-17"
  years_included: [2022, 2023, 2024]
  units: "acre-feet (AF)"

table1_annual_pumping:
  total_annual:
    min: 866.48       # Historical minimum
    max: 1372.90      # Historical maximum
    mean: 1071.62     # Historical mean
    std: 266.95       # Standard deviation
    soft_max: 2745.80 # 2x max (soft flag)
    hard_max: 4118.70 # 3x max (hard fail)
  wells:
    Well_B1: {min: 224.06, max: 601.27, mean: 427.82}
    # ... 12 more wells

table3_stream_depletions:
  rio_pojoaque_nambe: {min: 59.844, max: 60.797, mean: 60.321, std: 0.477}
  rio_tesuque: {min: 33.490, max: 33.583, mean: 33.521, std: 0.053}

table4_rio_grande_depletions:
  above_otowi: {min: 101.21, max: 101.43}
  below_otowi: {min: 842.0, max: 843.0}

table5_la_cienega:
  cumulative: {min: 3.37, max: 3.74}

derived_ratios:
  depletion_to_pumping: {min: 0.05, max: 0.15}

time_series:
  years: [2022, 2023, 2024]
  annual_pumping_af: {values: [975.47, 866.48, 1372.90]}
  rio_pojoaque_nambe_depletion_af: {values: [59.844, 60.323, 60.797]}
  rio_tesuque_depletion_af: {values: [33.490, 33.491, 33.583]}
  la_cienega_depletion_af: {values: [3.37, 3.54, 3.74]}

monthly_profile:
  mean_profile:
    JAN: 0.005463
    FEB: 0.053194
    # ... 10 more months (sum = 1.0)

thresholds:
  soft_flag_sigma: 2.0
  monotonic_tolerance_af: 0.01

## Appendix B: tolerances.yaml Structure

tables:
  Table_1:
    description: "Annual pumping by well (acre-feet/year)"
    tolerance_type: hybrid
    absolute_tolerance: 0.01
    relative_tolerance: 0.001

  Table_2:
    absolute_tolerance: 0.01
    relative_tolerance: 0.001

  Table_3:
    absolute_tolerance: 0.001   # Strict: 3-decimal AF
    relative_tolerance: 0.001

  Table_4:
    absolute_tolerance: 0.1
    relative_tolerance: 0.01

  Table_5:
    absolute_tolerance: 0.01
    relative_tolerance: 0.005
    max_year: 2024  # Only compare through 2024

global:
  nan_handling: "fail"
  empty_cell_handling: "match"

## Appendix C: run_all_tests.py Dataclasses

@dataclass
class TestResult:
    layer: str           # "Layer 0", "Layer 1", etc.
    name: str            # "smoke_tests", "conservation"
    passed: int
    failed: int
    skipped: int
    duration_sec: float
    hard_fail: bool
    error_message: str | None = None
    failed_tests: list[str] = field(default_factory=list)

@dataclass
class Flag:
    layer: str
    test_name: str
    value: float
    threshold: float
    message: str
    requires_review: bool = True

@dataclass
class TestSuite:
    year: int
    results: list[TestResult]
    flags: list[Flag]
    manifest_path: Path | None
    start_time: datetime
    end_time: datetime | None
    exit_code: int
    error_messages: list[str] = field(default_factory=list)

# IS-12: Provenance & Compliance (Layer 6)

Tier 2 Implementation Specification – A complete Claude Code session-sized prompt. Paste into a fresh session with Tier 1 domain docs available to get a working module back. Sequentially ordered with explicit dependencies.

## 1. Session Goal

Implement src/pipeline_manifest.py (JSON provenance manifest) and src/workflow_logger.py (MD + DOCX audit trail) for regulatory compliance documentation of the Buckman Wellfield depletion pipeline.

## 2. Prerequisites

### Prior Specs (must be complete)

- IS-01: Project scaffold (directory structure, src/ package)

- IS-09: Tables 4 & 5 generation (outputs referenced in inventories)

- IS-10: Test suite (test results consumed by manifest)

- IS-11: Validation framework (ballpark check, temporal consistency results consumed)

### Input Files (must exist)

| File | Path | Description |
| --- | --- | --- |
| Pumping CSV | input/csv/Buckman_Well_Prod_{year}.csv | Raw pumping input |
| MODFLOW templates | input/modflow/2023/thruCY2165.* | MODFLOW package files |
| Historical hashes | validation/historical/hashes.json | SHA-256 baseline hashes |
| Historical bounds | validation/historical/bounds.yaml | Validation bounds |
| Pipeline outputs | output/depletion/TABLE_{3,4,5}_*_{year}.xlsx | Generated depletion tables |
| Table 2 output | output/ingested_data/{year}_Table_2_output.csv | Monthly pumping summary |

### Domain Knowledge

- See DS-06 for regulatory compliance requirements (City of Santa Fe, OSE reporting)

- See DS-05 for Rio Grande Compact context

## 3. Context for Claude Code

Reporting compliance for groundwater depletion calculations requires a complete chain of custody from raw data to final reports. Two artifacts serve this purpose:

pipeline_manifest.py generates a machine-readable JSON manifest containing: 1. Input file inventory with SHA-256 hashes (cryptographic provenance) 2. Baseline hash verification (detect unauthorized changes) 3. Pipeline version information (git commit, script mod dates, executable info) 4. Test results from Layers 0, 1, and ballpark check 5. Flag register for items requiring human review 6. Runtime metadata (timestamps, machine info)

workflow_logger.py generates a human-readable audit trail (Markdown + DOCX) containing: 1. Header metadata (timestamp, operator, machine, git info) 2. Executive summary (key pumping and depletion numbers) 3. Input/output file inventories with SHA-256 hashes 4. Step-by-step execution log 5. Verification results summary 6. Physical interpretation with regulatory context 7. Assumptions and limitations 8. Approval signature block

### Key Concepts (Inline)

SHA-256 Hash:     Cryptographic fingerprint of file contents.
                  Any change to the file produces a completely different hash.
                  Used to prove inputs/outputs haven't been tampered with.

Hash Verification: Compare computed hash against stored baseline.
                   Mismatch -> inputs changed since baseline was established.
                   Hard stop unless --allow-hash-mismatch flag is used.

Manifest vs Log:  Manifest = machine-readable JSON for automated verification.
                  Log = human-readable MD/DOCX for regulatory review.

### Key Constants (Inline)

| Constant | Value | Context |
| --- | --- | --- |
| MANIFEST_VERSION | “1.0” | Manifest format version |
| RIO_GRANDE_COMPACT_NM_ALLOCATION | 405000 AF/yr | Approximate NM annual allocation |
| HISTORICAL_PUMPING_AVG_5YR | ~1050 AF (2024) | For year-over-year context |

## 4. Requirements (Numbered, Testable)

| Req | Description | Acceptance Criterion |
| --- | --- | --- |
| R1 | PipelineManifest class generates JSON manifest | generate() returns dict with 6 sections: input_manifest, hash_verification, pipeline_manifest, test_results_manifest, flag_register, run_metadata. save() writes to output/manifests/buckman_manifest_{year}.json. |
| R2 | PipelineManifest._collect_input_manifest() inventories all inputs | Lists: pumping CSV (with row count and date range), MODFLOW templates (all 8 files), historical baselines, bounds.yaml, regression inputs. Each entry has name, full_path, sha256, size_bytes. Note: PipelineManifest tracks all 8 MODFLOW templates, while WorkflowLogger.collect_input_inventory() (R9) tracks only 4 key templates. These are different functions with different coverage. |
| R3 | PipelineManifest._verify_baseline_hashes() validates file integrity | Reads validation/historical/hashes.json, computes SHA-256 for each listed file, compares. Returns status: VERIFIED/MISMATCH_ACKNOWLEDGED/SKIPPED. Raises HashMismatchError if mismatch and allow_hash_mismatch=False. |
| R4 | PipelineManifest._collect_pipeline_manifest() tracks code versions | Returns: git_commit (SHA), git_status (clean/dirty), python_version, script modification dates for 6 pipeline scripts, executable info for modflow96.exe and sfmodflx_2245.exe. |
| R5 | PipelineManifest._collect_test_results() aggregates test outcomes | Runs Layer 0 pytest (capturing pass/fail counts from output), loads Layer 1 results from JSON file, runs ballpark check. Returns structured results per layer. |
| R6 | PipelineManifest._get_flag_register() provides review placeholder | Returns empty entries list with documented entry_format: test_name, flagged_value, threshold, disposition, analyst_initials, date. |
| R7 | print_manifest_summary() formats one-page human-readable output | Standalone function (not a method of PipelineManifest). Prints: header with year, input files status, pipeline version (git + executables), test results (Layer 0/1/ballpark counts), flag register count, manifest save path. Fits on one terminal screen. |
| R8 | WorkflowLogger class generates MD + DOCX audit trail | generate_and_save(status) returns (md_path, docx_path). MD has 9 sections. DOCX conversion via pandoc. |
| R9 | WorkflowLogger.collect_input_inventory() hashes input files | Returns list of dicts with name, path, size_bytes, sha256, role for pumping CSV, MODFLOW templates, and bounds.yaml. |
| R10 | WorkflowLogger.collect_output_inventory() hashes output files | Returns list of dicts for Tables 1-5 and MODFLOW outputs (listing file, WEL file). |
| R11 | WorkflowLogger.collect_pumping_data() parses Table 2 | Extracts total_af, months_recorded, wells_active from Table 2 output (CSV or XLSX). |
| R12 | WorkflowLogger.collect_depletion_data() parses Tables 3-5 | Extracts: pojoaque_af, tesuque_af, above_otowi_af, below_otowi_af, la_cienega_cumulative_af. Handles NaN residuals (Pojoaque exhausted after 2015). |
| R13 | WorkflowLogger.generate_interpretation() fills physical context templates | Templates: pumping analysis (with 5-year average context), Rio Grande Compact (below Otowi as % of NM allocation), tributary impacts (Table 3 breakdown), La Cienega (Table 5 cumulative). |
| R14 | WorkflowLogger._convert_to_docx() converts MD to DOCX | Uses pandoc subprocess. Handles: pandoc not installed (RuntimeError with install instructions), timeout (60s), optional reference.docx template. |

## 5. Worked Example

### R1: PipelineManifest.generate()

#### Input

manifest_gen = PipelineManifest(year=2024, project_root=Path("."))
manifest = manifest_gen.generate()

#### Expected Output Structure

{
  "manifest_version": "1.0",
  "year": 2024,
  "input_manifest": [
    {
      "name": "Buckman_Well_Prod_2024.csv",
      "full_path": "/path/to/input/csv/Buckman_Well_Prod_2024.csv",
      "sha256": "a1b2c3d4...",
      "size_bytes": 45678,
      "row_count": 366,
      "date_range": "2024-01-01 to 2024-12-31"
    },
    {
      "name": "thruCY2165.wel",
      "full_path": "/path/to/input/modflow/2023/thruCY2165.wel",
      "sha256": "e5f6g7h8...",
      "size_bytes": 234567
    }
  ],
  "hash_verification": {
    "status": "VERIFIED",
    "baseline_file": "/path/to/validation/historical/hashes.json",
    "files_checked": 5,
    "mismatches": [],
    "override_flag_used": false
  },
  "pipeline_manifest": {
    "git_commit": "824d534abc...",
    "git_status": "clean",
    "python_version": "3.12.1",
    "scripts": [
      {"name": "step1_ingest_buckman_data.py", "modified": "2026-02-18T17:31:00"},
      {"name": "stream_depletions.py", "modified": "2026-02-20T16:02:00"}
    ],
    "executables": [
      {"name": "modflow96.exe", "modified": "2023-01-15T00:00:00", "size_bytes": 524288}
    ]
  },
  "test_results_manifest": {
    "layer_0_smoke": {"total": 152, "passed": 152, "failed": 0, "status": "PASS"},
    "layer_1_conservation": {"total": 4, "passed": 4, "failed": 0, "status": "PASS"},
    "ballpark_check": {"status": "PASS", "exit_code": 0, "hard_fails": 0, "soft_flags": 0}
  },
  "flag_register": {
    "entries": [],
    "note": "Populated by Layer 2 (temporal) and Layer 3 (cross-comparison) checks"
  },
  "run_metadata": {
    "pipeline_start": "2026-02-20T10:00:00",
    "pipeline_end": "2026-02-20T10:05:30",
    "total_runtime_seconds": 330.0,
    "machine_name": "water-modeling-01",
    "os": "Linux 5.15.167.4-microsoft-standard-WSL2",
    "python_executable": "/usr/bin/python3"
  }
}

### R3: Hash Verification

#### Input

// validation/historical/hashes.json
{
  "files": {
    "Table_1_historical.xlsx": "abc123...",
    "bounds.yaml": "def456..."
  }
}

#### Calculation Steps

Step 1: Load hashes.json -> stored_hashes

Step 2: For each file in stored_hashes:
        Compute SHA-256 of validation/historical/{filename}

Step 3: Compare:
        If match -> increment files_checked
        If mismatch -> append to mismatches list

Step 4: If mismatches AND NOT allow_hash_mismatch:
        Raise HashMismatchError with:
        - Which files mismatched
        - How to override (--allow-hash-mismatch)
        - Warning that override will be logged in manifest

### R13: Physical Interpretation – Rio Grande Compact

#### Input

below_otowi_af = 842.94
above_otowi_af = 277.50
year = 2024

#### Calculation Steps

Step 1: Compute percentage of NM allocation
        pct = 842.94 / 405000 * 100 = 0.2082%

Step 2: Fill template:
        "Depletions below Otowi Bridge (842.94 AF) are chargeable against
         New Mexico's Rio Grande Compact delivery obligation to Texas."

## 6. Files to Create/Modify

| Action | Path | Description |
| --- | --- | --- |
| Create/Modify | src/pipeline_manifest.py | PipelineManifest class, print_manifest_summary(), HashMismatchError |
| Create/Modify | src/workflow_logger.py | WorkflowLogger class, interpretation templates, DOCX conversion |
| Create/Modify | src/generate_workflow_log.py | CLI wrapper for WorkflowLogger (if exists) |

## 7. Acceptance Criteria

# Generate manifest:
python -c "
from src.pipeline_manifest import PipelineManifest, print_manifest_summary
m = PipelineManifest(year=2024, project_root='.', allow_hash_mismatch=True)
manifest = m.generate()
path = m.save(manifest)
print_manifest_summary(manifest)
print(f'Saved to: {path}')
"

# Generate workflow log:
python -c "
from src.workflow_logger import WorkflowLogger
logger = WorkflowLogger(year=2024)
md, docx = logger.generate_and_save(status='PASS')
print(f'MD: {md}')
print(f'DOCX: {docx}')
"

# Code quality:
ruff check src/pipeline_manifest.py src/workflow_logger.py
mypy src/pipeline_manifest.py src/workflow_logger.py

Expected output: Manifest JSON saved with all 6 sections populated. Workflow log MD generated with 9 sections. DOCX generated if pandoc is installed.

## 8. Known Gotchas

- ☐ HashMismatchError is a hard stop by default. The manifest generator raises this exception if baseline hashes don’t match and allow_hash_mismatch=False. The override flag (--allow-hash-mismatch) is logged in the manifest as override_flag_used: true for audit trail.

- ☐ **_run_layer_0_tests() invokes pytest as a subprocess**, not via pytest.main(). This isolates test execution from the manifest generator’s Python process. Parse pass/fail counts from stdout using regex (\d+) passed.

- ☐ WorkflowLogger.collect_depletion_data() parses XLSX files with complex column structures. Table 3 has merged headers – the year column is df.columns[0] (often “Unnamed: 0”), Pojoaque total is column index 3, Tesuque total is column index 6. Handle NaN values for exhausted residuals.

- ☐ DOCX conversion requires pandoc (sudo apt install pandoc). If pandoc is not installed, the logger returns (md_path, md_path) – the MD path is returned for both slots with a warning. Do NOT crash.

- ☐ **The manifest’s _get_csv_date_range() uses pandas** to parse the date column. The CSV may use different date formats (“1/1/2024”, “2024-01-01”, “Jan 1, 2024”). Use pd.to_datetime(errors='coerce') to handle all formats.

- ☐ Git subprocess calls must specify cwd=self.project_root to ensure they run in the correct repository, not wherever the Python process was launched from.

- ☐ HISTORICAL_PUMPING_AVG_5YR is a hardcoded dict with approximate values. These are used for context only (not validation) and should be updated annually.

- ☐ Workflow log timestamps include seconds in filenames: {year}_workflow_log_{YYYYMMDD_HHMMSS}_{status}.md. Multiple runs in the same session produce unique files.

## 9. Year-Chaining Behavior

| Data Element | Source for Year N | Source for Year N+1 |
| --- | --- | --- |
| Input manifest | Current year’s input files | Current year’s input files |
| Hash verification | validation/historical/hashes.json (static) | Same file (updated if baselines change) |
| Pipeline manifest | Current git state + script mod dates | Current git state |
| Test results | Live pytest + ballpark execution | Live execution |
| Workflow log | Current outputs + manifest | Current outputs + manifest |

There is no chaining in provenance – each year’s manifest and log are self-contained snapshots of the pipeline state at execution time.

## 10. Verification

# Quick verification (generates manifest only):
python src/pipeline_manifest.py --year 2024 --allow-hash-mismatch

# Full verification (generates manifest + workflow log):
python run_all_tests.py --year 2024

Expected result: Manifest JSON at output/manifests/buckman_manifest_2024.json with all sections populated. Workflow log at output/logs/{year}_workflow_log_*.md with 9 sections.

## 11. Cross-References

| Related Spec | Relationship |
| --- | --- |
| DS-05 | Rio Grande Compact context for interpretation templates |
| DS-06 | Regulatory compliance requirements driving manifest design |
| IS-09 | Tables 4 & 5 outputs inventoried in output manifest |
| IS-10 | Layer 0-1 test results consumed by PipelineManifest |
| IS-11 | Ballpark check and temporal consistency results consumed |

## 12. Interpretation Templates

### Pumping Analysis Template

**Total Pumping: {total_af:.2f} AF** in calendar year {year}.
- 5-year average: {avg_5yr:.0f} AF
- Year-over-year change: {yoy_change:+.1f}%
- {context_statement}

### Rio Grande Compact Template

Depletions below Otowi Bridge ({below_otowi_af:.2f} AF) are chargeable against
New Mexico's Rio Grande Compact delivery obligation to Texas.

- Below Otowi depletion: {below_otowi_af:.2f} AF
- Above Otowi depletion: {above_otowi_af:.2f} AF (does not affect compact)
- This represents {pct_of_allocation:.4f}% of typical annual NM allocation

### Tributary Template

**Rio Pojoaque:**
- {year} Depletion: {pojoaque_af:.2f} AF
- Residual Component: {pojoaque_residual:.2f} AF (pre-1988 pumping tail)
- Superposition Component: {pojoaque_super:.2f} AF (1988-{year} pumping)
- Note: Pojoaque residual reached zero in 2015

**Rio Tesuque:**
- {year} Depletion: {tesuque_af:.2f} AF
- Residual Component: {tesuque_residual:.2f} AF (pre-1988 pumping tail)
- Note: Tesuque residual continues through ~2050

### La Cienega Template

**{year} Status:**
- Cumulative impact since 2004: {cumulative_af:.2f} AF
- Annual increment: {annual_af:.2f} AF
- La Cienega Springs lies ~15 km south of Buckman Wellfield

## 13. Workflow Log 9-Section Structure

| Section | Title | Content |
| --- | --- | --- |
| 1 | Header / Metadata | Timestamp, operator, machine, git commit |
| 2 | Executive Summary | Total pumping, key depletions, verification status |
| 3 | Input File Inventory | Files with SHA-256 hashes and roles |
| 4 | Step-by-Step Execution | What each pipeline step produced |
| 5 | Output File Inventory | Generated files with SHA-256 hashes |
| 6 | Verification Summary | Layer-by-layer test results |
| 7 | Physical Interpretation | Regulatory context from templates |
| 8 | Assumptions & Limitations | MODFLOW model caveats |
| 9 | Approval Block | Prepared/Reviewed/Approved signature lines |